3GPP TSG-RAN4 Meeting #113	R4-2419398

# Orlando, United States, 18th - 22nd November, 2024

Agenda item:		7.13.2

Source:				Keysight Technologies UK Ltd

Title:					On STxMP Test Directions

Document for:		Approval

# Introduction

This contribution addresses whether and how a test direction other than the beam peak direction of single CC operation without STxMP can be used for the first AoA.

# Discussion

In the WF from the last meeting (RAN#112-bis), the following options were captured on the topic of whether a non-beam peak direction of single CC operation without STxMP can be used.

Issue 1-1-2: Whether a test direction other than the beam peak direction of single CC operation without STxMP can be used for first AoA - Agreements: - The test direction other than the beam peak direction of single CC operation without STxMP can be used for the first AoA if the test results based on the default AoA separation fail the limits in the spec. The following options can be considered: - Option 1: A local scan in xz plane is conducted. As long as test results do not pass the spec yet during scan, the local scan does not stop until AoA1 EIRP fail the legacy beam peak EIRP limit in both directions. FFS on the measurement grid of local scan. - Option 2: Alternative test directions on top of the default AoA separation can be declared by UE. FFS on the allowed offset of alternative test directions to the default test directions. - Option 1 and Option 2 only apply for EIRP PUMAX,f,c,k testing and will not have the impact on the EIRPmax and TRPmax test directions selection. - RAN4 to make down selection from Option 1 and Option 2 in next meeting

From a test system vendor presepective, option 1 is the least preferred option as it would require local searches that could be open ended and require quite extensive test time until a pass is achieved.

Observation 1: From a test system vendor presepective, option 1 is the least preferred option as it would require local searches that could be open ended and require quite extensive test time until a pass is achieved

Since OEMs have a much better understanding of the single CC and STxMP performance of the device submitted for certification, it should be relatively straightforward for them to determine a set of AoA1&AoA2 test directions with offsets from the set of {30deg, 60deg, 90deg, 120deg, 150deg} that should pass the test case. If the first set of declared AoA pairs yields a FAIL, the remaining sets of declared AoA pairs could be checked until a PASS is obtained. This option is preferred from a test system vendor perspective as this approach is straightforward and fast as it leverages the knowledge of DUT performance from the OEMs.

Observation 2: From a test system vendor presepective, option 2 is the most straightforward and fastest option as it leverages the knowledge of DUT performance from the OEMs

Proposal 1: Select option 2 for when a test direction other than the beam peak direction of single CC operation without STxMP must be used for the first AoA

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: From a test system vendor presepective, option 1 is the least preferred option as it would require local searches that could be open ended and require quite extensive test time until a pass is achieved

Observation 2: From a test system vendor presepective, option 2 is the most straightforward and fastest option as it leverages the knowledge of DUT performance from the OEMs

Proposal 1: Select option 2 for when a test direction other than the beam peak direction of single CC operation without STxMP must be used for the first AoA

# References

- R4-2416580, WF on [112bis][325] NR_FR2_OTA, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #112bis, October 2024