3GPP TSG-RAN WG4 Meeting #113                                   R4-2419400

Orlando, US, 18th – 22nd November, 2024

Source: 	MediaTek Inc.

Title: 	Discussion on power domain enhancement for NR single carrier

Agenda Item:	7.1.1.2.1

Document for:	Approval

- Introduction

In RAN#112bis, the WF was agreed upon in [1], and there are still some FFS issues. In this paper, we will propose our views and discuss these issues.

- Extended UE channel bandwidth further study

2.1 Ratio of UE channel bandwidth extension required

At RAN4#112bis, the following was agreed:

#### Issue 1-2-7: Ratio of extended CBW to UE CBW and to larger BS channel BW

- WF

  - To discuss the following aspects for the extended UE CBW:

    - Whether the extended CBW with new boundary of FOOB should comply with the recommendation by ITU-R on necessary bandwidth in terms of SE for the original UE BW

    - Whether default or fixed extension, e.g. 1/2 UE CBW could be stipulated in the spec or the extension ratio could be a UE capability to fulfil the conversion of outer to inner

    - Whether the extended UE CBW could belong to the regular defined channel bandwidth

    - Whether some premise should be established for the extension size, e.g.

      - minimum extended CBW compared to original UE CBW

      - minimum BS CBW compared to UE CBW

      - frequency separation of the RB allocation to the BS edge

    - Other identified issues are not precluded

If the BS CBW is such that there is insufficient bandwidth available to extend by that amount on each side, then EITHER the original emissions requirements and MPR would apply, OR asymmetrical UE channel bandwidth extension can be considered (see later proposal).

Regarding the proposal that BS CBW = extended UE CBW, there are 3 factors to consider:

- Spurious Emission requirement: from the SE analysis in this document, we believe there is some regulatory risk where the BS CBW > 2 x UE CBW.

- Putting responsibility on the UE to decide whether to apply the extended CBW: It appears feasible for the UE to use the carrierBW, offsetToPointA and offsetToCarrier in SIB1, as well as its BWP location, to check whether its extended CBW would go beyond the edge of the BS CBW, and it could be considered whether the UE should check this before applying the relaxation. Therefore, we see no strict need to directly couple the size of the extended CBW with the size of the BS CBW, as long as the extended CBW does not go beyond the spectrum block boundary, and while the spectrum block is confined within the maximum specified BS CBW. It should be an FFS point as to how the UE would do this if the spectrum block size is larger than the maximum BS CBW.

- Lack of benefit for UE vs more complexity: From our analysis provided in previous meetings, extending the channel bandwidth by 1/2 of the UE CBW symmetrically would enable the NRB to be equivalent to an Inner RB allocation from an MPR perspective, which aligns with the definition in TS38.101-1. Therefore, we think that extending the channel bandwidth by a maximum of 2 x the UE CBW symmetrically can be used for “symmetrical” UE channel bandwidth extension. For “asymmetrical” UE CBW extension, see section 2.7. If the BS CBW is larger than the extended CBW sizes above, then it would create more configuration and testing complexity for no further benefit in MPR reduction.

To further maximize flexibility of the extended CBW concept, there may be some benefit to consider allowing for extended CBW = BS CBW where the BS CBW is greater than the size of the UE CBW but smaller than the extended CBW. This could be an FFS point.

Proposal 1: Agree maximum value for symmetrical extended CBW = 40MHz for RedCap UE.

- FFS whether to define intermediate extended CBW equal to size of existing specified BS CBW, where BS CBW is in-between {20…40MHz} for RedCap UE.

Proposal 2: Agree maximum value for symmetrical extended CBW = 200MHz for non-RedCap UE

- FFS whether to define intermediate extended CBW equal to an existing specified “aggregated CA BW” size, where spectrum block size is between {100…200MHz} for non-RedCap UE.

Proposal 3: Confirm feasibility for UE in idle mode and connected mode to use existing available SIB information (i.e. carrierBW, offsetToPointA and offsetToCarrier in SIB1, as well as UE BWP location) to check that its extended UE CBW does not exceed the spectrum block size defined if spectrum block size = BS CBW.

  - FFS how the UE would do this if spectrum block size exceeded 100MHz.

2.2 Applicability of IBE in region between UE CBW and ext-CBW

#### At RAN4#112bis, the following was agreed:

#### Issue 1-2-3: Where to use IBE in the larger BS CBW

- WF

  - IBE should be used between edges of UE CBW and extended UE CBW

    - FFS whether IBE could be used in the guard band of UE CBW

    - FFS the full RB allocation scenario

    - FFS the impact to testability

The IBE should be extended to the edge of the extended CBW. If this is not done, then the IBE requirement may end up being tightened compared to the equivalent for a wider bandwidth and counteract the whole benefit of the shifting the OOBE boundary. A simple example is where there is a full RB allocation within the UE CBW. If IBE does not naturally apply in the same way as it would apply to an inner RB allocation, then the full RB allocation cannot be an inner RB allocation. See figure below for what is needed for symmetrical extended CBW of 40MHz (general IBE mask shown for simplicity). The method applies virtual non-allocated RBs within NRB of extended channel bandwidth rather than limiting to NRB of original UE CBW.

Figure 1: IBE extension to extended CBW edge

Proposal 4: Agree that an “extended” IBE shall apply in the region between UE CBW and extended CBW, i.e. where the NRB of extended channel bandwidth in the formula is assumed to apply as opposed to that of the original channel bandwidth.

2.3 Starting point of ΔfOOB for UE ACLR/SEM

#### At RAN4#112bis, the following was agreed:

#### Issue 1-2-4: Boundary to apply ACLR and SEM

- Proposals

  - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW.

  - Proposal 2: ACLR, SEM and spurious emissions would be defined based on BS channel bandwidth.

- WF

  - FFS in next meeting

    - ITU regulation of 250% necessary bandwidth should be considered in further analysis

Spurious Emission requirements are applied from 250% of the center frequency of the "necessary bandwidth" according to ITU-R radio regulations (ITU REC SM.329/328). If ACLR, SEM, and spurious emissions are defined based on the BS channel bandwidth, this may not be compatible with the stipulations of REC SM.329. For instance, consider a UE with a 10MHz CBW located at the center of a 100MHz BS CBW. Spurious Emissions would apply from the center frequency of the UE with a 25MHz (250% of CBW) shift, which is still within the BS channel. Therefore, we propose that ACLR and SEM should be applicable from the edge of the extended UE CBW.

Proposal 5: Agree that ACLR and SEM should be applicable from the edge of extended UE CBW.

  - Spurious Emission FOOB boundary

#### At RAN4#112bis, the following was agreed:

#### Issue 1-2-5: Boundary to apply SE

- Proposals

  - Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW.

  - Proposal 2: SE is applied at BS channel bandwidth.

- WF

  - FFS in next meeting

    - ITU regulation of 250% necessary bandwidth should be considered in further analysis

The Spurious Emission boundary is currently specified to be at the end of the OOBE boundary, and 5MHz away from the ACLR region. As stated in MediaTek documents [2] and [3] in the previous meetings, ITU allows the Spurious Emission to apply 250% of the channel BW from the centre of the channel. 3GPP requirements are more stringent but driven by legacy UMTS rather than regulation. Therefore, shifting the boundary compared to current 3GPP FOOB is feasible.

We do believe that from specification perspective it would be more reasonable to shift the FOOB if we apply an extended CBW. Also from a minimum requirements perspective, if we do not shift the SE boundary but we shift the OOBE, then there would be a point where the two requirements overlap and the Spurious Emission requirement would be more stringent than ACLR/SEM.

Moreover, if the SE boundary is adjusted according to the shift of the edge of the UE CBW, it remains compatible with ITU-R radio regulations. However, where the BS channel bandwidth edges are beyond 0.5 x UE CBW from edge of UE CBW, it may be difficult for the 3GPP ecosystem to claim that this complies with ITU-R radio regulations.

Proposal 6: Agree that the application range of SE should be altered with the shifting of the edge of the UE CBW.

2.5 Integral region of OOBE

At RAN4#112bis, the following was agreed:

#### Issue 1-2-6: Which CBW is utilized as the basis for the OOBE requirement and the applicable boundary

- Proposals

  - Proposal 1: The OOBE requirement and applicable boundary should be based on UE CBW.

  - Proposal 2: The OOBE requirement and applicable boundary is based on BS CBW.

- WF

  - FFS in next meeting

As explained above, if the SE boundary is shifted along with the OOBE region based on the CBW, then it can still meet the ITU requirements for 250% of the UE CBW. Additionally, our simulations suggest that the Inner RB Allocation MPR can still meet OOBE requirements based on UE CBW for all RB allocations within a 20MHz channel with such a shift from the UE CBW edge. If SE was to apply based on UE CBW and OOBE was based on BS CBW, then it would cause quite some complexity to the requirements specification. Therefore, we think it makes sense to apply the same principles for OOBE as proposed for SE.

Proposal 7: Agree that the OOBE requirement and applicable boundary should be based on UE CBW.

2.6 Asymmetrical extended CBW

At RAN4#112bis, the following was agreed:

#### Issue 1-2-8: Asymmetrical extended CBW approach

- WF

  - Asymmetrical extension is considered, FFS the solution.

In the last RAN4 meeting, some companies showed realistic deployment scenarios for asymmetrical extended CBW applications. We do believe that an asymmetrical extended CBW scenario can be considered to support this case. This scenario is depicted in the figure below. This was defined as “one-sided emissions relaxation” in MediaTek documents [2] and [3] submitted to RAN4#110bis and RAN4#111.

Figure 2: Asymmetrical extended CBW

Two aspects to consider for asymmetric channel bandwidths are:

- Based on existing Inner RB allocation definitions, it would not be feasible for a full RB allocation of the CBW to be equivalent to an Inner RB allocation of extended CBW.

- The centre location of the CBW would not be at the centre of the extended CBW, and this may lead to some differences in the leakage profile of the UE transmission and achievable MPR reduction for a given RB allocation.

Simulation analysis on this scenario is provided in section 3 for QPSK only at this stage, and demonstrates that, for an RB allocation of no more than 2/3 of the NRB of the CBW, with 1/3 of NRB unallocated as shown in figure 2, MPR reduction can still be applied.

The UE could determine whether to apply symmetrical or asymmetrical extended CBW based on its knowledge of its BWP location with respect to the spectrum block edge.

Proposal 8: For the case where a UE CBW is allocated at the edge of the BS CBW, agree to consider further an “Asymmetrical extended CBW” to allow outer RB allocation MPR reduction applicability for a UE CBW allocated at the edge of the BS CBW, considering an RB allocation of no more than 2/3 of NRB for QPSK and with 1/3 of NRB unallocated as shown in the figure.

Proposal 9: Consider an extended CBW of 1.5 x CBW (with OOBE shift of 0.5 x UE CBW from UE CBW edge) for UE operating an asymmetrical extended CBW (i.e. where the OOBE relaxation only applies at one side of the spectrum block).

Observation 1: The UE could determine whether to apply symmetrical or asymmetrical extended CBW based on its knowledge of its BWP location with respect to the spectrum block edge.

2.7 Signalling

At RAN4#112bis, the following was agreed:

#### Issue 1-2-10: Signaling aspects

- WF

- To further discuss the signalling aspects in conjunction with the solution to be adopted for MPR reduction.

  - Companies are encouraged to provide thinking on signalling impact together with the proposed solution

There is an increase in IBE within the channel when the OOBE relaxation is applied. The level of impact this has maybe small and may depend on the deployment scenario and system loading. We assume that operators may want to control whether a UE is permitted to apply the OOBE relaxation within a cell. For this reason, we assume some RRC signalling from BS to UE may be useful to indicate that the UE is permitted to perform the OOBE relaxation.

Related to the point raised in section 2.1 and Proposal 3, where the UE may not today know where the spectrum block edge is for the case where a spectrum block is >100MHz, the network may want to inform the UE of this.

Proposal 10: Agree that signalling from BS to UE(s) to inform that UE is permitted to apply the relaxation.

Proposal 11: Consider whether new BS to UE signalling is needed to inform the UE that a spectrum block is not confined within the size of maximum BS CBW specified today.

- 3. Simulation analysis

The following table provides the outcome of our simulation analysis and also evaluated the CP-OFDM and DFT-s-OFDM waveform with 15kHz SCS.

The results are provided assuming SEM/ACLR according to 20MHz CBW, with PC3 requirements assumed. We evaluated the following cases:

- 20MHz with “Symmetrical extended CBW” of 40MHz (see figure 3a)

- 20MHz with “Asymmetrical extended CBW” of 30MHz (see figure 3b)

Figure 3a: Symmetrical extended CBW evaluation scenario

Figure 3b: Asymmetrical extended CBW evaluation scenario

For Asymmetrical extended CBW, the following specific aspects are highlighted:

- The LO was allocated at the centre of the 20MHz CBW, i.e. 10MHz from left edge of 30MHz ext-CBW.

- The maximum evaluated size of LCRB and corresponding RBstart was selected to ensure that the number of non-allocated RBs (LCRB/2) inside the channel provided sufficient guardband to adhere to existing requirements at the left edge of the channel – values for DFTS and CP-OFDM are different due to restricted RB allocation sizes for DFTS-OFDM.

- We only modelled QPSK so far, but plan to evaluate 16QAM for the next meeting.

Table 1a: Evaluation results for Symmetric extended UE CBW

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {0, 100}, {0, 75}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| CP-OFDM, QPSK, 21.5dBm | {0, 106}, {0, 75}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {0, 100}, {0, 75}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| CP-OFDM, 16QAM, 21dBm | {0, 106}, {0, 75}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |

Table 1b: Evaluation results for Asymmetrical extended UE CBW

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {25, 50}, {105, 1} | ALL PASS | ALL PASS |
| CP-OFDM, QPSK, 21.5dBm | {35, 71}, {105, 1} | ALL PASS | ALL PASS |

Observation 2: All modelled RB allocations pass the test. These test points covered the most restrictive cases. We believe that this is sufficient to demonstrate that the requirements would be passed for all RB allocations within NRB of the actual UE CBW.

While not all aspects are modelled, we believed it can also be considered that:

- If the UE fulfils the requirements for OOBE integrated over actual UE CBW, then for OOBE integrated over Extended CBW (40MHz and 30MHz respectively) the requirements would also be fulfilled.

- For 30kHz SCS, the same outcome should also apply scaled for the corresponding NRB size adjustment within the UE CBW.

Observation 3: An extended CBW is shown to be feasible for reducing MPR for Outer RB allocation MPR for PC3, for Symmetrical Extended UE CBW (for QPSK and 16QAM) and for Asymmetrical extended UE CBW (for QPSK, with 16QAM FFS).

- 4. Conclusion

In this contribution, we have presented our perspectives on power domain enhancement for NR single carrier, and have provided the following observations and proposals:

Proposal 1: Agree maximum value for symmetrical extended CBW = 40MHz for RedCap UE.

- FFS whether to define intermediate extended CBW equal to size of existing specified BS CBW, where BS CBW is in-between {20…40MHz} for RedCap UE.

Proposal 2: Agree maximum value for symmetrical extended CBW = 200MHz for non-RedCap UE

- FFS whether to define intermediate extended CBW equal to an existing specified “aggregated CA BW” size, where spectrum block size is between {100…200MHz} for non-RedCap UE.

Proposal 3: Confirm feasibility for UE in idle mode and connected mode to use existing available SIB information (i.e. carrierBW, offsetToPointA and offsetToCarrier in SIB1, as well as UE BWP location) to check that its extended UE CBW does not exceed the spectrum block size defined if spectrum block size = BS CBW.

  - FFS how the UE would do this if spectrum block size exceeded 100MHz.

Proposal 4: Agree that an “extended” IBE shall apply in the region between UE CBW and extended CBW, i.e. where the NRB of extended channel bandwidth in the formula is assumed to apply as opposed to that of the original channel bandwidth.

Proposal 5: Agree that ACLR and SEM should be applicable from the edge of extended UE CBW.

Proposal 6: Agree that the application range of SE should be altered with the shifting of the edge of the UE CBW.

Proposal 7: Agree that the OOBE requirement and applicable boundary should be based on UE CBW.

Proposal 8: For the case where a UE CBW is allocated at the edge of the BS CBW, agree to consider further an “Asymmetrical extended CBW” to allow outer RB allocation MPR reduction applicability for a UE CBW allocated at the edge of the BS CBW, considering an RB allocation of no more than 2/3 of NRB for QPSK and with 1/3 of NRB unallocated as shown in the figure.

Proposal 9: Consider an extended CBW of 1.5 x CBW (with OOBE shift of 0.5 x UE CBW from UE CBW edge) for UE operating an asymmetrical extended CBW (i.e. where the OOBE relaxation only applies at one side of the spectrum block).

Proposal 10: Agree that signalling from BS to UE(s) to inform that UE is permitted to apply the relaxation.

Proposal 11: Consider whether new BS to UE signalling is needed to inform the UE that a spectrum block is not confined within the size of maximum BS CBW specified today.

Observation 1: The UE could determine whether to apply symmetrical or asymmetrical extended CBW based on its knowledge of its BWP location with respect to the spectrum block edge.

Observation 2: All modelled RB allocations pass the test. These test points covered the most restrictive cases. We believe that this is sufficient to demonstrate that the requirements would be passed for all RB allocations within NRB of the actual UE CBW.

Observation 3: An extended CBW is shown to be feasible for reducing MPR for Outer RB allocation MPR for PC3, for Symmetrical Extended UE CBW (for QPSK and 16QAM) and for Asymmetrical extended UE CBW (for QPSK, with 16QAM FFS).

- 5. References

- R4-2417204, “WF on power domain enhancements”, Huawei

- R4-2405619, “Discussion on power domain enhancement for NR single carrier”, MediaTek

- R4-2407910, “Discussion on power domain enhancement for NR single carrier”, MediaTek