3GPP TSG-RAN WG4 Meeting # 113															R4-2419418

Orlando, USA, 18th – 22nd November, 2024

Agenda item:			7.19.2

Source:	Samsung

Title:	Views on UE RF requirements of NR MIMO Phase 5

Document for:	Discussion

1	Introduction

RAN4 has initiated and discussed UE RF aspects of Rel-19 NR MIMO Phase 5 focusing on the objective of 3-antenna-port transmissions in the WID [1]. At its latest meeting, RAN4#112-bis, some meaningful progress was achieved and captured in the WF although it still has many ‘FFS’ on each 3Tx topic [2]. In addition to the WF, RAN4 also sent RAN1 an LS to check their progress and understanding of the WID even though such questions in the LS are very clear to some other companies in RAN4 [3]. In this contribution, we would like to propose our views on the ‘FFS’ parts in the WF including the issues in the LS.

2	Discussion

As mentioned earlier, RAN4 has identified that it is first necessary to study the impact of 3Tx on UE RF requirements. Some agreements were also made for the 3Tx scope as captured in [2]. Regarding the applicable power class for 3Tx, legacy power classes such as PC3/2/1.5 were discussed which are still on the table as below.

<Agreement>: PC for 3Tx requirement - PC for 3Tx requirement - PC3 - PC2 - FFS on PC1.5 - FFS on UE type, i.e., FWA and/or handheld UE - FFS on support of full power mode 0 - RAN4 understanding is that TxD for 3Tx is not in the scope of this WI - Full power mode 0 apply to PC2 and PC3 only. - FFS on for which UE types it should apply.

As highlighted in red above, the WF has FFS on whether to define PC1.5 (29 dBm) for 3Tx operations, which also needs to consider the applicable UE type and/or full power mode 0 of the uplink full power transmission capability (ULFPTx). However, as discussed in past meetings, PC1.5 cannot be supported at current level of maximum single PA output, e.g., 26dBm, in consideration of the mode 0. This is because, in our understanding, mode 0 also means to support a target power class with a single full rated port.

Observation 1:	PC1.5 (29dBm) cannot be supported in case of mode 0.

On the other hand, given the motivation of 3Tx described in [1], and that it is an optional feature for UE to support the uplink full power transmission (ULFPTx) for UL MIMO, PC1.5 can also be considered for the maximum output power for 3Tx. However, since it is not possible for mode 0 as mentioned earlier, in our view, RAN4 should focus on both PC3 and PC2 while waiting for the reply from RAN1. Although PC1.5 is introduced, it should be limited to larger form-factor devices such as FWA market as same as the previous assumption for 4Tx requirements in the specification.

Proposal 1:		RAN4 should focus on both PC3 and PC2 while waiting for the reply from RAN1.

Proposal 2:		Although PC1.5 is introduced, it should be limited to larger form-factor devices such as FWA market as same as the previous assumption for 4Tx requirements in the specification.

Similar situation happened for the supported MIMO layer discussion for mode 0 as captured as FFS below.

<Way forward >: FFS on Supported MIMO layer for ULFPTx mode 0

Although some related questions are captured in the LS to ask RAN1 about 2-layer support for mode 0 in [3], it is our understanding that the supported MIMO layer for mode 0 is independent with the supportive power class of 3Tx for mode 0 since 2-layer can be practically supported as long as a UE supports 3-layer UL MIMO by default. In that sense, whether to support 2-layer for mode 0 is up to RAN4 discussion for PC2 and PC3 since the rank adaptation in real networks would be varying between layer 1, 2 and 3 rather than 1 and 3 only.

Observation 2:	2-layer can be practically supported as long as a UE supports 3-layer UL MIMO by default.

Therefore, RAN4 should allow to capture 2-layer for mode 0 considering PC3 and PC2 for its maximum output powers regardless of the reply LS from RAN1.

Proposal 3:		RAN4 should allow to capture 2-layer for mode 0 considering PC3 and PC2 for its maximum output powers regardless of the reply LS from RAN1.

Regarding maximum power reduction (MPR) for 3Tx, RAN4 has reached some evaluation approach including PA configuration for each power class as below for MPR evaluation.

<Agreement>: Assumption for power splitting between antenna ports - Equal power split among non-zero ports according to RAN1 spec <Agreement>: PA configurations - For MPR evaluation, the assumptions of PA architecture are: - For PC3, 3x23dBm - For PC2 - 3x23dBm - 3x26dBm is not precluded and interested company can provide the evaluation <Agreement>: Different MPR for different UE type - Define separate MPR requirements for handheld and FWA UE <Way forward>: MPR derivation method for 3Tx - Companies are encouraged to evaluate MPR based on existing MPR requirements, e.g., PC, 2/4Tx, UE type, considering 3PA configuration agreed above - Option 1: Reuse legacy MPR requirements for each power class (OPPO, Huawei, Nokia) - Option 2: Further evaluate with 3PA configuration (Apple)

Currently, RAN4 has specified the MPR requirement for each power class. The number of Tx ports and device type have also been considered for the MPR requirements. Table 2 summarizes what RAN4 has defined for UL MIMO related MPR so far.

Table 1: Existing UL MIMO related MPR requirements

| Power class | Number of Tx | Requirements |
| --- | --- | --- |
| PC3 | 1 | Table 6.2.2-1 |
| PC2 | 2 | Table 6.2D.2-1, Table 6.2.2-2 |
| PC1.5 | 2 | Table 6.2D.2-2, Table 6.2D.2-3 (FWA) |
|  | 4 | Table 6.2D.2-4 (vehicular), 6.2D.2-5 (FWA) |

For the MPR requirement with 3Tx, RAN4 is supposed to define new MPR with 3Tx for target power classes to be decided. However, taking into account the existing MPR gap between 2 Tx and 4Tx for PC1.5, up to 1.5 dB gap for FWA devices as an example, it is reasonable for 3Tx to reuse existing MPR tables as much as possible. We’d like to propose Table 2 and Table 3 for PC2 (dual Tx) and PC3, respectively, for the MPR requirement of 3Tx.

Table 2: Proposed MPR for power class 3 with 3 Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1.2 | ≤ 0.2 |
|  |  | ≤ 0.5 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | 0 | 0 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 3: Proposed MPR for power class 2 with 3 Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1 | 01 |
|  | QPSK | ≤ 3.5 | ≤ 2 | 0.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | 16 QAM | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |

Observation 3:	Existing MPR gap between 2 Tx and 4Tx for PC1.5 is up to 1.5 dB gap.

Proposal 4:		It is proposed to reuse existing MPR tables (dual Tx requirements for PC2) given our previous experience and efficient discussion in Rel-19.

Regarding the objective of asymmetric DL sTRP/UL mTRP in [2], RAN4 also had discussed whether it has impact on the UE RF requirement, but sent the LS for RAN1’s confirmation in the end even though such questions in the LS are very clear to some other companies in RAN4 [3]. In our understanding, based on the WID [1] and RAN1 discussion, the main objective on this agenda item is to specify necessary enhancements for supporting UL mTRP deployment scenarios without any DL signal, which is depicted as in Figure 2. TRP1 (i.e., both DL/UL capable TRP) in this scenario can operate both DL transmission and UL reception, while TRP2 (i.e., UL TRP) can only operate UL reception.

Figure 2: Asymmetric DL sTRP/UL mTRP scenario

Basic assumptions of UL TRP are as follows:

-	From UL TRP, there is neither DL control information nor DL RS including pathloss RS (PL RS), hence scheduling/control information for UL TRP could be transmitted from TRP1 (i.e., both DL/UL capable TRP), and pathloss between UL only and a UE can be calculated by PL RS from TRP1, and pathloss offset (PL offset) configuration is additionally considered which is defined as difference between two pathloss values where the 1st pathloss is between TRP1 and UE, and the 2nd pathloss is between TRP2 and UE, respectively.

-	If the UE configured with pathloss (PL) offset and communicating with the UL TRP can receive SSBs, there is no motivation to introduce PL offset to identify pathloss between the UL TRP and the UE.

-	Even though the TRP2 can transmit SSB at a certain case, there is no impact for UR RF discussion since FR2 UE RF requirements are based on the minimum performance from the beginning.

Observation 4:	For asymmetric DL sTRP/UL mTRP scenario, TRP1 (i.e., both DL/UL capable TRP) can operate both DL transmission and UL reception, while TRP2 (i.e., UL TRP) can only operate UL reception.

Observation 5:	If the UE configured with pathloss (PL) offset and communicating with the UL TRP can receive SSBs, there is no motivation to introduce PL offset to identify pathloss between the UL TRP and the UE.

Observation 6:	Even though the TRP2 can transmit SSB at a certain case, there is no impact for UR RF discussion since FR2 UE RF requirements are based on the minimum performance from the beginning.

Proposal 5:		It is proposed for RAN4 not to discuss the potential FR2 enhancement topic on asymmetric DL sTRP/UL mTRP scenarios in UE RF discussion.

3	Conclusion

This contribution focuses on remaining open issues from the outcome of the last meeting [2]. Following observations and proposals were derived for consideration in this meeting.

Observation 1:	PC1.5 (29dBm) cannot be supported in case of mode 0.

Proposal 1:		RAN4 should focus on both PC3 and PC2 while waiting for the reply from RAN1.

Proposal 2:		Although PC1.5 is introduced, it should be limited to larger form-factor devices such as FWA market as same as the previous assumption for 4Tx requirements in the specification.

Observation 2:	2-layer can be practically supported as long as a UE supports 3-layer UL MIMO by default.

Proposal 3:		RAN4 should allow to capture 2-layer for mode 0 considering PC3 and PC2 for its maximum output powers regardless of the reply LS from RAN1.

Observation 3:	Existing MPR gap between 2 Tx and 4Tx for PC1.5 is up to 1.5 dB gap.

Proposal 4:		It is proposed to reuse existing MPR tables (dual Tx requirements for PC2) given our previous experience and efficient discussion in Rel-19.

Observation 4:	For asymmetric DL sTRP/UL mTRP scenario, TRP1 (i.e., both DL/UL capable TRP) can operate both DL transmission and UL reception, while TRP2 (i.e., UL TRP) can only operate UL reception.

Observation 5:	If the UE configured with pathloss (PL) offset and communicating with the UL TRP can receive SSBs, there is no motivation to introduce PL offset to identify pathloss between the UL TRP and the UE.

Observation 6:	Even though the TRP2 can transmit SSB at a certain case, there is no impact for UR RF discussion since FR2 UE RF requirements are based on the minimum performance from the beginning.

Proposal 5:		It is proposed for RAN4 not to discuss the potential FR2 enhancement topic on asymmetric DL sTRP/UL mTRP scenarios in UE RF discussion.

Reference

[1] 	RP-242394,	WID revision: NR MIMO Phase 5, RAN#105

[2] 	R4-2417116,	WF on UE RF requirements for Rel-19 MIMO enhancement, RAN4#112-bis

[3] R4-2417117,	LS on UE RF issues for Rel-19 MIMO enhancement, RAN4#112-bis