3GPP TSG-RAN WG4 Meeting #113									                        R4-2419428

Orlando, US, November 18 – 22, 2024

Agenda item:			7.1.1.3.3

Source:	Ericsson

Title:	On DeltaT_RxSRS requirements for 3Tx and 6Rx UE RF

Document for:	Approval

# Introduction

In RAN#104, a revised WID has been approved for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” WI [1]. One of the three main parts of the WI is to specify 6Rx UE RF requirements for both handheld and FWA UEs, with the following objectives:

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability

    - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability.

      - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this contribution, we focus on the second-to-last bullet point, which is to specify the requirements to support SRS antenna switching. In the last two meetings, the requirements for ‘t1r6’, ‘t2r6’ and ‘t1r6-t2r6’ SRS antenna switching (AS) capabilities have been agreed. Even though ‘t3r6’ AS capability is also included in the WID, it is not currently specified in TS38.306, so only in the revised WID it was agreed for “RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability”. The work in RAN1 has started in RAN1#118bis meeting (Oct. 2024), so in this contribution we will analyze ‘t3r6’, ‘t1r6-t3r6’, ‘t2r6-t3r6’, and ‘t1r6-t2r6-t3r6’ SRS AS capabilities and propose the corresponding ΔTRxSRS requirements.

# Discussion

In the approved WF from RAN4#112-bis [2], the following was captured on the SRS AS requirements topic:

Issue 2-1-1: Whether to consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

Agreement: RAN4 agrees to consider additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

Issue 2-2-1: Proposed ∆TRxSRS values for t1r6, t2r6, and t1r6-t2r6

Agreement: RAN4 agrees to adopt the values for Bands n79 and n104 as specified below.

Table: ΔTRxSRS values

| Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n79 | 5.5 | 5.0 | 6.0 |
| Band n104 | 6.5 | 5.5 | 7.0 |

Issue 2-2-2: Proposed ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6

Agreement: As this is the first RAN4 meeting since the confirmation that RAN1 will consider t3r6, companies are encouraged to provide ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 at RAN4#113 for further discussion with the Apple and Qualcomm proposals in R4-2415324 and R4-2416264, respectively.

ΔTRxSRS is a maximum allowed relaxation applied to the lower bound of the maximum configured transmitted power PCMAX,f,c due to the additional insertion loss of a given diversity branch relative to the main branch. The main “contributors” to the insertion loss are the antenna switches, filters, as well as PCB trace/RF cable routing located between the PA which sounds the given SRS port and the antenna port/connector.

ΔTRxSRS is measured at the antenna connector, and it is important not to have too loose margins in order to limit degradation of DL-MIMO performance, i.e. to have an adequate reciprocal CSI-RS estimation, especially since the gNB measures the resulting radiated SRS power per SRS port(s) which include different antenna element gains (in a typical implementation, antenna elements on diversity paths have a poorer quality compared with the antenna elements on the main path).

As it can be seen from the approved WF, ΔTRxSRS requirement has been agreed for ‘t1r6’, ‘t2r6’ and ‘t1r6-t2r6’ AS capabilities for all example bands. The remaining work to be done on this topic is to specify ΔTRxSRS requirements for ‘t3r6’, ‘t1r6-t3r6’, ‘t2r6-t3r6’, and ‘t1r6-t2r6-t3r6’ SRS AS capabilities.

## ‘t3r6’ SRS AS capability

The typical implementation architecture for ‘t3r6’ SRS AS capability (where no fallback to 1 PA or 2 PA is needed to be supported) corresponds to having three ‘t1r2’ architectures in parallel as shown on Figure 1 (there are other possibilities as well).

The exact value of the insertion loss introduced by each antenna switch, PCB trace/RF cable routing etc. depends on the particular component being used where values can vary considerably between different manufacturers. In addition, for the given component, the insertion loss depends mainly on the operating frequency point and for that reason in current specifications there are different ΔTRxSRS requirements for bands whose FUL_high is higher than the FUL_low of n79 and for bands whose FUL_high is lower than the FUL_low of n79. As it can be seen from the WF [2], an additional breakpoint was added for bands whose FUL_high is higher than the FUL_low of n104. The insertion loss of the given component also depends on the temperature conditions, but we will assume normal temperature conditions in all cases.

Figure 1: An example of UE architecture for ‘t3r6’ SRS antenna switching.

For bands whose FUL_high is lower than the FUL_low of n79, in current specifications [3] ΔTRxSRS is specified as 3 dB for ‘t1r2’ SRS AS capability where at the time of its specification both handheld and FWA/CPE types of UE were considered. For bands whose FUL_high is higher than the FUL_low of n79, up to Rel-18 the practice has been to adopt 1.5dB higher value than for bands whose FUL_high is lower than the FUL_low of n79, and the same principle has been reused in this WI. Similar to the agreement for ‘t2r6’ SRS AS capability (where the insertion loss is higher compared with ‘t3r6’ AS capability), 0.5dB increase compared with bands whose FUL_high is higher than the FUL_low of n79 band could be adopted for band n104.

Hence, we propose the following:

Proposal 1: For ‘t3r6’ SRS AS capability:

- For bands n41 and n77/78 ΔTRxSRS should be specified as 3.0 dB;

- For band n79 ΔTRxSRS should be specified as 4.5 dB;

- For band n104 ΔTRxSRS should be specified as 5.0 dB.

## ‘t1r6-t3r6’ SRS AS capability

For ‘t1r6-t3r6’ SRS AS, one out of three PAs should be implemented as a fallback in case the other two are switched off. One example architecture is shown on Figure 2.

Figure 2: An example of UE architecture for ‘t1r6-t3r6’ SRS antenna switching.

This architecture can be compared with ‘t1r6’ SRS AS architecture (for example, see [4]), where the IL of the “worst-cased” positioned antenna for ‘t1r6-t3r6’ SRS AS is equal to the IL of the “worst-cased” positioned antenna for ‘t1r6’ AS. For bands whose FUL_high is lower than the FUL_low of n79, in RAN4#112 ΔTRxSRS for ‘t1r6’ SRS AS was agreed to be 4.0 dB, and the same could be specified for ‘t1r6-t3r6’ SRS AS. For bands whose FUL_high is higher than the FUL_low of n79 and for bands whose FUL_high is higher than the FUL_low of n104, 1.5 dB and 2.0 dB increase compared with bands whose FUL_high is lower than the FUL_low of n79 could be adopted, respectively.

Proposal 2: For ‘t1r6-t3r6’ SRS AS capability:

- For bands n41 and n77/78 ΔTRxSRS should be specified as 4.0 dB;

- For band n79 ΔTRxSRS should be specified as 5.5 dB;

- For band n104 ΔTRxSRS should be specified as 6.0 dB.

## ‘t2r6-t3r6’ SRS AS capability

One example of ‘t2r6-t3r6’ SRS AS architecture is shown on Figure 3. One out of three PAs serves as a fallback for another PA in the case it is switched off.

Figure 3: An example of UE architecture for ‘t2r6-t3r6’ SRS antenna switching.

The IL of the “worst-cased” positioned antenna for ‘t2r6-t3r6’ SRS AS is comparable to the IL of the “worst-cased” positioned antenna for ‘t1r6-t3r6’ SRS AS. It can actually be a slightly lower, as much as the IL of SP3T antenna switch is smaller than the IL of SP4T antenna switch.

Since that difference is very small, we propose the following:

Proposal 3: For ‘t2r6-t3r6’ SRS AS capability:

- For bands n41 and n77/78 ΔTRxSRS should be specified as 4.0 dB;

- For band n79 ΔTRxSRS should be specified as 5.5 dB;

- For band n104 ΔTRxSRS should be specified as 6.0 dB.

## ‘t1r6-t2r6-t3r6’ SRS AS capability

The architecture for ‘t1r6-t2r6-t3r6’ SRS AS is in general more complex than for the previous cases, where one example is shown on Figure 4.

Figure 4: An example of UE architecture for ‘t1r6-t2r6-t3r6’ SRS antenna switching.

From the perspective of the IL of the “worst-cased” positioned antenna for ‘t1r6-t2r6-t3r6’ SRS AS, it can be derived from the IL of the “worst-cased” positioned antenna for ‘t1r6-t3r6’ SRS AS, by adding the value of the insertion loss of DP3T antenna switch, which is roughly equal to 1.0 dB at 3.5 GHz frequency.

Thus, we make the following proposal:

Proposal 4: For ‘t1r6-t2r6-t3r6’ SRS AS capability:

- For bands n41 and n77/78 ΔTRxSRS should be specified as 5.0 dB;

- For band n79 ΔTRxSRS should be specified as 6.5 dB;

- For band n104 ΔTRxSRS should be specified as 7.0 dB.

# Conclusion

Based on the discussion in the previous sections we have made the following proposals:

Proposal 1: For ‘t3r6’ SRS AS capability:

- For bands n41 and n77/78 ΔTRxSRS should be specified as 3.0 dB;

- For band n79 ΔTRxSRS should be specified as 4.5 dB;

- For band n104 ΔTRxSRS should be specified as 5.0 dB.

Proposal 2: For ‘t1r6-t3r6’ SRS AS capability:

- For bands n41 and n77/78 ΔTRxSRS should be specified as 4.0 dB;

- For band n79 ΔTRxSRS should be specified as 5.5 dB;

- For band n104 ΔTRxSRS should be specified as 6.0 dB.

Proposal 3: For ‘t2r6-t3r6’ SRS AS capability:

- For bands n41 and n77/78 ΔTRxSRS should be specified as 4.0 dB;

- For band n79 ΔTRxSRS should be specified as 5.5 dB;

- For band n104 ΔTRxSRS should be specified as 6.0 dB.

Proposal 4: For ‘t1r6-t2r6-t3r6’ SRS AS capability:

- For bands n41 and n77/78 ΔTRxSRS should be specified as 5.0 dB;

- For band n79 ΔTRxSRS should be specified as 6.5 dB;

- For band n104 ΔTRxSRS should be specified as 7.0 dB.

# References

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN#104.

- R4-2417209, “WF on 6Rx UE requirements”, AT&T, RAN4#112-bis.

- 3GPP TS38.101-1 V18.7.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

- R4-2413359, “On DeltaT_RxSRS requirement for 6Rx UE RF”, Ericsson, RAN4#112.