3GPP TSG-RAN WG4 Meeting #113	R4-2419455

Orlando, US, 18 – 22 November, 2024

Source:	Nokia

Title:	Discussion on Fragmented Carrier impacts on UE RF requirements

Agenda item:	7.6.3

Document for:	Discussion

Introduction

Agreements at previous meetings

The latest WF for NR FR1 DL fragmented carriers study [1] presents the open issues as found below:

#### Issue 2-3: Signaling related
(…)

#### Issue 3-3: Observations of impacts on DL Rx requirements

RAN4 discuss if following general aspects can be agreeable:

  - The largest interferer power in-gap shouldn’t larger than -25dBm for fragmented carriers using a single Rx RF chain

  - It is proposed to consider only same SCS for both CCs to ease receiver design for the common RF chain

  - ΔRIBNC for SCC may need to be updated to reflect the filter rejection reduction of common RF chain compared to separate RF chains for each intra-band non-contiguous CA band combination. Furthermore, ΔRIBNC needs to apply to PCC too, although the value of ΔRIBNC can be different for PCC and SCC

  - RAN4 shall not consider requirements for fragmented carriers which will result in performance degradation as compared to legacy NC IB CA or – only in case of a strong in-gap interferer – single CC operation

Discussion

Impacts on DL Rx requirements

The study item is currently active at identifying the DL Rx requirements of the UE. We present the Rx requirements mapped onto the example bands for further discussion.

The in-gap interference case of PCS (n25) in Toronto, that is an exemplary case with a large gap (25 MHz) when configured with two fragmented carriers CC1 and CC2 and – deviating from the available channel BWs for the following explanations – taking the receiver requirements of the minimum carrier bandwidth of 5 MHz, is shown in Fig. 1.

Receiving two fragmented component carriers of 5 MHz carrier bandwidth and equal PSD, the requirements map as indicated in the following figures that re-use the exemplary bands from [2].

Fig. 1: Example of fragmented spectrum with in-gap interference

Shifting CC2 of the grey operator, we indicate also the out-of-gap interferers for the carrier bandwidths of 5MHz, as shown in Fig. 2. All cases of adjacent-channel interference (ACI) and in-band blocking (IBB1 for case 1 and IBB2 for case 2) may be present including narrow-band blocking (NBB, not shown) in the exemplary band.

Fig. 2: Example of fragmented spectrum with adjacent transmissions

All cases of adjacent-channel interference and in-band blocking may be present.

On the matter of the collocated scenario, it may be assumed that there is only one source for the in-gap interference with a constant PSD, but there is a difference in PSD between collocated operators as shown in Fig.  3, which deviates from the requirements as shown in the former figures. While it may be the case for the exemplary band with the large gap that there is in-gap interference from only one operator, it is not necessarily granted that the PSDs of the different operators are equal in the minimum 5MHz carrier bandwidth intervals.

Fig. 3: Example of fragmented spectrum with adjacent transmissions from single cell

In Fig. 4 we show the exemplary case of BRS (n7) with two potential network deployments leading to unequal PSD of the in-gap interference relative to the equal PSD of the carriers CC1 and CC2. Note that we have corrected the original text at the asterisk of [2], so that each block is 10 MHz and the total n7 downlink bandwidth is 70 MHz, but kept the in-gap transmissions in the 5 MHz BW of TS 38.101-1 defining the BWInterferer.

Fig. 4: Example of fragmented spectrum with adjacent transmissions from multiple cells

And lastly the exemplary band n66 with similar presence of more than one operator in the gap between the two red CCs, CC1 and CC2 as show in Fig. 5.

Fig. 5: Example of fragmented spectrum with adjacent transmissions from two cells

There are plenty of reasons for higher PSD of in-gap interference despite co-location:

- different sector orientation and down-tilt,

- different antenna layout and beamforming gain,

- different fading and

- different Tx power or PSD

Furthermore, because of handover ping-pong mitigation (e.g. hysteresis), the UE may not be connected to the best possible serving cell, and the strongest in-gap interference may come from a different site than the site of the current serving cell. In the case of a network densification, a new site may not be taken into use simultaneously by the collocated operators. All of these listed reasons for different PSD may happen in the field.

There are no guarantees regarding which operators have what panels on a shared antenna site. The situation can therefore easily be that all operators have sectors pointing to the same direction. However, the coverage expectations for each operator may also be different and therefore the power may vary between the operators depending on the density of sites for the individual operator in a given area.

There can be a lot of variation between operators on same site and cross site depending on site density in a given area and it is given that operators will have large overlap geographically to cover the same customer base as the customers will be in the same directions for all operators.

In the field, co-location cannot safely preclude adverse in-gap interference conditions for fragmented carriers, hence the DL should, at interference conditions as specified for non-contiguous intra-band CA, provide at least the same user experience as with UEs not supporting fragmented carriers.

From the figures of the exemplary bands above, especially n7 and n66, it is obvious that with more than one operator active within the gap, there may be larger power variations. For this reason, it is not enough to consider only PSD, which is averaging power, but the peak power in the Rx channel filter BW should be considered too. This must be considered for the aspects of UE measurements.

It is obvious that with more than one operator active within the gap, there may be larger power variations. For this reason, it is not enough to consider only PSD, which is averaging power, but the peak power in the Rx channel filter BW should be considered too.

For a new UE capability for fragmented carrier support to make sense, the feature should provide at least at low and moderate in-gap interference the same user experience as required for non-contiguous intra-band CA. To prevent end user dissatisfaction, the user experience should always be at least on par with UEs not supporting fragmented carriers. Strong in-gap interference may require changing the UE's configuration, as a fallback, from receiving both CCs with all antennas by fully shared RF chains (cf. the reference architecture for fragmented carriers [1]) e.g.

- to receiving only the PCELL using all antennas or,

- if 4 antennas are available in a band where only 2 antennas are mandated, to receiving each CC with only 2 antennas, respectively.

Study how, depending on the in-gap interference, a UE may transition between carrier aggregation configurations that support fragmented carrier operation and carrier configurations that do not.

RAN4 shall not consider requirements for fragmented carriers which will result in a performance degradation compared with legacy non-contiguous intra-band CA or – only in case of a strong in-gap interferer – single CC operation.

A key aspect of the RF performance requirements for UEs supporting fragmented carriers will be the level of in-gap interference up to which these UEs must fulfil the throughput requirement for non-contiguous intra-band CA. The robustness against in-gap interference is not the only challenge for a fully shared RF chain. However, other challenges, e.g. related to sensitivity or out-of-gap interference (see section 2.2), may be solved by a smaller UE design implementation margin or by a performance improvement of the Rx RF chain.

In fragmented carrier operation, UEs should not need relaxations for sensitivity or out-of-gap interference, but they may need to achieve the throughout required for non-contiguous intra-band CA only in a limited range of the in-gap interference level, subject to discussion in RAN4.

UE channel filter

The adjacent channel selectivity and narrow/in-band blocking will be challenged when the UE applies a wider analogue channel filter, since the filter will not attenuate the in-gap interference between the, at least, two fragmented carriers. Moreover, also the out-of-gap adjacent channels will be subject to less attenuation from the analogue filter. If the analogue channel filter of the UE is of a fixed order, the gain relates to the bandwidth of the filter and drops in decades to the filter bandwidth. Since the fragmented carriers may be of 5 MHz channel bandwidth each and separated as much as the lowermost and uppermost channels of the entire operation band, the adjacent channel of same bandwidth (5 MHz) will become significantly less attenuated. As presented in Fig. 6, plotting various filter bandwidths (5, 10, 20, 30, 40, 50, 60, 80 and 100 MHz) and aligning the filters so that the upper 3dB cut-off frequency aligns for all bandwidths, we can compare in the domain of the positive frequencies the attenuation for the various filter bandwidths: The wider the channel filter is, the weaker would be its out-of-gap interference suppression if the filter order did not increase.

Fig. 6: Channel Filter Response Examples

We propose that this effect is noted in the technical report explaining the consequence of applying the wider analogue filter in the fully shared RF chain.

Guardband considerations

The analogue filter bandwidth also has an impact on the received fragmented carriers, in particular in the case of the minimum carrier bandwidth of 5 MHz and a large frequency offset – a frequency span of up to 100 MHz needs to be considered according to the SID.

For two fragmented carriers with 5 MHz channel BW each (-25 MHz to -20 MHz, 20 MHz to 25 MHz) and a gap of 40 MHz between them, Fig. 7 sketches two dedicated Rx channel filter responses in blue colour and, at the same filter order, an Rx channel filter with a passband of 50 MHz in red colour. The sketch indicates a challenge for the single, 50 MHz wide Rx channel filter in red colour to provide the same low attenuation inside the fragmented carriers' transmission BW configurations and the same stop-band attenuation outside the Rx channel filter BW as the two dedicated Rx channel filters in blue colour.

Fig. 7: 50 MHz channel filter and 5 MHz channel filters at the top and the bottom of the 50 MHz span

At a fixed filter order, supporting a narrow guard band between a wanted carrier's transmission BW configuration (passband) and a potential adjacent channel interferer (stopband) is the more difficult, the wider the filter is.

Fig. 8 shows the exemplary band n25. Let us focus on the operator with the grey spectrum. We see that the span needed for receiving the fragmented carriers is 50 MHz – this determines the Rx channel filter BW for the fragmented carrier operation. The minimum CC BW is 10 MHz, and the minimum guard band for a 10 MHz wide carrier is, according to TS 38.101-1 table 5.3.3-1 for 15 kHz SCS, 312.5 kHz. An Rx channel filter with a BW of 50 MHz, however, usually needs to be only designed for a guard band of 692.5 kHz according to the same table. The difference in guard band size is more than 2 RBs wide. This means that, to avoid too large attenuation of the outermost 2 RBs by a transition BW of the Rx channel filter designed according to the minimum guard band of a 50 MHz wide carrier instead of a 10 MHz wide CC, a UE supporting fragmented carrier operation may need a rather high filter order of the Rx channel filter.

Fig. 8: Fragment separation for n25

There is a precedent for the need of a rather high filter order of the Rx channel filter: In the case of contiguous intra-band CA – assuming that the aggregated CCs are received by a single Rx RF chain – the Rx channel filter for the joint reception of the adjacent CCs needs to be at least twice as wide as for the smallest CC. The requirements for the minimum guard bands, however, are rather based on the minimum guard bands for each CC than on the total BW of the Rx channel filter. UEs supporting contiguous intra-band CA using a single Rx RF chain can obviously cope with a guard band that is narrower than the minimum guard band listed in TS 38.101-1 table 5.3.3-1 for the Rx channel filter BW. In fragmented carrier operation, however, the ratio between the narrowest CC BW and the total Rx channel filter BW can be more extreme. Thus, the difference in minimum guard band according to TS 38.101-1 table 5.3.3-1 for the Rx channel filter BW and the minimum guard band for the narrowest CC of the fragmented carriers may be more challenging than in contiguous intra-band CA. Furthermore, contrary to the channel spacing flexibility in contiguous intra-band CA according to TS 38.101-1 subclause 5.4A.1, the network cannot increase the outermost guard bands of the fragmented carriers.

In CA, the Rx channel filter needs to support a narrower guard band than what is needed for a single carrier filling the entire Rx channel filter BW, and this aspect may require a higher filter order for fragmented carrier operation than for contiguous intra-band CA.

In order to achieve the same throughput as in legacy non-contiguous intra-band CA, the transmission BW configuration in terms of number of RBs cannot be reduced for increasing the guard band.

UEs supporting fragmented carriers must cope with narrower guard bands than the minimum guard band specified for a single carrier filling the entire Rx channel filter BW.

Impact of image

In Fig. 9 we present variants of fragmented carrier configurations that may create image interference using band n25 as the example. Related to the interference the placement of the UE downlink local oscillator (LO) is shown to be placed at the centre frequency of the gap, creating a symmetric downlink. The asymmetric cases are also shown where the LO placement may be at the wanted carrier edge or even inside the wanted carrier bandwidth of one of the fragmented carriers (CC1 in Fig. 9).

Fig. 9: LO placements with fragments in n25

The LO placement determines the location of the image interference, and the gap determines the affected range of the component carrier subject to image interference.

From the exemplary cases that studied the in-gap interference types, the strongest in-gap interference of the IBB2 can be used to evaluate the assumed image interference level. At -44dBm IBB2 interference with the UE receiving one of the CCs at Refsens+6dB, the image interference absolute level would be -44dBm -25dB= -69dBm, which is much stronger than Refsens + 6dB. Since it is only the UE that may determine the exact signal strength of the interference it is necessary to receive measurement information from the UE of the interference strength and affected subcarriers/BWP of the carrier that is subject to image interference.

Only the UE may determine the exact signal strength of the interference. It is necessary to receive measurement information from the UE of the interference strength and affected subcarriers/BWP of the carrier that is subject to image interference.

With information of the interference signal strength (average and peak), the network can assist the UE with scheduling restrictions in subcarriers/BWP affected by the image interference. It would be beneficial to discuss measurements of the in-gap interference for the sake of signaling and scheduling to determine how image interference is included in the UE receiver requirements.

It would be beneficial to discuss measurements of the in-gap interference for the sake of signaling and scheduling to determine how image interference is included in the UE receiver requirements.

TDD operation

Based on Fig. 9 showing the LO placement of the downlink for symmetric and asymmetric fragmented carrier reception, we would also like attention towards the LO placement in the uplink for TDD operation. One further RF architecture issue is the number of LO’s used when the fragmented carriers are TDD carriers and one of these carriers has uplink. In this case, it shall be considered whether the assumption of using a single LO is still in place or whether a second LO is needed for the uplink, as shown in Fig. 10.

RAN4 to clarify the number of LO’s needed for the use case of the fragmented carriers in TDD with one of the carriers having UL allocation.

Fig. 10: Multiple LO placements in the uplink

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

Observation 1: All cases of adjacent-channel interference and in-band blocking may be present.

Observation 2: There can be a lot of variation between operators on same site and cross site depending on site density in a given area and it is given that operators will have large overlap geographically to cover the same customer base as the customers will be in the same directions for all operators.

Observation 3: In the field, co-location cannot safely preclude adverse in-gap interference conditions for fragmented carriers, hence the DL should, at interference conditions as specified for non-contiguous intra-band CA, provide at least the same user experience as wth UEs not supporting fragmented carriers.

Proposal 1: It is obvious that with more than one operator active within the gap, there may be larger power variations. For this reason, it is not enough to consider only PSD, which is averaging power, but the peak power in the Rx channel filter BW should be considered too.

Proposal 2: Study how, depending on the in-gap interference, a UE may transition between carrier aggregation configurations that support fragmented carrier operation and carrier configurations that do not.

Proposal 3: RAN4 shall not consider requirements for fragmented carriers which will result in a performance degradation compared with legacy non-contiguous intra-band CA or – only in case of a strong in-gap interferer – single CC operation.

Proposal 4: In fragmented carrier operation, UEs should not need relaxations for sensitivity or out-of-gap interference, but they may need to achieve the throughout required for non-contiguous intra-band CA only in a limited range of the in-gap interference level, subject to discussion in RAN4.

Observation 4: At a fixed filter order, supporting a narrow guard band between a wanted carrier's transmission BW configuration (passband) and a potential adjacent channel interferer (stopband) is the more difficult, the wider the filter is.

Observation 5: In CA, the Rx channel filter needs to support a narrower guard band than what is needed for a single carrier filling the entire Rx channel filter BW, and this aspect may require a higher filter order for fragmented carrier operation than for contiguous intra-band CA.

Proposal 5: UEs supporting fragmented carriers must cope with narrower guard bands than the minimum guard band specified for a single carrier filling the entire Rx channel filter BW.

Observation 6: Only the UE may determine the exact signal strength of the interference. It is necessary to receive measurement information from the UE of the interference strength and affected subcarriers/BWP of the carrier that is subject to image interference.

Proposal 6: It would be beneficial to discuss measurements of the in-gap interference for the sake of signaling and scheduling to determine how image interference is included in the UE receiver requirements.

Proposal 7: RAN4 to clarify the number of LO’s needed for the use case of the fragmented carriers in TDD with one of the carriers having UL allocation.

- References

- R4-2417201, WF on NR FR1 DL fragmented carriers study, MediaTek Inc.

- RP-240503, Fragmented carriers in the DL, Telus, Bell Mobility and more.