3GPP TSG-RAN WG4 Meeting #113						R4-2419471

Orlando, US, November 18 – 22, 2024

Source:	Nokia

Title:	Conformance testing of Expected EIRP mask requirement

Agenda item:       7.11.4.1 U6GHz BS EEIRP mask [NR_BS_RF_req_evo-Perf]

Document for:	Approval

- Introduction

TSG RAN#103 meeting approved new Rel-19 work item “NR base station (BS) RF requirement evolution for FR1/FR2 and testing” [1], where first objective is related to expected EIRP mask for upper 6 GHz.

WRC-23 has identified the frequency range 6425 – 7125 MHz which is 3GPP band n104 for IMT as in the RR Footnotes 5.457D, 5.457E and 5.457F. The IMT identification comes with some technical conditions which includes the limits on the expected equivalent isotropic radiated power (e.i.r.p.) spectral density of IMT base stations for protecting Earth-to-space fixed satellite services (FSS) as contained in ITU-R Resolution 220 (WRC-23) [2].

Performance part of expected EIRP mask objective of work item [1] is as follow:

Expected EIRP mask for upper 6GHz - Specify manufacturer declarations, conformance test descriptions, test tolerances and test requirements. - Note: target completion date of this objective is December 2024.

RAN4#112 meeting discussions were concluded in agreed WF [3] and in this contribution we continue discussions for conformance testing for expected EIPR mask requirement for the upper 6 GHz.

We provide also text proposal (TP) with for clause 6 [4] to TR 38.908 (Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz).

In addition, we provide draft CR that includes test procedure details for BS conformance specification TS 38.141-2 in contribution [5] and draft CR for Annex N.

- Discussion

In this section we provide our views and proposals for open issues and details related to the testing of OTA spatial emission requirements.

  - Test vectors

The EEIRP calculation involves calculating the average e.i.r.p. over a defined set of elevation angles (around the base station, i.e. 360° azimuth) for the full range of beamforming directions. An example of the full range of beamforming directions is shown in Figure 2.1-1, with the electrical steering ranges set to 0° ≤ θetilt ≤ 16° and 60° ≤ -φescan ≤ +60°. The number of beamforming directions for this example is 2057 for a 1° steering resolution (i.e. 17 x 121). This figure is from the viewpoint of the UE into the BS antenna panel.

Figure 2.1-1: Reference Set of beamforming directions

Theoretically, this full range of beamforming directions are meant to be used for the EEIRP calculation, however, a representative subset of these beamforming directions can be considered as long as the overall averaging of the EEIRP is sufficiently accurate (as described in the Annex to the ITU Resolution 220). The motivation to create a small subset of beamforming directions (or Test Beams) is to reduce the test effort needed to calculate the EEIRP.

    - 2.1.1 Subset of beamforming directions

It is undoubtably that using the full range of beamforming directions will amount to extensive test effort as the e.i.r.p. for the entire upper hemisphere (around the base station) needs to be measured for every beamforming direction. Simplistically, halving the number of beamforming directions could potentially half the amount of effort needed to comply with the EEIRP requirement. Therefore, it is important to find a representative subset of these beamforming directions (Test Beams set) for the EEIRP calculation.

Six different sets of Test Beams are used to evaluate the accuracy compared to the Reference Set (i.e. full range of beamforming directions). These sets are shown in Figure 2.1.1-1 with different number of Test Beams; Test Beams Set 1, 2, 5 and 6 have 9 Test Beams, while Test Beams Set 3 has 17 Test Beams and Set 4 has 25 Test Beams.

Figure 2.1.1-1: Test Beams sets

Each of the Test Beams in their respective sets have equal weight.

    - 2.1.2 Antenna configurations for evaluation

These six different Test Beams sets are evaluated over six different antenna configurations to show how suitable these Test Beams sets are. The antenna parameters for these configurations are shown in Table 2.1.2-1 where Config 1 and 2 are non-sub-array model, whilst Config 3 to 6 are sub-array model.

| Parameter |  | Antenna |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| Element | Vertical HPBW (°) | 90 | 65 | 65 | 65 | 65 | 65 |
|  | Horizontal HPBW (°) | 90 | 90 | 90 | 90 | 90 | 90 |
|  | Element gain (dBi) | 5.5 | 5.5 | 6.4 | 6.6 | 6.6 | 6.8 |
|  | Front-to-back ratio (H/V) (dB) | 30 | 30 | 30 | 30 | 30 | 30 |
| Array configuration | Number of rows | 16 | 16 | 8 | 8 | 8 | 16 |
|  | Number of columns | 8 | 8 | 16 | 8 | 16 | 16 |
|  | Sub-array | - | - | 3x1 | 2x1 | 3x1 | 2x1 |
|  | Sub-array pre-tilt: (°) | - | - | 6 | 0 | 0 | 0 |
|  | Vertical spacing (λ) | 0.5 | 0.7 | 2.1 | 1.4 | 2.1 | 1.4 |
|  | Horizontal spacing (λ) | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |

Table 2.1.2-1: Antenna parameters

These parameters are just some example antenna configurations for the purpose of evaluating the suitability of the Test Beams sets rather than for evaluating against the specific EEIRP values required for the different elevation ranges. In addition, no specific antenna techniques such as sidelobe suppression are used where these techniques could potentially lower the EIRP level at various vertical and horizontal angles.

    - 2.1.3 Evaluation of Test Beams set

The Test Beams sets in clause 2.1.1 are used to evaluate against the antenna configurations in clause 2.1.2. The Reference Set illustrated in Figure 2.1-1 are based on 1° steering resolution, and the calculation of the EEIRP above the horizon (upper hemisphere) is based on 0.5° sampling resolution in order to capture the beam as accurately as possible.

The antenna pattern generated from these configurations were normalised, and just to note that the normalisation factor is only up to around 1.5 dB. The preliminary results shown in Table 2.1.3-1 to Table 2.1.3-6 is in the form of difference (delta) in dB comparing the EEIRP generated from those Test Beams set to the Reference Set as shown in Figure 2.1-1.

| Elevation Bin | Elevation angular range | Delta (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 9 Beams Inner Box |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | -1.4608 | -3.2679 | -5.0841 | -3.0766 | -4.728 | -4.5803 |
| 2 | 5<q<10 | -0.0081 | -0.516 | -0.5236 | -0.4393 | -0.5365 | -0.2118 |
| 3 | 10<q<15 | -0.8805 | -0.2915 | -0.6284 | -0.3543 | -0.1397 | -0.0243 |
| 4 | 15<q<20 | 0.1301 | -0.226 | -0.6791 | -0.691 | -0.4065 | -0.1564 |
| 5 | 20<q<30 | -0.1437 | 0.088 | -0.6729 | -0.466 | 0.1303 | -0.1388 |
| 6 | 30<q<60 | 0.234 | 0.3252 | -0.0622 | 0.2937 | 0.3319 | -0.7588 |
| 7 | 60<q<90 | 0.3149 | 1.03 | -0.1047 | 1.1276 | -0.2916 | 0.5118 |

Table 2.1.3-1: Difference of Test Beams Set 1 from Reference Set

Observation 1: For Test Beams Set 1, the highest over-estimation is 1.1 dB, and the lowest under-estimation is 5.1 dB.

| Elevation Bin | Elevation angular range | Delta (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 9 Beams Diamond |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | -0.3116 | 0.0388 | 0.8465 | 0.0309 | 0.8698 | 1.2551 |
| 2 | 5<q<10 | -0.0189 | -0.4121 | 0.0054 | -0.3496 | 0.0446 | -0.0898 |
| 3 | 10<q<15 | 0.1425 | -0.2527 | -0.8333 | -0.2389 | -0.9498 | -0.0487 |
| 4 | 15<q<20 | -0.2052 | -0.1904 | 0.4329 | -0.2258 | 0.6246 | -0.0038 |
| 5 | 20<q<30 | -0.0286 | -0.1016 | -0.4885 | -0.4863 | 0.7584 | -0.8374 |
| 6 | 30<q<60 | -0.0328 | 0.0878 | 0.228 | 0.0239 | 0.111 | 0.1813 |
| 7 | 60<q<90 | 0.7831 | 1.0257 | 3.1605 | 1.1277 | -0.8218 | 1.6613 |

Table 2.1.3-2: Difference of Test Beams Set 2 from Reference Set

Observation 2: For Test Beams Set 2, the highest over-estimation is 3.2 dB, and the lowest under-estimation is 0.9 dB.

| Elevation Bin | Elevation angular range | Delta (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 17 Beams Snowflake |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | -0.6299 | -0.7396 | -0.7994 | -0.7069 | -0.7315 | -0.6978 |
| 2 | 5<q<10 | -0.0951 | -0.2337 | -0.2436 | -0.1939 | -0.2362 | -0.2557 |
| 3 | 10<q<15 | -0.1189 | -0.1198 | -0.6819 | -0.136 | -0.6099 | -0.1412 |
| 4 | 15<q<20 | -0.1046 | -0.0779 | 0.2025 | -0.2279 | 0.3424 | -0.1541 |
| 5 | 20<q<30 | -0.0195 | -0.0259 | -0.5882 | -0.4999 | 0.5688 | -0.6117 |
| 6 | 30<q<60 | 0.0312 | 0.0723 | -0.0123 | 0.2526 | 0.3558 | 0.1325 |
| 7 | 60<q<90 | 0.3583 | 0.2292 | 0.8383 | 0.3112 | -0.6236 | 0.7318 |

Table 2.1.3-3: Difference of Test Beams Set 3 from Reference Set

Observation 3: For Test Beams Set 3, the highest over-estimation is 0.8 dB, and the lowest under-estimation is 0.8 dB.

| Elevation Bin | Elevation angular range | Delta (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 25 Beams Outer Box |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | Delta | Delta | Delta | Delta | Delta | Delta |
| 2 | 5<q<10 | 1.5134 | 2.1164 | 2.9124 | 2.0429 | 2.8129 | 3.4824 |
| 3 | 10<q<15 | 0.3272 | 0.2331 | 1.6075 | 0.1972 | 1.635 | 0.4246 |
| 4 | 15<q<20 | 0.5964 | 0.0476 | 0.9857 | 0.1416 | 0.692 | 0.2883 |
| 5 | 20<q<30 | 0.0521 | -0.0549 | -0.2774 | 0.4128 | -0.3568 | 0.6033 |
| 6 | 30<q<60 | -0.1762 | -0.2056 | 0.7284 | 0.7612 | -0.6394 | 0.8725 |
| 7 | 60<q<90 | -0.6552 | -0.5928 | -0.0892 | -1.16 | -0.7773 | -0.7967 |

Table 2.1.3-4: Difference of Test Beams Set 4 from Reference Set

Observation 4: For Test Beams Set 4, the highest over-estimation is 3.5 dB, and the lowest under-estimation is 1.2 dB.

| Elevation Bin | Elevation angular range | Delta (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 9 Beams Cross |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | -0.3482 | -0.0329 | 0.7519 | -0.0409 | 0.7764 | 1.2068 |
| 2 | 5<q<10 | -0.8269 | -0.5177 | -0.214 | -0.4516 | -0.1709 | -0.1547 |
| 3 | 10<q<15 | -0.1282 | -0.3236 | -1.3632 | -0.3402 | -1.622 | -0.0768 |
| 4 | 15<q<20 | 0.4243 | -0.2385 | -0.5578 | -0.3774 | 0.5643 | -0.0743 |
| 5 | 20<q<30 | -0.4467 | -0.083 | -0.9157 | -1.1594 | 1.6961 | -1.4633 |
| 6 | 30<q<60 | 0.2311 | 0.2477 | -0.1359 | 0.4965 | 0.2255 | 0.3896 |
| 7 | 60<q<90 | 2.4748 | 2.5679 | 3.827 | 2.695 | 1.254 | 3.1414 |

Table 2.1.3-5: Difference of Test Beams Set 5 from Reference Set

Observation 5: For Test Beams Set 5, the highest over-estimation is 3.8 dB, and the lowest under-estimation is 1.6 dB.

| Elevation Bin | Elevation angular range | Delta (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 9 Beams Outer Box |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 3.2888 | 4.0314 | 4.7719 | 3.9189 | 4.6313 | 5.5009 |
| 2 | 5<q<10 | -0.2424 | 0.9396 | 3.079 | 0.7969 | 3.1072 | 1.04 |
| 3 | 10<q<15 | 0.9011 | 0.3785 | 2.4225 | 0.53 | 1.8733 | 0.7291 |
| 4 | 15<q<20 | 1.1637 | 0.0849 | -4.7013 | 1.0487 | -2.648 | 1.3044 |
| 5 | 20<q<30 | -0.9829 | -0.3273 | 1.7515 | 1.6968 | -0.8069 | 2.1979 |
| 6 | 30<q<60 | -1.3338 | -1.4132 | -0.5435 | -3.2474 | -2.1632 | -3.9098 |
| 7 | 60<q<90 | 0.0874 | 0.3925 | 3.0333 | 0.5617 | -7.1736 | 1.3674 |

Table 2.1.3-6: Difference of Test Beams Set 6 from Reference Set

Observation 6: For Test Beams Set 6, the highest over-estimation is 5.5 dB, and the lowest under-estimation is 7.2 dB.

Observation 7: Different Test Beams set (which is a subset of the Reference Set) will result in different accuracies to the Reference Set (for different elevation bins). However, based on the different antenna configurations, more Test Beams does not necessarily lead to better accuracy.

Proposal 1: RAN4 to consider a suitable Test Beams set pattern to fulfil the requirement.

Observation 8: The difference resulting from the Test Beams set and the Reference Set can be quantified as over-estimation or under-estimation and the magnitude of these are different. However, from the regulatory perspective, under-estimation is more concerning than over-estimation of the measured EEIRP.

  - Measurement grid

We already presented analysis on angular step size in our contribution [6].

Observation 9: The ability to define separate elevation and azimuthal step size for the different elevation bin could reduce the number of measurement points whilst not impacting the averaging accuracy.

Furthermore, it is agreed elevation step size to be 1deg. Azimuth step size not yet fully agreed.

Proposal 2: RAN4 to consider defining up to 5deg step size or reference angular step size for azimuth. Δϕref= 180∘πλDcyl

  - RF channels

Number of frequencies for OTA spatial emission conformance testing should be performed with appropriate frequencies selected. Many tests in BS conformance specifications are performed with appropriate frequencies in the bottom, middle and top channels of the supported frequency range of the BS. These are denoted as RF channels B (bottom), M (middle) and T (top). During last RAN4 meeting companies had similar views that number of RF channels for conformance testing can be lower. Following 3 options were proposed:

Proposal 1: B, T

Proposal 2: M

Proposal 3: T

Observation 10: Number of frequencies to measure in operating band will impact test effort.

Due to nature of OTA spatial emission requirement as a protection for co-existing with FSS UL services it seems reasonable to set upper frequency range extreme i.e. middle (M) RF channel. That should assure respective test coverage taking into account reasonable test effort.

Proposal 3: It is proposed to use middle (M) channel as frequency to measure when defining conformance testing for Expected EIRP.

  - Measurement uncertainty and CI

Resolution 220 of the ITU Radio Regulation [2] contains the EEIRP technical condition for the protection of the FSS (Earth-to-space) and the Annex accompanying this Resolution outlines the theoretical calculation of this EEIRP. This Annex describes that the EEIRP is calculated in two processes where the beamforming directions are averaged in the first process while the second process describes the averaging of the required horizontal and vertical angles. The averaging of these two processes needs to be accurate and this can be done in different ways where one example suggested in the Resolution is by means of using confidence interval of 95%. This does not exclude other metrics to evaluate the accuracy of the EEIRP calculation that is done in step 1 and step 2.

For conformance testing, it could be more practical to define the accuracy by means that has already be done such as measurement uncertainty or even evaluating the over-estimation or under-estimation of the EEIRP calculation due to sampling of the EIRP when the measurements are done and/or using a representative Test Beams set. As described, the Test Beams set is a subset of all the beamforming directions within the steering range of the base station and this Test Beams set can be considered as long as the overall averaging of the EEIRP is sufficiently accurate.

From the regulation perspective, under-estimation of the EEIRP compared to the required limits is more concerning than over-estimation. Therefore, evaluating the accuracy of the EEIRP calculation can be based solely on the under-estimation of the EEIRP calculation.

Proposal 4: RAN4 to consider defining the accuracy of the EEIRP calculation required by the regulation based only on the under-estimation of the calculation.

- Conclusion

In this contribution we continue discussions for conformance testing for expected EIPR mask requirement for the upper 6 GHz.

We provide also text proposal (TP) with for clause 6 [4] to TR 38.908 (Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz). In addition, we provide draft CR that includes test procedure for BS conformance specification TS 38.141-2 in contribution [5] and draft CR for Annex N [7].

We have made following observations and proposals:

Observation 1: For Test Beams Set 1, the highest over-estimation is 1.1 dB, and the lowest under-estimation is 5.1 dB.

Observation 2: For Test Beams Set 2, the highest over-estimation is 3.2 dB, and the lowest under-estimation is 0.9 dB.

Observation 3: For Test Beams Set 3, the highest over-estimation is 0.8 dB, and the lowest under-estimation is 0.8 dB.

Observation 4: For Test Beams Set 4, the highest over-estimation is 3.5 dB, and the lowest under-estimation is 1.2 dB.

Observation 5: For Test Beams Set 5, the highest over-estimation is 3.8 dB, and the lowest under-estimation is 1.6 dB.

Observation 6: For Test Beams Set 6, the highest over-estimation is 5.5 dB, and the lowest under-estimation is 7.2 dB.

Observation 7: Different Test Beams set (which is a subset of the Reference Set) will result in different accuracies to the Reference Set (for different elevation bins). However, based on the different antenna configurations, more Test Beams does not necessarily lead to better accuracy.

Proposal 1: RAN4 to consider a suitable Test Beams set pattern to fulfil the requirement.

Observation 8: The difference resulting from the Test Beams set and the Reference Set can be quantified as over-estimation or under-estimation and the magnitude of these are different. However, from the regulatory perspective, under-estimation is more concerning than over-estimation of the measured EEIRP.

Observation 9: The ability to define separate elevation and azimuthal step size for the different elevation bin could reduce the number of measurement points whilst not impacting the averaging accuracy.

Proposal 2: RAN4 to consider defining up to 5deg step size or reference angular step size for azimuth. Δϕref= 180∘πλDcyl

Observation 10: Number of frequencies to measure in operating band will impact test effort.

Proposal 3: It is proposed to use middle (M) channel as frequency to measure when defining conformance testing for Expected EIRP.

Proposal 4: RAN4 to consider defining the accuracy of the EEIRP calculation required by the regulation based only on the under-estimation of the calculation.

- 4. References

- RP-241284, Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, ZTE Corporation

- ITU WRC-23, Final Acts, Resolution 220

- R4-2413509, WF for [112][305] NR_BS_RF, ZTE Corporation

- R4-2419473, TP to TR 38.908 clause 6 – Conformance testing, Nokia

- R4-2419470, Draft CR to TS 38.141-2 with EEIRP mask testing procedure, Nokia

- R4-2412899, Expected EIRP mask testing, Nokia

- R4-2419474, Draft CR to TS 38.141-2 with Annex N Expected EIRP calculation, Nokia