3GPP TSG-RAN WG4 Meeting #113															R4-2419472

Orlando, US, November 18 – 22, 2024

Source:	Nokia

Title:	TP to TR 38.908 with updates to core part of RF requirement for OTA spatial emission

Agenda item:	7.11.2

Document for:	Approval

# Introduction

This is text proposal to Technical Report TR 38.908 v0.1.0. with updates to core part of RF requirement for OTA spatial emission to align with core technical specification TS 38.104.

# TP to TR 38.908

<Start of TP>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU World Radiocommunication Conference 2023 (WRC-23) Final Acts, Resolution 220

<Next part of TP>

# 5	RF requirement

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions. This requirement shall be applied to BS operating in band n104 to protect FSS (Earth-to-space) satellite receiver.

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz, shall not exceed the values specified in table 5-1.

The expected EIRP (EEIRP) is defined as the average value of the EIRP, with the averaging being performed:

‒ over azimuth angles from −180º to +180º with reference to BS horizontal boresight and over the specified range of angles θHLi ≤ θ < θHHi , above the horizon as defined in Table 5-1, and

‒ over the supported beam directions.

Note: The requirement is defined based on [2].

Note: The beamforming directions over time means directions are measured sequentially in the test.

Table 5-1: EEIRP limits as function of elevation above horizon

| Bin number i | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0<<5 | 27 |
| 2 | 5<<10 | 23 |
| 3 | 10<<15 | 19 |
| 4 | 15<<20 | 18 |
| 5 | 20<<30 | 16 |
| 6 | 30<<60 | 15 |
| 7 | 60<<90 | 15 |
| NOTE: The requirement shall apply to all supported mechanical tilts. |  |  |

The definitions of elevation angle θHLi and θHHi are illustrated in Figure 5-1.

Figure 5-1: Definitions of θHLi and θHHi angles.