3GPP TSG-RAN WG4 Meeting # 113						R4-2419475

Orlando, US,18th–22nd November 2024

Source:		Ericsson

Title:			TP for A-IoT BS RF requirement

Agenda item:		7.21.3.1

Document for:		Approval

# Introduction

In this paper, we present our overview for A-IoT BS RF requirements.

# Discussion

Current the coexisting scope includes the scenarios and cases which describes the different factors to be considered for the A-IoT BS. These factors are listed in Table 1 below. As RAN4 agree that there is no frequency shifting for D2R transmission from device side, the DL spectrum will be used for D2R when CW transmission occurs in DL spectrum. The A-IoT BS can transmit CW transmission  and this may depend on manufacture declaration..

A-IoT BS can transmit the CW signal and this can be up to the manufacture declaration.

Table 1: Factors impacting the A-IoT BS operation

|  | Spectrum for R2D | Spectrum for D2R | Spectrum for CW transmission | CW transmission power at antenna | A-IoT BS transmit the CW signal | CW power at antenna of A-IoT BS | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A-IoT BS | DL spectrum | DL spectrum | DL spectrum | 33 dBm | Yes | 33 dBm |  |
|  | DL spectrum | DL spectrum | DL spectrum | 33 dBm | No | >3 dBm and <33 dBm | Antenna isolation between 10 and 30 dB |
|  | DL spectrum | UL spectrum | UL spectrum | 23 dBm | Yes | 23 dBm |  |
|  | DL spectrum | UL spectrum | UL spectrum | 23 dBm | No | >- 7dBm and <13 dBm | Antenna isolation between 10 and 30 dB |

From the Table 1, it can be observed that for different spectrum usage for A-IoT operation. As CW transmission may interfere the R2D reception, the R2D and D2R should be TDM. If a FDD band would be defined, it should be half duplex mode. As it is not clear how the spectrum will be used and it may depend on coexisting study, we recommend prioritizing the FDD band in half duplex mode in study phase.

For FDD band definition the A-IoT operation band, the half duplex (HF-FDD) mode may be relevant to band definition.

RAN4 prioritize the FDD band in half duplex mode in study phase.

For A-IoT BS declared to transmit the CW signal, CW signal needs to be transmitted according to RAN1and RAN  agreements below:

RAN1#116:

Agreement For R19 A-IoT study item, at least single-tone unmodulated sinusoid waveform is a candidate waveform for carrier wave for D2R backscattering. Agreement For R19 A-IoT study item, multi-tone waveforms for carrier wave for D2R backscattering can be studied.

RAN1#116bis:

Agreement For CW waveform for D2R backscattering, multiple unmodulated single-tone is studied compared to single-tone in R19 SI. - Two unmodulated single-tones as a starting point - FFS: Other number of tones - FFS: how large gap is needed between tones Agreement For CW waveform for D2R backscattering, contiguous multi-tone OFDM signal is not studied in R19 SI. Agreement Study at least the following characteristics of unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering: - For D2R - Reception performance - Spectrum utilization of backscattered signal corresponding to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and CW cancellation capability value/range (if any) - For scenarios ’A1’, ’A2’ and ’B’ - Relative complexity of CW generation

RAN#103:

Proposal 3v2 (endorsed) - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc.

From the RAN1 and RAN agreements, it can be observed that the CW waveform is defined with unmodulated sinusoid waveform, and maximum number of CW signal is limited to two. As the CW transmission is controlled in terms of the timing, power, bandwidth, and spectrum, CW signal is relevant and RAN4 can continue to specify the RF characteristic of CW signal based on these basic assumptions.

CW is unmodulated sinusoid signal and can be one or two tones with FFS frequency gap.

CW transmission’s timing, power, bandwidth, spectrum is controlled.

CW RF characteristic can be studied in RAN4 according to the RAN1 and RAN agreements.

From previous discussion, the FDD band in half duplex mode should be prioritized because otherwise the simultaneous CW and R2D transmission may interfere the R2D signal reception at device 1/2a. Therefore, the R2D and CW transmission is TDM and CW signal transmission requirement should be discussed separately with R2D transmission.

CW and R2D transmission should be TDM to prevent the CW interfering with the R2D reception at device.

CW and R2D transmission requirement can be discussed separately.

For A-IoT R2D transmitter requirement, as the R2D signal design very similar with LP-WUS signal design, for example, he OOK1 and OOK4 design are following the LP-WUS signal design. therefore, the transmitter requirement can depend on the LP-WUS work item conclusion and there is no need to discuss it separately in A-IoT study.

No need to discuss the A-IoT R2D transmitter requirement but depending on the LP-WUS work item conclusion.

Then only the CW transmission requirements are relevant in study phase: the CW signal is new compared to legacy NR BS. Therefore, RF requirement relating to the CW signal should be discussed for transmitter requirement. For a dedicated CW node, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission are relevant requirement. As there is modulated signal interferer in coexisting study. Also the inhibition of the reverse IM product from co-located/nearby CW transmitter should be applied as multiple tone of CW is allowed in RAN1.

### Issue 2-6: CW for D1T1

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

For separated CW node, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission and the intermodulation requirement should be applied.

As the CW transmission power may be defined with certain bandwidth, this may not be the same as the A-IoT R2D signal transmission power. For A-IoT BS declared to transmit the CW signal, if following the NR BS power dynamic range, the output power may be scaled with the transmission bandwidth, therefore, if CW transmission bandwidth is one tone (e.g 15kHz), the CW transmission power would be maximum carrier power – 10.7 dB (e. g 10log (12/1)). So the CW transmission power would be lower if CW signal occupied bandwidth would be smaller than A-IoT R2D signal bandwidth. However, this is not expected from coexisting simulation. Our understanding is that CW signal power should be 33 dBm or 23 dBm. It can be done with current BS specification framework if the CW transmission bandwidth would be different with A-IoT R2D bandwidth or transmission bandwidth would be defined differently and then two different carrier power would be possible to be configured and tested. To study the needed bandwidth, a simulation with CW signal with phase noise profile in TR 38.803 is illustrated in Figure 3 and it can be observed that CW OCC is around 100Hz. If channel bandwidth would be defined around 100Hz, it will be possible to transmit high output power. Alternatively, the transmission bandwidth could be defined differently for CW transmission but the channel bandwidth is the same with A-IoT BS.  As CW and A-IoT R2D signal is TDM operation, transmitting CW and A-IoT R2D with different power with two different channel bandwidth is possible. However, there may be some re-configuration time needed and there may be some transient time between R2D and CW signal transmission. For the device utilizing the CW signal to harvest the energy, the transient time may not be too long otherwise the passive device may loose the power.  RAN4 could discuss whether to define a new timing mask for this case for output power dynamic requirement.

Figure 2: Occupied BW for a CW signal

Transient time between R2D and CW may impact the passive device power hold-up time.

Another issue is unwanted emission generated by the phase noise and the transmitter non-linearity, such unwanted emission level may appear higher than the backscatter signal at the antenna. This is illustrated in Figure 3. The phase noise profile in TR 38.803 is used and it can be observed that around -50 dBm at the frequency offset of the 300kHz. If the reader REFSENS is around -70 dBm, such level of emission will drown the backscatter signal. If reader could equip with CW cancellation with a local copy, the unwanted emission caused by phase noise may be cancelled also. To enable the CW cancellation function, reader needs some time to train the circuit to generate the reverse CW leakage with the amplitude and negative phase. Considering this time and also the transient time to transmit the CW signal, the timing relation between the R2D, CW and D2R is illustrated in Figure 4.  The transient time can be specified and tested but the CW cancellation training time must be accommodated with protocol design.

For a separated CW node, it may be difficult to cancel the phase noise from another CW node. As such, RAN4 should continue study the unwanted emission mask for CW signal. Our view on CW node RF requirement is listed in Table 2. As the phase noise can be indirectly specified by unwanted emission level, there is no need to specify it directly. Unwanted emission can also include the transmitter nonlinearity impact.

Table 2: CW node RF requirement

| RF Requirement for CW for D1T1 | Requirements potentially needed or not |
| --- | --- |
| Output power | YES |
| ON/OFF power | YES |
| Frequency error | YES |
| Phase noise | NO |
| Unwanted emission | YES |
| Spurious emissions | YES |
| Intermodulation | YES |

Different channel bandwidth may be defined for CW and A-IoT R2D transmission for BS can transmit CW signal or different transmission bandwidth defined for the same channel bandwidth.

A new timing mask should be specified to test R2D and CW transmission for outpower dynamic and shared architecture.

Unwanted emission level from phase noise for CW transmission may be higher than backscatter signal.

CW cancellation may be needed to combat the phase noise.

Unwanted emission mask should be considered for CW transmission.

Figure 3: the CW transmission with phase noise

Figure 4: Transient time for CW and R2D transmission and CW cancellation training time before D2R reception

For receiver requirements, no matter how to define the operating bands, from RF perspective, the A-IoT BS needs to tolerate CW signal while receiving the D2R signal. For receiver requirement, as the coexisting scenario have both inside and outside topology, it should discuss whether two different set of the RF requirements should be defined corresponding to the either inside topology or outside topology. For inside topology, the receiver needs to tolerate the high leakage CW power from either the transmitter if BS transmits CW signal or co-located CW transmission node. For outside topology, CW transmission is at least the distance of the A-IoT BS for D1T1-B, so the received CW power would be less issue for receiver to decode the backscattered signal and linearity requirement on receiver is relaxed. In our companion discussion paper for CW cancellation, the additional CW cancellation circuit would be expected if the linearity of the receiver would be compromised. Such would not be needed for outside topology where the leakage power from CW transmission is in par or smaller than backscattered signal. These considerations motivate different receiver architecture depending whether CW inside topology or outside topology is deployed.

Define two sets of the A-IoT BS receiver requirement corresponding to inside topology and outside topology respectively.

The benefit of the defining two sets of receiver requirement can be the reader range. For the REFSENS designed for inside topology, the SNR degradation would be allowed as the high leakage power desensitize the receiver. For the REFSENS designed for outside topology, the need of further relaxing the REFSENS may not be needed therefore larger Reader to device range could be achieved. There are more factors to consider for REFSENS discussion, for example, the energy harvesting sensitivity of device 1 may limit the R2D distance and the min received power may be decided by R2D distance. Such discussion should be in the context of the link budget. Another example, the coexisting may set the limit on the cell range and therefore, the reader to device distance further be limited. For these factors to impact the REFSENS definition, RAN4 can identity these factors in study phase and further address it in work item phase to define the REFSENS.

Two different sets of receiver requirement may target to different reader to device range.

Both link budget and coexisting impact the REFSENS definition.

REFSENS definition depend on link budget analysis and coexisting scenario conclusion.

For the receiver requirements defined for inside topology, the tolerance of the maximum CW power at antenna can be declared by the manufacture. The REFSENS should be tested with a CW signal coupled to the antenna via OTA if A-IoT BS cannot transit the CW signal, how to test it can be further discussed in work item phase. If the A-IoT BS can transit the CW signal, the REFSENS can be tested with CW transmitting at the same time.

REFSENS for inside topology should be tested with the CW signal either transmitted in BS or coupled with OTA.

- For the receiver requirements defined for outside topology, it can be discussed whether there is no need to test it with a CW signal. In current D1T1-B coexisting simulation, the CW is two-cell radii away and PSD difference between backscattered power and CW signal may depend on the backscatter loss, e.g 6 dB for OOK in RAN1 assumption. The close-in noise from CW signal should not distort the backscattered signal and this should be the same with inside topology. Further investigate the PSD difference between CW signal and D2R signal for outside topology.

Further discuss if there is a need to test the REFSENS with CW signal or not.

- For ACS and inband block requirements, it depends on the coexisting study. For the ICS (in-channel selectivity), currently only single tone and single A-IoT UE D2R reception is agreed in RAN1 and FDMA for multiple UE is under discussion. RAN4 can further wait RAN1 progress to discuss this.

Wait further Ran1 progress to discuss the ICS requirements.

TP is started here:

### 6.7.2	Ambient IoT BS

General

Current the coexisting scope includes the scenarios and cases which describes the different factors to be considered for the A-IoT BS. These factors are listed in Table 6.7.2-1 below. CW signal can be transmitted by A-IoT BS if manufacture declare this capability. CW can be transmitted by a separate node, therefore CW related RF requirements can be specified separately for A-IoT BS or a separate CW node.

Table 6.7.2-1: Factors impacting the A-IoT BS operation

|  | Spectrum for R2D | Spectrum for D2R | Spectrum for CW transmission | CW transmission power at antenna | A-IoT BS transmitting the CW signal |
| --- | --- | --- | --- | --- | --- |
| A-IoT BS | DL spectrum | DL spectrum | DL spectrum | 33 dBm | Yes |
|  | DL spectrum | DL spectrum | DL spectrum | 33 dBm | No |
|  | DL spectrum | UL spectrum | UL spectrum | 23 dBm | Yes |
|  | DL spectrum | UL spectrum | UL spectrum | 23 dBm | No |

Simultaneously CW and R2D transmission will interfere the R2D reception, as such the R2D and D2R transmission should be operated in TDM.

When A-IoT transmits the R2D signal, as the R2D signal design very similar with LP-WUS signal design, for example, he OOK1 and OOK4 design are following the LP-WUS signal design. therefore, the transmitter requirement can refer to the LP-WUS BS RF requirements.

Output power dynamics

For A-IoT Base station that is declared to have capability to transmit CW signal, there is a need to specify the transient time between R2D and CW transmission due to the different channel bandwidth could be configured for R2D and CW transmission illustrated in Figure 6.7.2-1.

Figure 6.7.2-1: Transient time for CW and R2D transmission

Reference sensitivity level

For inside topology, the receiver needs to tolerate the high leakage CW power from the transmitter when intermediate node can transmit CW signal and desensitization of receiver may be allowed with a CW cancellation function enabled in such case. For outside topology, the received CW power would be less and in this case, receiver to decode the backscattered signal without desensitization of receiver caused by CW leakage. Therefore, for REFSENS requirements, two sets of requirements can be defined, corresponding to inside topology and outside topology respectively.

CW transmission in separate node

When a CW is transmitted in a separated node, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission and transmitter intermodulation can be specified.

# Conclusions

In this contribution, we present our overview on the A-IoT BS RF with below proposal and observations:

Observation 1 For FDD band definition the A-IoT operation band, the half duplex (HF-FDD) mode may be relevant to band definition.

Proposal-2: RAN4 prioritize the FDD band in half duplex mode in study phase.

Observation 2 CW is unmodulated sinusoid signal and can be one or two tones with FFS frequency gap.

Observation 3 CW transmission’s timing, power, bandwidth, spectrum is controlled.

Observation 4 CW RF characteristic can be studied in RAN4 according to the RAN1 and RAN agreements.

Observation 5 CW and R2D transmission should be TDM to prevent the CW interfering with the R2D reception at device.

Observation 6 CW and R2D transmission requirement can be discussed separately.

Proposal-3: No need to discuss the A-IoT R2D transmitter requirement but depending on the LP-WUS work item conclusion.

Proposal-4: For separated CW node, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission and the intermodulation requirement should be applied.

Observation 8 Different channel bandwidth may be defined for CW and A-IoT R2D transmission

Proposal-5: A new timing mask should be specified to test R2D and CW transmission for outpower dynamic and shared architecture.

Observation 9 Unwanted emission level from phase noise for CW transmission may be higher than backscatter signal.

Proposal-6: Unwanted emission mask should be considered for CW transmission.

Proposal-7: Define two sets of the A-IoT BS receiver requirement corresponding to inside topology and outside topology respectively.

Observation 11 Two different sets of receiver requirement may target to different reader to device range.

Observation 12 Both link budget and coexisting impact the REFSENS definition.

Proposal-8: REFSENS definition depend on link budget analysis and coexisting scenario conclusion.

Proposal-9: REFSENS for inside topology should be tested with the CW signal either transmitted in BS or coupled with OTA.

Proposal-10: Further discuss if there is a need to test the REFSENS with CW signal or not.

Observation 13 Wait further Ran1 progress to discuss the ICS requirements

# References

- R4-2410597, WF on impacts of A-IoT on RF requirements, Huawei

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR