3GPP TSG-RAN WG4 Meeting # 113						R4-2419476

Orlando, US,18th–22nd November 2024

Source:		Ericsson

Title:			Coexisting study simulation assumptions and result for A-IoT

Agenda item:		7.21.2.2

Document for:		Approval

# Introduction

In this paper, we discuss additional simulation assumptions and present simulation results for some of cases of coexisting for A-IoT.

# Discussion

## Simulation assumptions

In WF [1], the interference modelling is stated below:

Issue 4-1-1:  A-IOT reader and NR BS

Agreement:

|  | In-band |  |
| --- | --- | --- |
|  | Tx | Rx |
| NR UE/A-IOT Intermediate UE | For NR UE: - Baseline: Legacy UE IBE For intermediate UE: - Baseline: legacy UE IBE - Optional: NB-IOT ACLR | In-band A-IOT signal interference to NR is not considered when calculating SINR. Companies to check the interference caused by non-orthogonal A-IOT signal |
| NR BS | Option 1: 30dBc Option 2: 17dBc Option 3: 12dBc with 1 blank RB | In-band A-IOT signal interference to NR is not considered when calculating SINR. Companies to check the interference caused by non-orthogonal A-IOT signal |
| A-IOT BS | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dBc，ACLR2:50dBc) | ACS of legacy gNB |

Figure 1: interference from NR BS to A-IoT BS receiver

For the case where the NR BS is aggressor in DL, the interference from NR BS on the A-IoT BS or A-IoT device mainly is decided by the iFFT sidelobe level. Figure 1 illustrates the case where NR BS DL interfere with A-IoT Reader.  The interference level can be illustrated by the simulation of OFDM modulation where one RB is blanked. Figure 2 shows that the interference to the blanked RB can be around 17 dB in some subcarrier relative to the average NR transmission power.   Last meeting, one company proposed the average value in the blanked RB and derive a 12 dB and we agree this should be used.

The inband leakage from iFFT modulation should be 12 dB considering averaging all leakage within the blanked RB.

Figure 2: IBE for NR DL with one RB blanking

Issue 4-1-2: CW unwanted emissions

Agreement in RAN4#112: Discuss whether and how to model the CW unwanted emission.

Agreement:

The impact of CW node to adjacent NR carrier can be analysed and captured in TR.Note: no impact to co-existence simulation

Using the TR 38.803 phase noise profile, the CW transmission and the unwanted emission by phase noise is simulated and illustrated in Figure 3. The ACLR1 is around 40 dBc and ACLR2 is around 50 dBc.  Adding the PA non-linearity margin of 3 dB, we think the unwanted emission if modelled as the ACLR1 and ACLR2 should be 37 dBc and 47 dBc.

CW unwanted emission can be modeled with 37 dBc for ACLR1 and 47 dBc for ACLR2.

Figure 3: the CW transmission with phase noise

## Observations from initial simulation results

### R2D SINR estimation

Using the approached proposed in our companion paper for simulation methodology. the SINR CDF curve is simulated with the 10MHz noise bandwidth in WF [1]. It can be observed that 10MHz noise bandwidth is too pessimistic for device 1/2a and using the approach in our companion paper, a reasonable inter-system interference can be observed. Using the SNR of 17.8 dB as decoding threshold, one can observe that the device of outage increase from 20% to 27%, which is 7% increasing of outage device which are below SINR decoding threshold. The outage increase will be 55% (from 20% to 75%) if using the 10MHz noise bandwidth. Such big difference should motivate RAN4 to consider a reasonable way to factor in the NR interference noise.

7% outage increase from baseline to enabling inter-system interference using the LLS simulation result to factor in the NR signal interference.

55% outage increase from baseline to enabling inter-system interference using 10MHz noise bandwidth

Figure 4: The CDF curve for inter-system SINR comparing between 10 MHz noise bandwidth and new approach 180kHz noise bandwith

### Updates with conclusion for simulation results.

Our conclusion from the simulation results are listed in Table 3 below.

We have observed a large impact from NR (BS and UE) on the reader. This is due to a large leakage from adjacent RB transmitter from the NR system. There is a need to tighten the leakage from BS and simplest way to do it is to use guard RB between the RB used for A-IoT system and NR signal. The exact impact needs to be further evaluated based on modified assumptions.

The SNR degradation of reader as victim is more than 1 dB from the interference of NR system

The leakage from NR base station should be lowered by adding more guard RB, how to model this FFS.

Figure 5: CDF of SINR for NR DL (agressor) to reader (victim)

Table 3: Coexisting conclusions

| deployment scenario and topology |  | spectrum | Evaluation case No. | (Aggressor -> Victim) | Tentative Co-existence conclusion |
| --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |  |
|  |  |  | b | NR DL->reader | SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions. |  |
|  |  |  | c | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |  |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage |  |
| 1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |  |
|  |  |  | b | NR DL->reader | SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions. |  |
|  |  |  | c | Reader->NR DL | FFS: NR DL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range FFS on the UE blocking issue. Interference coordination/mitigation solution can be considered. |  |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |  |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |  |
|  |  |  | f | NR UL->reader | SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions.. |  |
|  |  |  | c | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed. |  |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |  |
| 2-2 | D1T1-B-legacy UE indoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |  |
|  |  |  | f | NR UL->reader | SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions.. |  |
|  |  |  | c | Reader->NR DL | FFS: NR DL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range level FFS on the UE blocking issue. Interference coordination/mitigation solution can be considered. |  |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |  |
| 3 (O) | D1T1-A2-legacy UE indoor | R2D: UL CW2D and D2R: DL | a | Device->NR DL | SINR degradation is less than 1dB. Inter-system interference is negligible. |  |
|  |  |  | b | NR DL->reader | SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions.. |  |
|  |  |  | g | Reader->NR UL | SINR degradation is less than 1dB. Inter-system interference is negligible. |  |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. |  |
| 4 (O) | D1T1-B-legacy UE indoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | SINR degradation is less than 1dB. Inter-system interference is negligible. |  |
|  |  |  | f | NR UL->reader | SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions.. |  |
|  |  |  | g | Reader->NR UL | SINR degradation is less than 1dB. Inter-system interference is negligible. |  |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. |  |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |  |
|  |  |  | f | NR UL->reader | SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions. TBD on the outage percentage. |  |
|  |  |  | g | Reader->NR UL | SINR degradation is less than 1dB. Inter-system interference is negligible. |  |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. TBD on the outage percentage. |  |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed |  |
|  |  |  | f | NR UL->reader | SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions.. |  |
|  |  |  | g | Reader->NR UL | FFS: NR UL throughput degradation for both average throughput and cell edge throughput is observed. Note: TBD on the throughput degradation range Interference coordination/mitigation solution can be considered. |  |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. |  |
| 6-1(O) | D2T2-B-legacy UE only outdoor | R2D: UL CW2D and D2R: DL | a | Device->NR DL | SINR degradation is less than 1dB. Inter-system interference is negligible. |  |
|  |  |  | b | NR DL->reader | SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions.. |  |
|  |  |  | g | Reader->NR UL | SINR degradation is less than 1dB. Inter-system interference is negligible. |  |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. |  |
| 6-2(O) | D2T2-B-legacy UE indoor | R2D: UL CW2D and D2R: DL | a | Device->NR DL | SINR degradation is less than 1dB. Inter-system interference is negligible. |  |
|  |  |  | b | NR DL->reader | SNR degradation larger than 1dB. Coexistence not feasible based on current assumptions.. |  |
|  |  |  | g | Reader->NR UL | SINR degradation is less than 1dB. Inter-system interference is negligible. |  |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. |  |

# Conclusions

In this contribution, we present our view on the coexisting simulation with below proposal and observation:

Proposal-1: The inband leakage from iFFT modulation should be 12 dB considering averaging all leakage within the blanked RB.

Proposal-2: CW unwanted emission can be modeled with 37 dBc for ACLR1 and 47 dBc for ACLR2.

Observation 1 7% outage increase from baseline to enabling inter-system interference using the LLS simulation result to factor in the NR signal interference.

Observation 2 55% outage increase from baseline to enabling inter-system interference using 10MHz noise bandwidth

Observation 3 The SNR degradation of reader as victim is more than 1 dB from the interference of NR system

Proposal-3: The leakage from NR base station should be lowered by adding more guard RB, how to model this FFS.

# References

- R4-2417196, WF on Co-existence study for ambient IoT, CMCC

# Annex