3GPP TSG-RAN WG4 Meeting # 113						R4-241xyz

Orlando, US,18th–22nd November 2024

Source:		Ericsson

Title:			TP to TR 38.769:  A-IoT general overview

Agenda item:		7.21.1

Document for:		Approval

# Introduction

In this paper, we present our thoughts on general issues for A-IoT SI.

# Discussion

## system parameter

In WF[1], the remaining issue is listed below for system parameter.

### Issue 1-1: System parameter

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| System parameter for A-IoT |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | A-IoT BS | A-IoT divice | Intermediate UE |
| Operating band |  | YES |  |  |
| Channel bandwidth | Transmission bandwidth | YES the value needs be discussed in WI phase depending on other WG conclusion. |  |  |
|  | channel bandwidth | YES，as RAN4#112 agreed |  |  |
| Channel arrangement | Channel spacing | TBD for standalone NA for in-band/Guard-band |  |  |
|  | Channel raster | YES | TBD | TBD |
|  | Synchronization raster | NO for device 1/2a，as RAN4#112 agreed |  |  |

For channel spacing, this is the arrangement of multiple channels and in current coexisting scope. Although current A-IoT channel is configured only one, it should not preclude the possibility to be configured with multiple channel in the future depending additional coexisting scope. On top of this, RAN1 also discuss the FDM of the uplink reception. It can be observed that FDM for D2R is under discussion but FDM for R2D is agreed to be not supported for passive device.

FDM for D2R is under discussion but FDM for R2D is agreed to be not supported for passive device.

RAN1#116bis:

Agreement Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Agreement Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS. Agreement Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS.

RAN1#118bis:

Agreement Capture in the TR that for R2D: - Time-domain multiplexing is the baseline. - Code-domain multiplexing is not considered for device 1/2a/2b. - Frequency-domain multiplexing is not considered for the devices with RF-ED receiver, and studied for Device 2b with IF-ED or ZIF receiver considering the technical aspects in Table 6.1.1.x-1.

TP starts here:

### 6.7.1	System parameters

#### 6.7.1.1	Operating band

According to the SID (RP-240826), Ambient IoT uses FR1 licensed spectrum in FDD. NR band n8 can be used as an example band.

[Table 6.7.1.1-1 summarized the A-IoT system operation cases according to coexisting scenario.] It can be observed that The spectrum usage for A-IoT operation can be flexible, and it depends on several factors ,such as whether the reader is A-IoT BS  or intermediate UE, and whether CW transmission is deployed inside or outside topology. For the case where different spectrum is used for D2R and R2D, the FDD band definition can be kept. As CW transmission may interfere the R2D reception, the R2D and D2R should be TDM and therefore, for FDD band definition, it should be half duplex mode.

[Table 6.7.1.1-1: Factors impacting the A-IoT BS and UE as reader operation]

|  | Spectrum for R2D | Spectrum for D2R | Spectrum for CW transmission | Comment |
| --- | --- | --- | --- | --- |
| A-IoT BS | DL spectrum | DL spectrum | DL spectrum | [Coexisting scenario D1T1-A2] |
|  | DL spectrum | UL spectrum | UL spectrum | [Coexisting scenario D1T1-B] |
|  | UL spectrum | DL spectrum | DL spectrum | [] |
| UE as reader | UL spectrum | UL spectrum | UL spectrum | [Coexisting scenario D2T2-A2] |
|  | UL spectrum | DL spectrum | DL spectrum | [Coexisting scenario D2T2-B] |

#### 6.7.1.2	Channel bandwidth

The R2D waveform is agreed to be OFDM-based waveform, therefore the channel bandwidth definition can follow the channel bandwidth definition in NR.

#### 6.7.1.3	Channel spacing

The channel spacing would be specified for D2R FDM operation for device 1/2a.

# Conclusions

In this contribution, we present our overview on the A-IoT general issues with below observation:

Observation 1 FDM for D2R is under discussion but FDM for R2D is agreed to be not supported for passive device.

# References

- R4-2417088, WF on impacts of A-IoT on RF requirements, Huawei