# 3GPP TSG-RAN WG4 Meeting # 113						R4-2419484

# Orlando, US,18th–22nd November 2024

Source:		Ericsson

Title:			TP for TR 38.774: System parameter for WUR

Agenda item:		7.23.2.1

Document for:		Approval

# Introduction

In this paper, we present our overview on the system parameter for WUR RF requirement perspective.

# Discussion

In WF[1], the channel raster for WUR is not decided.

Issue 1-2-1: Channel raster for LP-WUR

Agreements:

  - Companies directly bring the specification impact if any and those will be discussed instead of whether a channel raster is required or not.

Issue 1-2-2: considerations on LP-WUS allocation

Agreements:

  - RAN4 to assume PRB grid alignment between LP-WUS and NR.

  - RAN4 to assume the LP-WUS RBs can be flexibly allocated within the wider NR carrier.

In RAN2#126 agreement, the LP-WUS will be configured in system information for RRC idle and inactive and this means UE can locate the LP-WUS when the SIB is obtained.

### 8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Entry/exit condition

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions.

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

-	LP-SS configuration

-	LP-WUS configuration

-	Entry/exit condition for LP-WUS monitoring (FFS if it is always configured)

In RAN2#127, below is agreed.

### 8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

LP-WUS configuration

(Whether dedicated configration is needed or not)

Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE

In RAN2#127bis, the below is agreed:

### 8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Entry/exit condition

If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

The LPWUS monitoring exit condition does not include MR measurements.

From the above observation, the WUR will not be configured with a channel rather than an RB location within a system bandwidth. Network needs to assure the LP-WUS signal should be transmitted together with other NR signals using the same iFFT modulator, this also means the LP-WUS signal position should be referenced with the serving cell RB grid. The channel will be configured in MR and LR will be indicated with WUS RB location within the cell RB grid. From this perspective, there is no new channel raster is needed for LR. Some company argue the WUR should configure a LP-WUS signal which a channel raster where the center of LP-WUS is the WUR channel frequency and therefore the additional 10kHz channel for a NR band with 100kHz channel raster will be needed. Our view is that this should be treated as UE implementation specific as there is no network parameter configure the LP-WUS center frequency. For a Zero-IF RF architecture, UE can receive wider bandwidth than LP-WUS and filter can be applied at baseband to filter out the non-WUS if UE knows where the wanted signal located in base band. From testing perspective, if the LP-WUS RB and MR RB grid is not aligned, this cannot be observed from the WUR RF test, especially if UE has good margin and the consequence of the misaligned LP-WUS RB alignment and MR RB only results in limited performance degradation.

RAN2 agrees to configure LP-WUS in SIB.

Treat the RB alignment between MR and LR as implementation specific.

No new channel raster for LR.

TP for system parameter starts here:

# 5	System parameters

## 5.1	General

<Editor’s note: general aspects, details TBD. >

The frequency ranges in which NR WUR can operate are identified as described in Table 5.1-1. Whenever the FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered, unless otherwise stated.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

## 5.2	Operating bands

<Editor’s note: LP-WUS is a generic feature can apply to all bands.  >

LP-WUS is a general feature not limited to specific example band(s).

## 5.3	Channel raster

<Editor’s note: FFS whether new considerations for LP-WUS/SS raster. >

  - Th PRB grid between LR and MR is aligned and the LP-WUS RBs can be flexibly allocated within the wider NR carrier. How to make sure the RB alignment between LP-WUS RB and MR RB grid is aligned is UE implementation specific.

# Conclusions

In this contribution, we present our view on system parameter for WUR with below proposal:

Observation 1 RAN2 agrees to configure LP-WUS in SIB.

Proposal-1: Channel raster does not apply to WUR.

# References

- R4-2417112, WF on LP-WUS UE RF requirements, vivo