3GPP TSG-RAN WG4 Meeting #113									                        R4-2419510

Orlando, US, November 18 – 22, 2024

Agenda item:			7.19.2

Source:	Ericsson

Title:	On UE RF requirements for NR MIMO Phase 5

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for the RAN1-led WI on “NR MIMO Phase 5” WI [1]. In RAN4#112-bis, a WF has been approved [2], where the objectives of the WI with the identified potential RAN4 UE RF impacts are the following:

RAN1:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

Acronyms:

- sTRP: single TRP (transmit-receive point)

- mTRP: multiple TRP (transmit-receive point)

- CJT: coherent joint transmission

- DU: distributed unit

- CRI: CSI-RS resource indicator

RAN2:

Specify higher layer support of the enhancements listed above.

RAN4:

Specify necessary core requirements for the enhancements listed above.

In this contribution, we further discuss the impact of “Support for 3-antenna-port codebook-based transmissions” and “Asymmetric DL sTRP/UL mTRP” working areas on the UE RF requirements in RAN4.

# Discussion

## 3Tx

In the following, the open issues from the approved WF [2] on “3Tx” topic will be discussed.

### Power class

On the support for different power classes, the following has been agreed in the previous meeting:

<Sub-topic 1-1> Power class

<Agreement>: PC for 3Tx requirement

- PC for 3Tx requirement

  - PC3

  - PC2

  - FFS on PC1.5

    - FFS on UE type, i.e., FWA and/or handheld UE

    - FFS on support of full power mode 0

- RAN4 understanding is that TxD for 3Tx is not in the scope of this WI

- Full power mode 0 apply to PC2 and PC3 only.

  - FFS on for which UE types it should apply.

<Way forward>: FFS on New capability for power class per band per declared UL power mode for 3Tx

According to the WID [1], the UL full power transmission modes 1 and 2 are not in the scope, and the only remaining candidate mode is the “Mode-full power” (ULFPTx mode 0). For PC1.5, the support for that mode implies that the UE would have to be equipped with a PA of 29dBm in every Tx branch. However, according to our understanding, there are currently no UEs in the field which are equipped with a PA of 29dBm. PC1.5 could potentially only be specified for a “non-full power” mode, i.e. the Rel-15 behavior.

Proposal 1: PC1.5 could be supported with no full power modes, i.e. Rel-15 behavior. It should be feasible for both FWA and handheld UEs.

For PC3, the support for full power mode 0 implies that the UE would have to be equipped with a PA of 23dBm in every Tx branch, which is feasible for both FWA and handheld UEs.

For PC2, the support for full power mode 0 implies that the UE would have to be equipped with a PA of 26dBm in every Tx branch. This should not be a problem for FWA, while for handheld UEs it would be challenging but possible in theory. We would not like to prevent such an implementation if feasible for some UEs in the field.

Proposal 2: For both PC3 and PC2, the specifications should allow the support for full power mode 0 for both FWA and handheld UEs.

When it comes to “FFS on New capability for power class per band per declared UL power mode for 3Tx” issue in the WF [2], in our understanding it was proposed for the case if both Rel-15 behavior and full power mode 0 were supported for PC1.5. Since in our view full power mode 0 should not be supported for PC1.5, there is no need to introduce such capability.

Proposal 3: There is no need to introduce new capability for power class per band per declared UL power mode for 3Tx.

### MPR

On the MPR requirements topic, the following was agreed in the previous meeting [2]:

<Sub-topic 1-3> MPR

<Agreement>: Different MPR for different UE type

- Define separate MPR requirements for handheld and FWA UE

<Way forward>: MPR derivation method for 3Tx

- Companies are encouraged to evaluate MPR based on existing MPR requirements, e.g., PC, 2/4Tx, UE type, considering 3PA configuration agreed above

  - Option 1: Reuse legacy MPR requirements for each power class (OPPO, Huawei, Nokia)

  - Option 2: Further evaluate with 3PA configuration (Apple)

In current specifications, the same PC3 MPR requirement applies for all of the 1Tx, 2Tx and 4Tx. The same should hold for 3Tx as well.

According to some evaluations submitted in the previous meeting, e.g. [3], for both PC2 and PC1.5 it was concluded that the MPR required is slightly smaller than the current corresponding 2Tx UL MIMO MPR requirement. Thus, it was proposed to reuse corresponding 2Tx MPR requirements for 3Tx as well, which is reasonable in our view.

Proposal 4: For all power classes, reuse the legacy 2Tx MPR requirements for 3Tx MPR requirements.

### Supported layer for mode 0

For “ULFPTx mode 0” issue, the following was agreed in the previous meeting [2]:

<Sub-topic 1-4> Supported layer for mode 0

<Way forward >: FFS on Supported MIMO layer for ULFPTx mode 0

In RAN4#112-bis meeting, an LS was sent to RAN1 [4] with the following question:

Question 3: RAN4 would like to check with RAN1 whether 3Tx UE can support UL full power transmission mode 0 with 2Layer 3-port codebooks but not support 1Layer 3-port codebooks.

As stated in previous clauses, if a 3Tx UE supports Mode 0, it should deliver full power for any TPMI, and so it would require three 29 dBm PAs. That is, the UE would have to support the selection PMIs where one port is active, and so assuming no virtualization, i.e. one connector per port, the PAs would have to be 29 dBm. According to our investigation, a 3Tx UE cannot support UL full power transmission mode 0 with 2Layer 3-port codebooks without supporting 1Layer 3-port codebooks.

‘Full-power mode 0’ 1-layer 3-port codebooks should be feasible for PC3 and PC2, and both DFT-s-OFDM and CP-OFDM waveforms should be supported. Hence, we make the following proposal:

Proposal 5: For ULFPTx mode 0, support 1-layer with 3 Tx ports case for both DFT-s-OFDM and CP-OFDM waveforms, i.e. add 1-layer for 3Tx with W=13100,13010,13001 and add both DFT-s-OFDM and CP-OFDM.

## Asymmetric DL sTRP/UL mTRP

In the approved WF [2], the following was captured on this topic:

<Sub-topic 2-1> UE RF impact

<Agreement>: Whether to assume different TCI state between DL and UL

- Assume different TCI state between DL and UL

<Way forward>: Whether to study FR2 enhancements for asymmetric TRPs scenario in Rel-19

- RAN4 will not study FR2 enhancements until RAN1 considers the scenario in Option 1

  - Option 1: Yes, UL-specific TRPs are assumed to have limited DL capability

  - Option 2: No, need more check

  - Option 3: Check with RAN1 sending an LS

On this topic, the following question has been sent to RAN1 in the approved LS [4]:

Question 1: RAN4 would like to inquire if the UL TRP(s) can transmit SSBs per the scope of the WID.

According to our understanding, such scenario is not precluded from the WID and can be considered in addition to the scenario where the UL TRPs cannot transmit SSBs. From technical perspective, including a scenario where the UL TRP can transmit SSB makes sense, to help the accuracy of the UL power control. Ultimately, whether to consider such scenario will depend on RAN1 response.

Observation 1: In our understanding, a scenario where the UL TRP(s) can transmit SSBs is not precluded by the WID, but the decision on whether to consider such scenario will depend on RAN1 response.

# Conclusion

Based on the discussion in the previous sections we have made the following observation:

Observation 1: In our understanding, a scenario where the UL TRP(s) can transmit SSBs is not precluded by the WID, but the decision on whether to consider such scenario will depend on RAN1 response.

We have also made the following proposals:

Proposal 1: PC1.5 could be supported with no full power modes, i.e. Rel-15 behavior. It should be feasible for both FWA and handheld UEs.

Proposal 2: For both PC3 and PC2, the specifications should allow the support for full power mode 0 for both FWA and handheld UEs.

Proposal 3: There is no need to introduce new capability for power class per band per declared UL power mode for 3Tx.

Proposal 4: For all power classes, reuse the legacy 2Tx MPR requirements for 3Tx MPR requirements.

Proposal 5: For ULFPTx mode 0, support 1-layer with 3 Tx ports case for both DFT-s-OFDM and CP-OFDM waveforms, i.e. add 1-layer for 3Tx with W=13100,13010,13001 and add both DFT-s-OFDM and CP-OFDM.

# References

- RP-242394, “WID revision: NR MIMO Phase 5”, RAN#105.

- R4-2417116, “WF on UE RF requirements for MIMO enhancement”, Samsung, RAN4#112-bis.

- R4-2414983, “R19 MPR for 3Tx”, OPPO, RAN4#112-bis.

- R4-2417117, “LS on UE RF issues for Rel-19 MIMO enhancement”, Huawei, RAN4#112-bis.