3GPP TSG-RAN WG4 Meeting #113 	R4-2419512

Orlando, USA, November 18th – 22nd, 2024

Agenda Item: 7.20.3.1

Source: MediaTek inc.

Title: Discussion on SBFD UE-to-UE CLI

Document for: Discussion

# Introduction

The work item (WI) for Evolution of NR duplex operation: Sub-band full duplex (SBFD) has the following objectives [1]:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In the last meeting, RRM group discussed several issues on the impacted RRM specification and whether new RRM requirements are needed to handle the new SBFD operation and CLI handling mechanism, where the issues and agreements are captured in the way forward (WF) [2]. In this contribution, we provide further discussion on the issues captured in the WF.

# Discussion on the RRM impact for CLI handling mechanisms

In the previous meeting, RAN4 RRM captured the following issues in the WF:

### Sub-topic 1-1: RRM impacts of UE-to-UE CLI handling #### Issue 1-1-2: Baseline for defining requirements - Recommended WF - Close the issue, and RAN4 to focus on the exact requirement development rather than the baseline. #### Issue 1-1-3: Measurement methods - Recommended WF: Further discuss the issue #### Issue 1-1-4: Rx beam - Recommended WF: Further discuss the issue #### Issue 1-1-5: Measurement resources - Recommended WF: Further discuss the issue #### Issue 1-1-7: Measurement period - Recommended WF: Further discuss the issue #### Issue 1-1-8: Measurement reporting - Recommended WF: Further discuss the issue #### Issue 1-1-9: Measurement accuracy - Recommended WF: RAN4 to define measurement accuracy requirements for L1-SRS-RSRP and L1-CLI-RSSI in the Performance part of the WI. #### Issue 1-1-10: Scheduling and measurement restriction - Recommended WF: Further discuss the issue #### Issue 1-1-11: Measurement capability - Recommended WF: Further discuss the issue #### Issue 1-1-12: Report mapping - Recommended WF: Further discuss the issue #### Issue 1-1-13: Requirement applicability - Recommended WF:Further discuss the issue

In legacy 5G NR TDD, the RRM specification cover the CLI measurements delay and accuracy for SRS-RSRP and CLI-RSSI. The UE reports these measurements to the gNB to help the dynamic allocation of uplink/downlink in time. Nevertheless, these measurements are based on L3 measurements, i.e. it requires L3 filtering, which has longer measurement delay compared to L1 measurements. Therefore, in SBFD Rel-19 WID objective, new CLI measurements and reporting shall be introduced for L1 based UE-to-UE CLI with new measurement quantities, such as L1-SRS-RSRP, L1-CLI-RSSI.

Issue 1-1-3: Measurement methods: 
RAN1 has the following agreement regarding the measurements methods:

Agreement (from RAN1#118) For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband

In general, RAN4 should specify requirements for the measurement method agreed in RAN1, i.e. method#1 and method#2. However, method#2 requires the UE to measure in the UL SBFD slots, hence, the UE is expected to behave as DL during the measurement period or during the slot carrying the CLI measurement resources. RAN4 should further discuss what limitation and restriction are needed to enable method#2.

Observation 1: Method#2 specified by RAN1 requires the UE to perform measurements on UL resources.

- RAN4 RRM shall define core requirements for method#1: UE measures RSSI within DL subband.

- RAN4 RRM shall further discuss the restriction/limitation of enabling method#2: UE measures RSRP of aggressor UE within UL subband.

Issue 1-1-4: Rx beam: 
From RRM spec point of view, the following spec is already exist in R16 spec: ‘For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET’. Unless RAN1 has a new design or agreement, the above should be reused from R16.

- RAN4 RRM shall consider both R16 CLI measurement requirements assumption, which is ‘For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET’.

Issue 1-1-5: Measurement resources: 
Provided that the measurements methods, resources should be defined by RAN1 WG while measurements accuracy and delay requirements should be defined by RAN4 RRM. Thus, RAN4 should consider the agreements made in RAN1:

Agreement For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands - FFS: Number of resources that can be configured - FFS: UE behavior for measurement Agreement For L1 UE-to-UE CLI measurement and reporting, the following are supported - Wideband CLI-RSRP reporting - Wideband CLI-RSSI reporting - FFS: Subband CLI-RSSI reporting

Based on RAN1 agreement, RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement based on wide-band reporting. In addition, RAN4 to discuss whether there is impact from the measurement resources type defined by RAN1. Firstly, for measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband, existing requirements are applicable provided that the minimum number of resources are satisfied. Second, for measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands, further discussion is needed to identify whether the existing requirements are applicable.

- RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement based on wide-band reporting.

- For measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband, existing requirements are applicable provided that the minimum number of resources are satisfied.

- For measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands, further discussion is needed to identify whether the existing requirements are applicable.

Issue 1-1-7: Measurement period: 
In the previous meetings, RAN4 agreed to define RRM requirements for L1 based UE-to-UE CLI measurements and reporting, also, RAN4 agreed to define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. As mentioned earlier that the main motivation for defining the new requirements is introduce faster reporting based on shorter measurement delay compared to that of R16 CLI measurements. Therefore, RAN4 should consider to define L1 CLI with shorter measurements delay compared to that of L3 CLI defined in R16. Nevertheless, the number of measurement delay samples for R16 CLI-RSRP and R16 CLI-RSSI are equal to three and one sample(s). Therefore, RAN4 can define the new measurement quantity of L1-CLI-RSRP with number of measurements samples less than three. However, given that CLI-RSSI measurement is based on one sample then RAN4 to define L1-CLI-RSSI based on one sample. Nevertheless, the final agreed values should be based on the simulation performance results.

Observation 1: The measurement delay for the new measurement quantities, such as L1-SRS-RSRP, L1-CLI-RSSI, should be faster compared to the existing measurement delay for SRS-RSRP.

Observation 2: The number of samples for the existing measurements of R16 CLI-RSSI is just one sample, while R16 CLI-RSRP has three samples.

- L1-SRS-RSRP, further define single set of requirements for the following options based on simulation performance results: (i) single set of requirements based on 1 sample, (ii) single set of requirements based on 2-3 sample.

Issue 1-1-8: Measurement reporting: 
The current recommended WF is agreeable, which is: (i) RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting; (ii) FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement; and (iii) whether L1-RSRP measurement reporting requirement can be re-used.

- RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting and L1-RSRP measurement reporting requirement can be re-used pending on RAN1 agreement.

Issue 1-1-9: Measurement accuracy: 
The current recommended WF is agreeable, i.e. RAN4 to define measurement accuracy requirements for L1 based UE-to-UE CLI measurement based on the agreed side condition and measurement period in the performance part discussion of this WI.

- RAN4 to define measurement accuracy requirements for L1 based UE-to-UE CLI measurement based on the simulation performance results in the performance part of this WI.

Issue 1-1-10: Scheduling and measurement restriction: 
The current recommended WF is agreeable, i.e. RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement if it is needed and FFS the impact of measurement methods and Rx beam assumption. However, to our understanding that the existing requirements already consider the ‘Rx beam assumptions’ and ‘measurement timing offsets’. Thus, RAN4 should clarify whether existing R16 requirements consider ‘FR2 ‘typeD’ QCI assumption for L1 CLI measurement’ and (ii) Timing difference assumption between SRS arriving timing from aggressor UE and DL timing of serving cell.

- RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement following R16 requirements and FFS the impact of measurement methods and Rx beam assumption.

- RAN4 to clarify whether existing R16 scheduling restrictions requirements already considered ‘Rx beam assumptions’ and ‘measurement timing offsets’.

Issue 1-1-11: Measurement capability:

Provided that measurement capability is based on the RAN1 design and agreements, hence, RAN4 not to discuss measurement capability requirements unless requested by other WGs.

- RAN4 not to discuss measurement capability requirements unless requested by other WGs.

Issue 1-1-12: Report mapping:

Provided that measurement mapping is not related to the type of CLI measurements, hence, RAN4 might wait for RAN1 to complete the framework design for these new measurements.

- RAN4 not to discuss measurement report mapping unless requested by other WGs.

Issue 1-1-13: Requirement applicability:

In the previous meeting, RAN4 discussed applicability of L1-SRS-RSRP and L1-CLI-RSSI measurement requirement in terms of frequency location of measurement resources. In our view, L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE.

- L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE.

# Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 RRM shall define core requirements for method#1: UE measures RSSI within DL subband.

Proposal 2: RAN4 RRM shall further discuss the restriction/limitation of enabling method#2: UE measures RSRP of aggressor UE within UL subband.

Proposal 3: RAN4 RRM shall consider both R16 CLI measurement requirements assumption, which is ‘For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET’.

Proposal 4: RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement based on wide-band reporting.

Proposal 5: For measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband, existing requirements are applicable provided that the minimum number of resources are satisfied.

Proposal 6: For measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands, further discussion is needed to identify whether the existing requirements are applicable.

Proposal 7: L1-SRS-RSRP, further define single set of requirements for the following options based on simulation performance results: (i) single set of requirements based on 1 sample, (ii) single set of requirements based on 2-3 sample.

Proposal 8: RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting and L1-RSRP measurement reporting requirement can be re-used pending on RAN1 agreement.

Proposal 9: RAN4 to define measurement accuracy requirements for L1 based UE-to-UE CLI measurement based on the simulation performance results in the performance part of this WI.

Proposal 10: RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement following R16 requirements and FFS the impact of measurement methods and Rx beam assumption.

Proposal 11: RAN4 to clarify whether existing R16 scheduling restrictions requirements already considered ‘Rx beam assumptions’ and ‘measurement timing offsets’.

Proposal 12: RAN4 not to discuss measurement capability requirements unless requested by other WGs.

Proposal 13: RAN4 not to discuss measurement report mapping unless requested by other WGs.

Proposal 14: L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE.

# References

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June 2024.

- R4-2416858, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, Hefei, China, Oct 2024.