3GPP TSG-RAN WG4 Meeting #113 	R4-2419513

Orlando, USA, November 18th – 22nd, 2024

Agenda Item: 7.20.3.2

Source: MediaTek inc.

Title: Discussion on SBFD Operation

Document for: Discussion

# Introduction

The work item (WI) for Evolution of NR duplex operation: Sub-band full duplex (SBFD) has the following objectives [1]:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In the last meeting, RRM group discussed several issues on the impacted RRM specification and whether new RRM requirements are needed to handle the new SBFD operation and CLI handling mechanism, where the issues and agreements are captured in the way forward (WF) [2]. In this contribution, we provide further discussion on the issues captured in the WF.

# Discussion on the RRM impact due SBFD operation

In SBFD operation UE remains a half-duplex node. This implies that in SBFD symbols, the UE can perform either DL reception (on DL subbands) or UL transmission (on UL subband), but not both. To avoid collisions between DL receptions and UL transmissions, the network must provide a way for UE to determine the link direction on SBFD symbols, i.e., whether the UE transmit or receive in a SBFD symbol. Therefore, several issues were brought up to the RRM discussion by multiple companies, which are captured in the WF.

#### Issue 1-2-1: Requirements for SSB based measurement Agreements (from online session) - For SBFD-aware UE, existing requirements apply for SSB-based serving cell measurement. Further discuss for SSB based neighbour cell measurement in RAN4. - Note: The serving cell is SBFD cell, and the neighbour cell is SBFD or non-SBFD cell. Agreement from the last meeting: - Down-select one of the two options: - Option 1: (Samsung, MTK, QC, Nokia, ZTE, HW, LGE, CATT, vivo) - As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. - This can be revisited if it proves to be conflicting with further RAN1 agreements. - Option 2: (E///) - For inter-frequency measurement without gap, UL transmission is prioritized. - Follow option 1 in other cases.

Before assessing which measurements are impacted with the SBFD operation, RAN4 RRM should take into consideration the agreements made in RAN1. Selected RAN1 agreements related to the measurements are provided below:

In RAN1#117, the following agreements were made. SBFD TX/RX/measurement procedures Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

Figure 1: Serving cell (L1/L3) and neighbouring cell SSB based measurements

Based on the agreements from RAN1 and RAN4, L1/RLM/BFD SSB based measurements are prioritised to the UL transmission. Yet, RAN4 highlighted that SSB based neighbouring cell measurements are not part of the RAN1 discussion and the issue should be discussed and resolved in RAN4. Therefore, we provide our analysis regarding this matter.

For the neighbouring cell measurement scenarios that the SSB falls within the active BWP, the neighbouring cells and serving cell are measured at the same time within the SMTC. Now, given that serving cell measurements are prioritised to UL transmission, therefore, other neighbouring cell with SSB within the active BWP shall be prioritised over UL transmission and existing requirements shall apply. In addition, when the neighbouring cell SSB is outside the active BWP then MG is needed, and given that MG interrupt UL/DL, thus, the existing requirements can apply.

Observation 5: The serving SSB resources are prioritized over UL transmission and hence there RLM/BFD/RRM measurements based SSB are not impacted by the SBFD operation.

Observation 6: The neighbouring SSB resources within the active BWP are measured with serving cell measurements, hence, SSB based measurements are not impacted by the SBFD operation.

- For SBFD-aware UE, SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission for SSB based neighbouring cell measurement.

- For SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement.

Issue 2-3-3: Requirements for CSI-RS based measurement:

In RAN1#117, the following agreements were made. SBFD TX/RX/measurement procedures Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

RAN4 agreed to discuss the impact of SBFD operation on the CSI-RS measurement requirements. Based on the agreement made in RAN1 that the CSI-RS can be overlapped with SBFD subbands and the CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. Hence, CSI-RS based measurements are impacted with the SBFD operation. Besides, as stated in the scope of the WID, for SBFD operation in a slot defined as downlink and/or flexible by tdd-UL-DL-ConfigurationCommon, uplink transmission is within uplink subband only whereas downlink reception is within downlink subband(s) only. Therefore, the CSI-RS based measurements are based on the CSI-RS frequency resources within DL usable PRBs. Figure 2 depicts the configured CSI-RS resources, which are overlapped with SBFD subbands. The figure also depicts the CSI-RS resources mapped within DL usable PRBs.

Figure 2: depicts the configured CSI-RS resources overlapped with SBFD subbands.

Agreement: - RAN4 to discuss whether and how L1 CSI-RS measurement requirements would be impacted by the following SBFD operations. - CSI-RS colliding with dynamically scheduled UL transmission - CSI-RS puncturing in frequency domain in SBFD symbols - CSI-RS distributed over SBFD and non-SBFD symbols - Align the understanding whether above SBFD operations are applicable for CSI-RS for RLM, BFR and L1-RSRP, or they are only applicable for CSI-RS for CSI. - RAN4 to discuss whether and how L3 CSI-RS measurement requirements would be impacted by SBFD operations, and further RAN1/2 conclusions can be considered.

Furthermore, in the previous meeting RAN4 reached the following agreement:

Firstly, for the issue of ‘CSI-RS colliding with dynamically scheduled UL transmission’: RAN4 should follow the existing requirement of CSI-RS colliding with dynamically scheduled UL transmission, which prioritize the measurements. Second, for the issue of ‘CSI-RS puncturing in frequency domain in SBFD symbols’: in order for the UE to be able to do CSI-RS based measurements the UE should be aware of which CSI-RS resources are overlapped with SBFD subband. To ensure this, RAN4 can highlight that to RAN2 to ensure that the signalling design cover the issue. Besides, RAN4 should discuss whether the measurements are based on two DL SBFD or a single DL SBFD. Third, for the issue of ‘CSI-RS distributed over SBFD and non-SBFD symbols’: the accuracy of the SBFD and non-SBFD symbols are different due to the interference among these symbols, therefore, the symbols used for a measurement should be of the same symbol type. Fourth, to our understanding that the CSI-RS is applicable for all L1/RLM/BFD serving cell measurements. Fifth, given that the UE can’t tell the exact CSI-RS puncturing of the neighbouring cell, hence, our preference to deprioritize the CSI-RS based L3 measurements for SBFD operation.

Observation 7: The configured CSI-RS resources can be overlapped with SBFD subbands and hence the CSI-RS based measurements might be impacted.

Observation 8: The RRM requirements for CSI-RS based measurements are defined by the minimum number of PRBs of CSI-RS resources used for the measurements.

- For SBFD-aware UE, NW shall configure the UE with the punctured CSI-RS resources by the SBFD subband operation.

- RAN4 shall check whether there is a need to update the CSI-RS based measurements based on the minimum number of CSI-RS resources after being punctured by the SBFD subband.

- For CSI-RS colliding with dynamically scheduled UL transmission, CSI-RS should be prioritize to the UL transmission.

- For CSI-RS puncturing in frequency domain in SBFD symbols, RAN4 shall clarify whether the CSI-RS are based on one DL subband or two DL subband?

- For CSI-RS distributed over SBFD and non-SBFD symbols, RAN4 shall define requirements based on a single symbols type, i.e. either SBFD or non-SBFD.

- RAN4 should assume that the SBFD operation is applicable for CSI-RS based L1/RLM/BFD measurements.

- RAN4 should deprioritize CSI-RS based L3 measurements for SBFD operation.

Issue 1-2-3: Requirements for scheduling restriction:

This issue is related to the SBFD operation rather than requirements and measurements only. Therefore, it should be discussed in RAN1 rather than RAN4.

- The issue of DL/UL transition time and whether SSB falls fully into the DL subband impact on SBFD operation should be discussed in RAN1 rather than RAN4 RRM.

Issue 1-2-4: Requirements for RACH requirements:

Based on our observation, we didn’t identify any requirements impact due to the SBFD operation, yet, we are fine with other companies suggestion to check whether there is a need to define test case during the performance part.

- there is no impact on RACH requirements due to SBFD operation.

Issue 1-2-5: Requirements for UL PC and or spatial relation update:

It is not clear to us what UL power control is referred to it in here, besides, we don’t see any impact on spatial relation update. Our preference is to close this issue without agreement on whether there is impact or not. If companies identify there is impact in the future then companies can bring further proposals.

- RAN4 RRM should close this issue of UL PC and spatial relation update without agreement on whether there is impact or not. If companies identify there is impact in the future then companies can bring further proposals.

Issue 1-2-6: Requirements for MG and BWP switch:

The wording of SBFD can be for both UL/DL so at the end all what we need is to keep the requirements as UL slots regardless of whether they are SBFD-UL or TDD-UL.

- There is no need to update the term UL slot for MG and BWP switch.

Issue 1-2-7: Requirements for UL resource muting:

It is not clear which existing RRM requirements that could be impacted or not for UL resource muting. Our preference is to close this issue without agreement on whether there is impact or not. If companies identify there is impact in the future then companies can bring further proposals.

- RAN4 RRM should close this issue of UL resource muting without agreement on whether there is impact or not. If companies identify there is impact in the future, then companies can bring further proposals.

Issue 1-2-8: Requirements for generic SBFD operation:

It is not clear which existing RRM requirements that could be impacted by generic SBFD operation. From the previous meeting, it seems that this issue is the same as scheduling restriction, which already discussed in another issue. Thus, RAN4 should close this issue with no further agreement.

- RAN4 RRM should close this issue of generic SBFD operation with no further agreement.

# Summary

In this contribution, we have the following proposals:

Proposal 1: For SBFD-aware UE, SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission for SSB based neighbouring cell measurement.

Proposal 2: For SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement.

Proposal 3: For SBFD-aware UE, NW shall configure the UE with the punctured CSI-RS resources by the SBFD subband operation.

Proposal 4: RAN4 shall check whether there is a need to update the CSI-RS based measurements based on the minimum number of CSI-RS resources after being punctured by the SBFD subband.

Proposal 5: For CSI-RS colliding with dynamically scheduled UL transmission, CSI-RS should be prioritize to the UL transmission.

Proposal 6: For CSI-RS puncturing in frequency domain in SBFD symbols, RAN4 shall clarify whether the CSI-RS are based on one DL subband or two DL subband?

Proposal 7: For CSI-RS distributed over SBFD and non-SBFD symbols, RAN4 shall define requirements based on a single symbols type, i.e. either SBFD or non-SBFD.

Proposal 8: RAN4 should assume that the SBFD operation is applicable for CSI-RS based L1/RLM/BFD measurements.

Proposal 9: RAN4 should deprioritize CSI-RS based L3 measurements for SBFD operation.

Proposal 10: The issue of DL/UL transition time and whether SSB falls fully into the DL subband impact on SBFD operation should be discussed in RAN1 rather than RAN4 RRM.

Proposal 11: there is no impact on RACH requirements due to SBFD operation.

Proposal 12: RAN4 RRM should close this issue of UL PC and spatial relation update without agreement on whether there is impact or not. If companies identify there is impact in the future then companies can bring further proposals.

Proposal 13: There is no need to update the term UL slot for MG and BWP switch.

Proposal 14: RAN4 RRM should close this issue of UL resource muting without agreement on whether there is impact or not. If companies identify there is impact in the future, then companies can bring further proposals.

Proposal 15: RAN4 RRM should close this issue of generic SBFD operation with no further agreement.

# References

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June 2024.

- R4-2416858, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, Hefei, China, Oct 2024.