3GPP TSG-RAN WG4 Meeting #113 	R4-2419515

Orlando, USA, November 18th – 22nd, 2024

Agenda Item: 7.25.2

Source: MediaTek inc.

Title: Discussion on RRM requirements for XR

Document for: Discussion

# Introduction

The work item (WI) for XR (eXtended Reality) for NR Phase 3 has the following objectives [1]:

The Rel-19 XR ph3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. - Note: Check in RAN#105 (check also other WG involvement if needed). - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Note: LCP implementation complexity need to be taken into account when evaluating solutions. - Note: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - If justified, define a mechanism for transmitter to inform the receiver of SN gap (or missing SNs) in PDCP. - Specify Core requirements related to the above objectives as necessary [RAN4]

In this contribution, the potential directions that leads to enhance the gap cancellation mechanisms are discussed then other RRM impact issues are raised, which requires some studies.

# Discussion

On one hand, the typical characteristics of XR DL and UL packet traffic is periodic and bursty with a tight delay budget [2]. On the other hand, measurement gaps are periodic with DL/UL data interruption duration of up to 6 ms. As shown in Figure 1, when the XR packet arrival timing overlaps with RRM measurement occasions, the XR transmission/reception will be impacted and therefore causing reduced system capacity.

Figure 1 Periodic XR traffic overlapping with RRM measurement occasions diminishes system capacity

According to the WID objectives, RAN4 RRM should specify enhancements allowing transmission/reception in the gap occasions or scheduling restrictions slots/symbols. Hence, in this contribution, we provide potential directions for completing this objective.

# Discussion on scenarios for measurement skipping

In the previous RRM meeting, issues related to the scenarios are captured in the way forward [3].

Issue 1-1: Deployment scenarios:

< Agreement>: - Frequency ranges - Agree on FR1 and FR2-1 - Scenarios - Start the discussion for - NR-SA - Carrier aggregation <Way forward>: - Further discussion needed for - FR2-2 - NR-DC, EN-DC, NE-DC

Due to the limited TU for this WI, RAN4 should de-prioritize FR2-2 and NR-DC, EN-DC, NE-DC.

- RAN4 should de-prioritize FR2-2 and NR-DC, EN-DC, NE-DC.

Issue 1-2: Types of measurements to be considered:

<Way forward>: - Option 1: Agree at least on the following and further discuss other scenario - Intra-frequency - Inter-Frequency - LTE inter-RAT - Option 2: Agree at least on the following and further discuss other scenario - Intra-frequency - Inter-Frequency - LTE inter-RAT - XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC - Option 3: include all related scenarios

Based on existing requirements of Rel-17 Pre-MG, the minimum time offset to cancel a measurement gap used for 5G NR intra-f/inter-f is equal to 5ms. However, when it comes to consider gap cancellation for inter-RAT, RAN4 should keep in mind that similar procedure to other WI should be followed that no impact on existing design for measuring inter-RAT LTE/3G/2G. In addition, it should be noted that the minimum time offset to cancel MG for 3G/2G is much longer compared to that used for 5G/4G. Therefore, RAN4 should either allow longer MG cancellation delay for inter-RAT measurements or that the UE doesn’t have to measure 3G/2G when operating with XR traffic. In addition, RAN4 agreed to discuss possible solution on how to handle the measurement delay extension. In our view, given that the measurement delay is scaled up with the number of measured frequency layers, hence, the measurement delay extension R4 RRM can be reduced by reducing the number of frequency layers to be measured. However, reducing the number of frequency layers for inter-frequency NR and inter-RAT LTE might have impact on mobility. Nevertheless, given that the UE is operating in XR, which requires high data rate, it is unlikely that the UE might handover to 2G/3G. Therefore, RAN4 RRM should specify that the UE is not required to measure 2G/3G inter-RAT frequency layers.

- In order to maintain short time offset for gap cancellation, XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC.

- In order to reduce the measurement delay, RAN4 shall specify that the UE operating with XR is not required to measure 2G/3G frequency layers.

- RAN4 RRM shall consider that XR requirements are applicable to (i) intra-freq, (ii) inter-freq, and (iii) LTE inter-RAT.

Issue 1-3/1-4: L3/L1 Measurements without gaps with scheduling restrictions:

< Agreement> from meeting RAN4#112: - As starting point, RAN4 to discuss the Tx/Rx in occasions of L3 measurement. - The progress in RAN1/2 should be taken into account. - FFS whether/when to start the discussion of Tx/Rx in occasions of scheduling restriction due to L1 operation. - RAN4 to discuss - The potential impact on measurement requirements due to measurement cancellation - The corresponding solution to address the impact if needed. < Way forward> from meeting RAN4#112bis: Issue 1-3: L3 Measurements without gaps with scheduling restrictions • Option 1: Support skipping of measurements without gaps • Option 2: Measurement skipping does not apply for measurements without gaps Issue 1-4: L1 measurements with scheduling restrictions and procedures • Option 1: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc • Option 2: Measurement skipping does not apply for L1 measurements and procedures

RAN4 RRM agreed to discuss the impact of L3 measurements occasions on XR Tx/Rx. Meanwhile, the impact of L1 measurements remains for further study. The main reason is that when L3 measurements is based on SSB outside the active BWP, the UE requires Tx/Rx interruption over a slot-level for all CCs in the form of MG or NCSG or NeedForInterruption (NFG). Out of the later measurements criteria, MG can have the most severe impact on Tx/Rx due to its long interruption period, which can be up to 6ms. On the other hand, scheduling restriction causes interruption on a symbol-level for the serving cell CC. Therefore, given the workload of this WI compared to the TU allocated for it, we suggest to focus the RRM discussion on L3 measurements impact due to MG.

Observation 1: Measurement gap based L3 measurements is the main cause of reducing XR system capacity, while the scheduling restriction impact of L1/L3 can be tolerated.

- RAN4 should keep the scheduling restriction impact on XR traffic FFS for now until there are sufficient progress in MG cancellation impact.

# Discussion on measurements with gaps

In the previous RRM meeting, issues related to the scenarios are captured in the way forward [3].

Issue 2-1: General approach for defining requirements for measurements with gaps:

Issue 2-1: General approach for defining requirements for measurements with gaps: <Way forward>: - Option 1: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type. - Option 2: As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS. Issue 1-5: L1/L2 triggered mobility measurements and procedures: <Way forward>: - Option 1: support skipping of L1 measurements and procedures for LTM within and outside gap - Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM

In our view, the above two issues should be merged. Besides, when it comes to discuss the priority of different gaps and interruptions, we should take into consideration the workload and necessity to introduce such enhancements. Also, RAN4 should consider whether the solution for a specific feature can be extended to newer features. For example, whether the solution for Type-1 MG can be applied for concurrent gaps, NCSG, Pre-MG, etc. Therefore, RAN4 RRM should focus on the solution mechanism at this stage of discussion then RAN4 can work the solutions for a specific feature if it is found that some gap features cannot be supported.

- RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.

Issue 2-2: Types of measurement gaps to consider:

In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG. <Way forward>: Please discuss the priorities for RAN4 to work based on the following table: Cap types; First prio; Second prio; Don’t include. Type 1. Type 2. NCSG. MUSIM. Positioning. Rel 18 concurrent gap. PreConfigured.

RAN4 shall decide on whether the UE is expected to do all the measurements while operating with XR data traffic. In our view, Type-1 should be considered with high priority, while all other measurements gaps/interruption can be second priority. Besides, RAN4 should discuss the necessity of having positioning and MUSIM measurement gaps while the UE is operating with XR traffic.

- RAN4 shall discuss whether the UE is required to perform positioning and MUSIM measurements while the UE is operating with XR data traffic.

# Discussion on XR skipping requirements impact

In the previous RRM meeting, issues related to the scenarios are captured in the way forward [3].

## Issue 3-1: How to capture impact of measurement skipping <Way forward>: - Option 1: - As minimum requirements, cell detection, index reading, and measurement period is extended. - FFS how to extend - Reuse existing measurement accuracy requirements. - Option 2: - For the cases where measurement impact is needed - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended. - FFS how to extend. - Reuse existing measurement accuracy requirements. - For the cases where measurement impact is not needed - FFS the feasible conditions and necessary UE assistance information. ## Issue 3-2: Principles for measurement delay calculation <Way forward>: - Option 1: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay. - Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period. - Other options are not precluded ## Issue 3-3: Conditions for measurement skipping <Way forward>: - Proposal 1: Define a max number of occasions cancelled during a measurement period. - Proposal 2: RAN4 to discuss which conditions the measurement gap configurations need to meet to be relevant for the UE XR operation, e.g., - minimum MGL, - maximum MGL, - maximum MGRP, etc. - Proposal 3: RAN4 to discuss the impact on RRM samples and how they are combined during the UE XR operation, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements. - Proposal 4: Other conditions.

Furthermore, RAN4 discussed the impact on the measurements delay requirements due to MG cancellation. The number of samples for a single layer measurement was decided based on simulation evaluation for different channel environment. Given that the XR is meant to operate with NR devices then the focus shall be on the impact of measurement gap cancellation and how to compensate the dropped measurement samples. In our view, when the XR data leads to MG cancellation then the total measurement delay shall be extended by a puncturing scaling factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window.

Observation 2: Number of samples for measurement delay was calculated based on simulation evaluation under different channel environment.

- RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic.

- Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay.

- RAN4 shall introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window.

# Discussion on time offset for measurement skipping processing

In the previous RRM meeting, issues related to the scenarios are captured in the way forward [3].

## Issue 4-1: Time offset requirement for measurement gap skipping <Way forward>: - Option 1: 7 ms - Option 2: 5 ms - Option 3: DCI decoding time defined in TS 38.214 + [5]ms - Option 4: assume subclause 5.3.1 of TS38.214 as a baseline for X offset. - Option 5: Fraction of a slot - Option 6: besides the DCI decoding delay, no additional processing delay is needed. - Option 7: N2 symbols, which is to follow the same approach for Rel-17 positioning about the time offset between UE determination of the presence of DL channels and DL PRS processing window (N2 is about UE PUSCH preparation procedure time, which is defined in Clause 6.4, TS38.214)

Based on existing requirements of Rel-17 Pre-MG, the minimum time offset to cancel a measurement gap used for 5G NR intra-f/inter-f is equal to 5ms. However, when it comes to consider gap cancellation for inter-RAT, RAN4 should keep in mind that similar procedure to other WI should be followed that no impact on existing design for measuring inter-RAT LTE/3G/2G. In addition, it should be noted that the minimum time offset to cancel MG for 3G/2G is much longer compared to that used for 5G/4G. Therefore, RAN4 should either allow longer MG cancellation delay for inter-RAT measurements or that the UE doesn’t have to measure 3G/2G when operating with XR traffic. If the XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC, then the time offset requirements for MG skipping can be 5ms.

- If the XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC, then the time offset requirements for MG skipping can be 5ms.

# Discussion on need/feasibility of UE assistance information

Issue 5-1: General on UE assistance information (UAI):

<Way forward>: - Companies are tasked to discuss the need and feasibility of UAI, based on the following options - Option 1: UAI is specified - Companies supporting this option are encouraged to bring contribution with exact proposals on what information is to be included in UAI. - Option 2: UAI is not specified

In existing legacy NR spec, the UE assistance information (UAI) is a special corresponding message to the RRC reconfiguration, which is sent from the UE to the NW, in which, the UE inform the NW with its internal status. In addition, not all the UAI messages have defined corresponding actions from the NW nor from the UE, yet, some UAI messages indicate the initiation of a different UE behaviour. For example, A UE capable of relaxing its RLM measurements of a cell group in RRC_CONNECTED state shall initiate the procedure for providing an indication of its relaxation state for RLM measurements (a UEAssistanceInformation message with rlm-MeasRelaxationState) upon being configured to do so, and upon change of its relaxation state for RLM measurements in RRC_CONNECTED state.

For Rel-19 XR, RAN1 has discussed different possibilities for UAI, which are summarised in their agreements listed in the LS sent to RAN4 [4]. This includes: (i) UE assistance information related to measurement occasions, (ii) UE assistance information related to channel conditions, (iii) UE assistance information related to traffic, and (IV) UE assistance information related to UE mobility. In addition, RAN1 has deprioritised UAI related to channel condition, traffic and UE mobility. Besides, RAN1 mentioned that there is no consensus on whether to support UAI related to measurements occasions. Given that RAN1 has discussed this in length without a clear consensus, therefore, RAN4 RRM can deprioritize the UAI based solution.

Observation 3: RAN1 discussed UAI in length without clear outcome on how to use it.

- RAN4 shall follow RAN1 agreement to deprioritise the UAI based solution.

# Discussion on additional skipping schemes

Issue 6-1: Semi-static schemes:

<Way forward>: - Option 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1. - Option 2: RAN4 also suggest RAN1/2 to introduce the semi-static approach (Alt.3) that NW indicate the indicate occasions where to skip gaps/restrictions, and UE can have a stable measurement pattern/period. - Option 3: To guarantee mobility performance, one potential improvement on dynamic indication by DCI is configuring gaps to be skipped during a time period by RRC in advance and gaps indicated by DCI should be subset of gaps configured by RRC - Option 4: Follow RAN1 working assumption and consider DCI only

From implementation point of view, semi-static approach that NW indicate which occasions to be skipped using RRC and MAC-CE is preferable.

- RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

# Discussion on reply LS to RAN1

Issue 7-1: Response on RAN1 working assumption and time offset:

<Way forward>: - Companies are encouraged to bring analysis on the feasibility of the working assumption from the RAN1 LS in R4-2414910.

RAN4 should highlight the impact raised during the RRM discussion also to report to RAN1 the agreed value for the time offset needed to complete the gap skipping. Besides, RAN4 shall indicate that based on RRM discussion, companies prefer to bring back the semi-static solution too.

- RAN4 to send reply to RAN1 with the following information: (i) XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC, (ii) time offset for gap skipping is equal to 5ms, and (iii) RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

# Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 should de-prioritize FR2-2 and NR-DC, EN-DC, NE-DC.

Proposal 2: In order to maintain short time offset for gap cancellation, XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC.

Proposal 3: In order to reduce the measurement delay, RAN4 shall specify that the UE operating with XR is not required to measure 2G/3G frequency layers.

Proposal 4: RAN4 RRM shall consider that XR requirements are applicable to (i) intra-freq, (ii) inter-freq, and (iii) LTE inter-RAT.

Proposal 5: RAN4 should keep the scheduling restriction impact on XR traffic FFS for now until there are sufficient progress in MG cancellation impact.

Proposal 6: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.

Proposal 7: RAN4 shall discuss whether the UE is required to perform positioning and MUSIM measurements while the UE is operating with XR data traffic.

Proposal 8: RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic.

Proposal 9: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay.

Proposal 10: RAN4 shall introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window.

Proposal 11: If the XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC, then the time offset requirements for MG skipping can be 5ms.

Proposal 12: RAN4 shall follow RAN1 agreement to deprioritise the UAI based solution.

Proposal 13: RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

Proposal 14: RAN4 to send reply to RAN1 with the following information: (i) XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC, (ii) time offset for gap skipping is equal to 5ms, and (iii) RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

# References

- RP-234080, “New WID: XR (eXtended Reality) for NR Phase 3”, MediaTek (Moderator), Dec 2023.

- TR 38.838, v17.0.0, “Study on XR (Extended Reality) Evaluations for NR (Release 17)”, 2021-12.

- R4-2416864, “WF on RRM requirements for XR_Ph3”, Hefei, China, Oct, 2024.

- R1-2405736, “LS on UE assistance information”, Nokia, May, 2024.

- R1-2407561, “LS on gaps/restrictions that are caused by RRM measurements”, Maastricht, NL, Aug, 2024.