3GPP TSG-RAN WG4 Meeting #113	R4-2419520

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	6.18.1

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN B252 UE RF requirements

Document for:	Discussion

- Introduction

In RAN#104 plenary meeting, the spectrum WID on introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) was approved in [1]. One of the main objectives is to specify UE RF core requirements for fulfilling the urgent deployment demand.

In the contribution, the consideration for IoT-NTN B252 UE RF requirements is provided for discussion.

- Discussion

2.1		Spectrum WI scope for an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)

First, regarding UE RF requirements, the objective of the WI RF core part is shown below for reference.

- Specify a new E-UTRA NTN FDD band for NB-IoT/eMTC NTN operation, with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario; - Support UE Categories NB1, NB2, M1; - Support UE Power Class 3 (+23dBm) and Power Class 5 (+20dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. The specification work of this WI shall leverage the studies and requirements for NR/LTE NTN n256/B256, n255/B255, n254/B254, where applicable.

2.2		Previous Meeting Agreements

In RAN4#112bis, the approved WF [2] provided the guidance for following works. The partial contents are listed below for reference.

Issue 2-1-2: UE TX-RX Separation - Agreement - Consider 180MHz default Tx-Rx separation as starting point. - Support of flexible Tx-Rx separation is not precluded. - Applicability for other bands should also be considered.

Issue 2-1-3: UE TX Regulatory - Agreement - Further discuss whether and how NTN antenna gain could be considered when evaluating other IoT-NTN UE TX requirements related to regulatory requirement. Issue 2-1-4: UE Maximum Output Power for Cat M1 and NB1/NB2 - Agreement - FFS on the solutions - FFS on whether the general solution is needed. Issue 2-1-10: Out-of-band blocking for Cat-M1 - Agreement - FFS Issue 2-1-11: Out-of-band blocking for Cat-NB1/NB2 - Agreement - Further study whether same issue as B256 applies.

2.3		UE RF requirements

2.3.1	Tx-Rx frequency separation

NR-NTN bands n256, n255, n254 and n252 are allowed to support TX-RX frequency separation.

Observation 1: NR-NTN bands n256, n255, n254 and n252 are allowed to support variable TX-RX frequency separation.

It also observed that RAN2 specifications already allow NB-IoT to configure SIB2-UL and SIB1-DL center frequencies independently. Satellite industry has a strong need to use different TX-RX spacing in different deployments for IoT NTN.

Observation 2: RAN2 specifications already allow NB-IoT to configure SIB2-UL and SIB1-DL center frequencies independently.

Proposal 1: To support the variable Tx-Rx frequency separation is feasible for all IoT-NTN bands (e.g., 256, 255, 254, 253, and 252).

2.3.2	UE maximum output power (MOP) and MOP tolerance

Regarding the eMTC/NB-IoT NTN UE RF requirements, the TX maximum output power (MOP) requirements can be associated with RF HD-FDD operation.

It is observed that the HD-FDD RF front-end insertion loss (FE IL) is lower compared to FD-FDD FE IL and can improve the RX REFSENS and TX MOP simultaneously.

Observation 3: HD-FDD FE IL is obviously lower compared to FD-FDD IL, which can improve HD-FDD TX MOP with 0.8dB compared to FD-FDD TX MOP.

From EC Decision 2016/687[3], it is observed that a tolerance of up to + 2 dB is to take account of operation under extreme environmental conditions and production spread. Regarding UE TX output power, the power limits are specified as equivalent isotropically radiated power (EIRP) for terminal stations designed to be mobile or nomadic.

Observation 4: From EC Decision 2016/687, at least a tolerance of up to +2 dB is to take account of operation under extreme environmental conditions and production spread.

Observation 5: The actual NTN-IoT PC3 HD-FDD TX output power upper bound could be 25.8dBm (i.e., 23.8dBm+2dB) but exceeds conventional PC3 TX upper power bound of 25dBm.

From ETSI EN 301 908[4], it is observed that PC3 output power upper bound can be specified as 23dBm +2.7dB or 23dBm +3.

Observation 6: From ETSI EN 301 908, PC3 TX output power upper bound can be specified higher (i.e., 23dBm +2.7dB or 23dBm +3dB).

Additionally, in our understanding, the regulatory requirements for in-band TX output power are associated with SAR testing.  The UE TX output power upper bound would be considered when performing SAR testing. Compared with FD-FDD TX SAR, HD-FDD TX SAR issues have been further alleviated.

Regarding WF [2], it guides that FFS on the solution for PC3 HD-FDD TX output power upper bound higher than 25dBm and whether a general solution would be needed for all IoT-NTN bands.

Proposal 2: Regarding solution for PC3 HD-FDD TX output power upper bound higher than 25dBm, consider a specific or general solution for all IoT-NTN bands. The solution is applicable for solving the same issue in PC5 and PC2. The options are as follows.

- Option1: Add additional note to increase the upper tolerance for NTN HD-FDD operation in applicable regions.

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 255 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 254 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 253 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 252 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When UEs recognizes and supports NTN HD-FDD operation in XXX region, the upper tolerance requirement can be increased from +2dB to [+2.7dB or +TT-NTN-HDFDD]. |  |  |  |  |  |  |

- Option 2: Add additional UE capability and NW signal to control NTN HD-FDD power increment.

- Option 2-1:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 255 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 254 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 253 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 252 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The MOP upper tolerance is further increased by ΔPNTNHdFdd from 2dB. |  |  |  |  |  |  |

- Option 2-2:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 255 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 254 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 253 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 252 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The reference power is increased by ΔPNTNHdFdd. |  |  |  |  |  |  |

- Option 3: Add new power classes (e.g., PC4.5_NTN-HD-FDD, PC2.5_NTN-HD-FDD, PC1.75_NTN-HD-FDD).

2.3.3	Out-of-band Blocking for B252 for Cat-M1 and Cat-NB1/NB2

It is observed that B256 has different out-of-band blocking requirements because of using the same duplexer filter to cover B65 and B256.

Table 7.6A.3-2: Out of-band blocking for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 253, 2542, 255 | Finterferer (C`W) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band 256 lower frequency ranges are modified to enable specific implementations. NOTE 2: The power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. |  |  |  |  |  |

Table 7.6B.3-1: Out-of-band blocking parameters for category NB1 and NB2 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -153 |
| 253, 2545, 255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2562 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Void. NOTE 2: Band 256 lower frequency ranges are modified to enable specific implementations. NOTE 3: For operating bands which downlink band frequency range is between 1475.9 MHz < f < 2690 MHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: -20 dBm for the frequency range which is bounded by FDL_low- 200 MHz of the lowest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz and FDL_high + 200 MHz of the highest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz.” NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. NOTE 5: The power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. |  |  |  |  |  |

Currently, the new duplexer filter (e.g., Murata PN: SAYRZ2G00BA0E0A) would be used for B256 and B252.

Observation 7: It is expected that the same duplexer filter would be used for both B256 and B252.

Proposal 3: Consider using tables below as a starting point for band B252 out-of-band blocking requirements.

Table X: Out of-band blocking parameters for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2523 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 3: Band 252 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

Table Y: Out-of-band blocking parameters for category NB1 and NB2 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -153 |
| 2526 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 6: Band 252 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

3 Conclusion

In this paper, our views on IoT-NTN B252 UE RF requirements are provided.

Observation 1: NR-NTN bands n256, n255, n254 and n252 are allowed to support variable TX-RX frequency separation.

Observation 2: RAN2 specifications already allow NB-IoT to configure SIB2-UL and SIB1-DL center frequencies independently.

Proposal 1: To support the variable Tx-Rx frequency separation is feasible for all IoT-NTN bands (e.g., 256, 255, 254, 253, and 252).

Observation 3: HD-FDD FE IL is obviously lower compared to FD-FDD IL, which can improve HD-FDD TX MOP with 0.8dB compared to FD-FDD TX MOP.

Observation 4: From EC Decision 2016/687, at least a tolerance of up to +2 dB is to take account of operation under extreme environmental conditions and production spread.

Observation 5: The actual NTN-IoT PC3 HD-FDD Tx output power upper bound could be 25.8dBm (i.e., 23.8dBm+2dB) but exceeds conventional PC3 TX upper power bound of 25dBm.

Observation 6: From ETSI EN 301 908, PC3 TX output power upper bound can be specified higher (i.e., 23dBm +2.7dB or 23dBm +3dB).

Proposal 2: Regarding solution for PC3 HD-FDD TX output power upper bound higher than 25dBm, consider a specific or general solution for all IoT-NTN bands. The solution is applicable for solving the same issue in PC5 and PC2. The options are as follows.

- Option1: Add additional note to increase the upper tolerance for NTN HD-FDD operation in applicable regions.

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 255 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 254 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 253 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 252 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When UEs recognizes and supports NTN HD-FDD operation in XXX region, the upper tolerance requirement can be increased from +2dB to [+2.7dB or +TT-NTN-HDFDD]. |  |  |  |  |  |  |

- Option 2: Add additional UE capability and NW signal to control NTN HD-FDD power increment.

- Option 2-1:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 255 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 254 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 253 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 252 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The MOP upper tolerance is further increased by ΔPNTNHdFdd from 2dB. |  |  |  |  |  |  |

- Option 2-2:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 255 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 254 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 253 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 252 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The reference power is increased by ΔPNTNHdFdd. |  |  |  |  |  |  |

- Option 3: Add new power classes (e.g., PC4.5_NTN-HD-FDD, PC2.5_NTN-HD-FDD, PC1.75_NTN-HD-FDD).

Observation 7: It is expected that the same duplexer filter would be used for both B256 and B252.

Proposal 3: Consider using tables below as a starting point for band B252 out-of-band blocking requirements.

Table X: Out of-band blocking parameters for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2523 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 3: Band 252 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

Table Y: Out-of-band blocking parameters for category NB1 and NB2 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -153 |
| 2526 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 6: Band 252 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

4 Annex

EC Decision 2016/687

The power limits are specified as equivalent isotropically radiated power (EIRP) for terminal stations designed to be fixed or installed and as total radiated power (TRP) (2) for terminal stations designed to be mobile or nomadic.

Total radiated power (TRP) is a measure of how much power the antenna actually radiates. The TRP is defined as the integral of the power transmitted in different directions over the entire radiation sphere.

ETSI EN 301 908

4.2.2 	Transmitter Maximum Output Power

4.2.2.1 	Transmitter maximum output power for Single Carrier

4.2.2.1.1 	Definition

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub-frame (1 ms).

4.2.2.1.2 	Limits

The UE maximum output power shall be within the shown value in table 4.2.2.1.2-1.

Table 4.2.2.1.2-1: UE power classes

| E-UTRA Band | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| 1 | 23 | ±2,7 |
| 3 | 23 | ±2,7 (see note) |
| 7 | 23 | ±2,7 (see note) |
| 8 | 23 | ±2,7 (see note) |
| 20 | 23 | ±2,7 (see note) |
| 22 | 23 | +3,0/-4,5 |
| 28 | 23 | +2,7/-3,2 |
| 31 | 23 | ±2,7 |
| 33 | 23 | ±2,7 |
| 34 | 23 | ±2,7 |
| 38 | 23 | ±2,7 |
| 40 | 23 | ±2,7 |
| 42 | 23 | +3,0/-4,0 |
| 43 | 23 | +3,0/-4,0 |
| 65 | 23 | ±2,7 |
| 68 | 23 | ±2,7 |
| NOTE: For transmission bandwidths (ETSI TS 136 521-1 [1], clause 5) confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1,5 dB (tolerance = +2,7/-4,2). |  |  |

5 Reference

- RP-241662, “New WID on Introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)”, DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- R4-2417206, “WF on NTN and IoT-NTN bands”, Inmarsat

- EC Decision 2016/687

- ETSI EN 301 908-13 V13.2.0 (2021-11)