3GPP TSG-RAN WG4 Meeting#113							         		R4-2419521

Orlando, Florida, USA, 18 – 22 November 2024

Title: 	PA architectures for handheld NTN HPUE

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.8.2.2.1	Tx requirements	[NR_IoT_NTN_req_test_enh-Core]

Document for:	Approval

# 1	Background

In RAN#112bis, a way forward [1] discussed the PA architecture for NTN HPUE. In this contribution, we discuss some architectural aspects and limitations for handheld UEs.

# Discussion

## 2.1 Way forward inputs

The way forward [1] captured agreements on the NTN HPUE aspects in italic below.

Issue 2-2: UE architecture/type considerations for NR-NTN PC2 UE

Agreement: For PC2 NR NTN 1Tx and 2Tx are to be considered for requirement

Issue 2-3: UE architecture/type considerations for NR-NTN PC1.5 UE

Agreement:

- Both 1Tx and 2Tx for PC1.5 non-handheld UE can be considered as starting point for requirements discussions:

- FFS whether PC1.5 non-handheld UE need to be specified if PC1 non-handheld UE be specified.

- FFS whether PC1.5 handheld UE can be specified or not.

Issue 2-4: UE architecture/type considerations for NR-NTN PC1 UE

Agreement:

- Confirm feasibility of PC1 1Tx for non-handheld UE.

- For PC1, at least 1Tx to be considered for requirements.

- Further discuss consideration of PC1 1Tx, 2Tx, 3Tx for handheld UE.

- Further discuss consideration of PC1 2Tx, 3Tx for non-handheld UE.

Issue 2-5: ACLR

Agreement: Co-existence study results and conclusion is an important factor for deriving out-band emission requirements

  - Check the derived ACLR for A-MPR and additional spurious emission against regulatory information.

Issue 2-8: SEM

Agreement: To re-use existing SEM requirements from PC3 to other PCs as starting point.

In this contribution, we focus on handheld HPUE aspects as this is the core of our products where it is important to maximize the reuse of TN implementations to ensure a smooth integration of NTN into the TN system. This incurs a reasonable additional cost.

## 2.1 Handheld TN HPUE implementations

Current TN handheld implementations are based on three types of PAs:

- 20dBm PAs which are used for Wi-Fi and NR-U (TDD only) and are not of interest for NTN.

- 23dBm PAs that can be used for PC3 1Tx, PC2 2Tx (FDD and TDD) and PC1.5 4Tx (TDD only)

- 26dBm PAs that can be used for PC2 1Tx (FDD and TDD) or PC1.5 2Tx (TDD only).

Furthermore, with the large number of bands, band combinations, ULCA and UL MIMO cases to be supported, the RF front-end tries to share hardware as much as possible.

To that effect, PAs are shared across bands over a large frequency range:

- One PA can cover all the low bands (660-915MHz)

- One PA covers all the mid-bands (1610-2025MHz) => all NTN bands fall in this range.

  - The bands in the 1425-1520MHz use the same PA or a dedicated PA in some cases.

- One PA covers the high bands (2300-2690MHz)

- One PA covers the very high bands (3300-5000MHz)

- One PA covers the >5GHz bands (5150-7125MHz)

- A number of these PAs are duplicated to enable either:

  - Intra-band NC ULCA

  - Inter-band ULCA in the same range

  - UL MIMO

  - HPUE with 2Tx

Observation: In handheld UEs all NTN bands are covered by a single PA that is shared with the TN mid-bands.

To increase the transmitter efficiency and limit the current drain on the battery, the PAs are supplied by dedicated DC/DC using either APT or ET operation. To enable all the possible UL CA and UL MIMO combinations, the PAs share up to three DC/DC converters which can cover a PC2 PA power consumption but may assume some maximum duty cycle.

Observation: In handheld UEs, the PA current drain is limited by the available maximum current from the PA DC/DC which currently can support a 26dBm PA but may assume some duty cycle.

The PAs linearity is optimized for optimal power efficiency and respects the SEM mask and 30dB ACLR for PC3 and 31dB ACLR for PC2 and PC1.5.

For now, the TN SEM and ACLR also applies for NTN PC3, and from agreements in [1], the TN SEM will stay valid for NTN HPUE.

Observation: With the current agreement on SEM and even with a reduced ACLR, it is not clear that a significant power boosting can be achieved. In handheld UEs, this will also be limited by the supported amount of current drain for the PA DC/DCs (this was already concluded for the power boosting option for HPUE).

Given all the above aspects, here are our proposals for reasonable HPUE implementation for NTN in handheld UEs.

Proposal: for handheld UEs, HPUE NTN implementations should be based on existing PA capabilities of 23dBm and 26dBm:

- For PC2

  - 1Tx and 2Tx are covered with TN 26dBm and 23dBm PAs respectively.

  - FFS which MPR reduction/power boosting may be feasible if NTN ACLR is relaxed (but not SEM?)

  - FFS which duty cycle constraints

- For PC1.5

  - 2Tx is covered with TN 26dBm PAs.

  - 1Tx is not supported for handheld since, even with power boosting or dedicated PA, there would be limitations on the shared TN DC/DC current.

  - FFS which MPR reduction/power boosting may be feasible if NTN ACLR is relaxed (but not SEM?)

  - FFS which duty cycle constraints are present.

- For PC1: Assuming up to three 26dBm TN PAs, the maximum power could be up to 30.8dBm but only for 31dB ACLR, where PC1 ACLR is 37dB.

  - PC1 1Tx or 2Tx is not supported for handheld.

  - To enable PC1 3Tx in handheld would require the ACLR to be < 30dB. This assumes 3Tx TxD is enabled, or some form of MIMO is used.

## 2.1 Non-handheld TN HPUE implementations

Although our focus is on handheld PA, we also manufacture higher power PAs for other applications in lower quantities and 1Tx PC1 PAs are definitively feasible. However, it is unclear what NTN ACLR requirement applies as the current ACLR for TN PC1 is 37dB. Considering this, we are not sure if there is an interest to cover 2Tx or 3Tx in non-handheld UEs unless some UL MIMO or beam forming configuration is foreseen.

Proposal: for non-handheld UEs:

- PC1 1Tx is prioritized but the ACLR requirement should be clarified (37dB or lower?)

- If multiple Tx is needed, it is suggested to reuse existing TN PAs and thus target three 26dBm PAs for 3Tx assuming the ACLR requirement is < 30dB for NTN. This assumes 3Tx TxD is enabled, or some form of MIMO is used.

# Conclusion

In this contribution, based on our experience in RF front-end and PA manufacturing, we formulate the following proposals for NTN HPUE.

Proposal: for handheld UEs, HPUE NTN implementations should be based on existing PA capabilities of 23dBm and 26dBm:

- For PC2

  - 1Tx and 2Tx are covered with TN 26dBm and 23dBm PAs respectively.

  - FFS which MPR reduction/power boosting may be feasible if NTN ACLR is relaxed (but not SEM?)

  - FFS which duty cycle constraints

- For PC1.5

  - 2Tx is covered with TN 26dBm PAs.

  - 1Tx is not supported for handheld since, even with power boosting or dedicated PA, there would be limitations on the shared TN DC/DC current.

  - FFS which MPR reduction/power boosting may be feasible if NTN ACLR is relaxed (but not SEM?)

  - FFS which duty cycle constraints are present.

- For PC1: Assuming up to three 26dBm TN PAs, the maximum power could be up to 30.8dBm but only for 31dB ACLR, where PC1 ACLR is 37dB.

  - PC1 1Tx or 2Tx is not supported for handheld.

  - To enable PC1 3Tx in handheld would require the ACLR to be < 30dB. This assumes 3Tx TxD is enabled, or some form of MIMO is used.

Proposal: for non-handheld UEs:

- PC1 1Tx is prioritized but the ACLR requirement should be clarified (37dB or lower?)

- If multiple Tx is needed, it is suggested to reuse existing TN PAs and thus target three 26dBm PAs for 3Tx assuming the ACLR requirement is < 30dB for NTN. This assumes 3Tx TxD is enabled, or some form of MIMO is used.

# References

[1] R4-2417122 WF on co-existence study and HPUE for NR-NTN, Samsung, RAN4#112bis