3GPP TSG-RAN WG4 Meeting #113	R4-2419522

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	6.17.1

Source:	MediaTek Inc.

Title:	Discussion on UE-to-UE coexistence of n252 UL interference with n2 DL and n25 DL

Document for:	Discussion

- Introduction

In RAN#104 plenary meeting, the spectrum WID on introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) was approved in [1]. One of the main objectives is to perform necessary UE-UE co-existence analysis for adjacent bands (e.g., n2 and n25).

In the contribution, the consideration for UE-to-UE coexistence of n252 UL interference with n2 DL and n25 DL is provided for discussion.

- Discussion

2.1		Spectrum WI scope for an NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)

First, regarding UE RF requirements, the objective of the WI RF core part is shown below for reference.

- Specify a new NR-NTN FDD band with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario - Support channel bandwidths and SCS as presented in Table 4.1-1 below; - Mandatory support of channel raster points at step of 10 kHz; - Support UE Power class 3 (+23dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. Table 4.1-1: Channel bandwidth and SCS system parameters. SCS (kHz); Channel bandwidth (MHz). 15; 5; 10; 15; 20. 30; 10; 15; 20. 60; 10; 15; 20. The specification work of this WI shall leverage the studies and requirements for NR NTN n256, n255, n254, where applicable. All UE requirements specified as part of this WI shall be Release-independent from Rel-17.

2.2		Previous Meeting Agreements

In RAN4#112bis, the approved WF [2] provided the guidance for following works. The partial contents are listed below for reference.

Issue 1-1-2: General Methodology for UE-to-UE Coexistence between n252 UL and n2/b2/n25/b25 DL - Agreement: - Define a new more relaxed spurious emission requirement for TN-NTN UE to UE coexistence compared to -50dBm/MHz, that allows the NTN service to work effectively, across the full n252 band, whilst still protecting the TN UE at least in the majority of cases - Further discuss the actual value and methodology for the requirement (e.g., corresponding separation assumption). - Companies are requested to propose what emission level in dBm/MHz could be achieved for frequencies below 1995 MHz, considering the UE RF implementation, different uplink channel bandwidths and RB allocations. - Further discuss how to capture the solutions in the specification.

2.3		UE-to-UE Coexistence

2.3.1	Configuration for evaluating the n252 UE-to-UE Coexistence between n252 UL and n2/b2/n25/b25 DL

To evaluate UE-to-UE coexistence requirements between n252 UL and n2/n25 DL, the configuration for testing channels is shown below.

Emission requirement of -30dBm/MHz and -50dBm/MHz for channel bandwidth 5MHz and 10MHz. - Emission below 1995MHz - for n25 - To evaluate A-MPR for the following Fc. - for 5MHz channel bandwidth, Fc: 2002.5, 2007.5, 2012.5, 2017.5 (MHz) - for 10MHz channel bandwidth, Fc; 2005, 2010, 2015 (MHz) - Emission below 1990MHz - for n2 - For n2, do a similar A-MPR evaluation.

The preliminary simulation A-MPR results are as follows.

2.3.2 	N252 UL A-MPR values for N25 DL and N2 DL when UL CBW 10MHz CBW is used.

2.3.2-1 	Consider NTN-UE to TN-UE co-existence requirement = -50dBm/MHz

Observation 1: Regarding UL CBW of 10MHz and co-existence requirement of -50dBm/MHz, the n252 large A-MPR values for n25 DL and n2 DL are shown in Table 1.

| Table 1 | Co-existence requirement = -50dBm/MHz |  |
| --- | --- | --- |
| N252 UL with full RBs Fc, CBW (MHz) | N252 A-MPR (dB) For N25 DL | N252 A-MPR (dB) For N2 DL |
| 2005, 10 | [13] | [5] |
| 2010, 10 | [5] | [1.5] |
| 2015, 10 | [1.5] | 0 |

2.3.2-2 	Consider NTN-UE to TN-UE co-existence requirement = -30dBm/MHz

Observation 2: Regarding UL CBW of 10MHz and co-existence requirement of -30dBm/MHz, the n252 small A-MPR values for n25 DL and n2 DL are shown in Table 2.

| Table 2 | Co-existence requirement = -30dBm/MHz |  |
| --- | --- | --- |
| N252 UL with full RBs Fc, CBW (MHz) | N252 A-MPR (dB) For N25 DL | N252 A-MPR (dB) For N2 DL |
| 2005, 10 | [5] | [0.5] |
| 2010, 10 | [0.5] | 0 |
| 2015, 10 | 0 | 0 |

2.3.3 	N252 UL A-MPR values for N25 DL and N2 DL when UL CBW 5MHz CBW is used.

2.3.3-1 	Consider NTN-UE to TN-UE co-existence requirement = -50dBm/MHz

Observation 3: Regarding UL CBW of 5MHz and co-existence requirement of -50dBm/MHz, the n252 medium A-MPR values for n25 DL and n2 DL are shown in Table 3.

| Table 3 | Co-existence requirement = -50dBm/MHz |  |
| --- | --- | --- |
| N252 UL with full RBs Fc, CBW (MHz) | N252 A-MPR (dB) For N25 DL | N252 A-MPR (dB) For N2 DL |
| 2002.5, 5 | [5.5] | [2] |
| 2007.5, 5 | [2] | 0 |
| 2012.5, 5 | 0 | 0 |
| 2017.5, 5 | 0 | 0 |

2.3.3-2 	Consider NTN-UE to TN-UE co-existence requirement = -30dBm/MHz

Observation 4: Regarding UL CBW of 5MHz and co-existence requirement of -30dBm/MHz, the very small A-MPR values of n252 for n25 DL and n2 DL are shown in Table 4.

| Table 4 | Co-existence requirement = -30dBm/MHz |  |
| --- | --- | --- |
| N252 UL with full RBs Fc, CBW (MHz) | N252 A-MPR (dB) For N25 DL | N252 A-MPR (dB) For N2 DL |
| 2002.5, 5 | [1] | 0 |
| 2007.5, 5 | 0 | 0 |
| 2012.5, 5 | 0 | 0 |
| 2017.5, 5 | 0 | 0 |

2.3.4 	N252 UL A-MPR values for N25 DL when UL partial RB allocations are used.

2.3.4-1 	Consider NTN-UE to TN-UE co-existence requirement = -50dBm/MHz

Observation 5: Regarding UL RB allocation and co-existence requirement of -50dBm/MHz, the n252 A-MPR values for n25 DL are shown in Table 5.

| Table 5 | Co-existence requirement = -50dBm/MHz |  |
| --- | --- | --- |
| N252 UL Fc, CBW (MHz) | N252 UL with full RBs, N252 A-MPR (dB) For N25 DL | N252 UL with partial RBs allocation, N252 A-MPR (dB) for N25 DL |
| 2002.5, 5 | [5.5] | The 10 RBs are located on the right side inside CBW. [0dB] A-MPR |
| 2007.5, 5 | [2] | The half of total RBs is located on the left side inside CBW. [0dB] A-MPR |

Proposal 1: According to the evaluation results of observations 1 to 5, further discuss whether a new more relaxed spurious emission requirement for NTN-UE to TN-UE coexistence compared to -50dBm/MHz, thereby allowing the NTN service to work effectively.

3 Conclusion

In this paper, our views on UE-to-UE coexistence requirements between n252 UL and n2/n25 DL are provided.

Observation 1: Regarding UL CBW of 10MHz and co-existence requirement of -50dBm/MHz, the n252 large A-MPR values for n25 DL and n2 DL are shown in Table 1.

Observation 2: Regarding UL CBW of 10MHz and co-existence requirement of -30dBm/MHz, the n252 small A-MPR values for n25 DL and n2 DL are shown in Table 2.

Observation 3: Regarding UL CBW of 5MHz and co-existence requirement of -50dBm/MHz, the n252 medium A-MPR values for n25 DL and n2 DL are shown in Table 3.

Observation 4: Regarding UL CBW of 5MHz and co-existence requirement of -30dBm/MHz, the very small A-MPR values of n252 for n25 DL and n2 DL are shown in Table 4.

Observation 5: Regarding UL RB allocation and co-existence requirement of -50dBm/MHz, the n252 A-MPR values for n25 DL are shown in Table 5.

Proposal 1: According to the evaluation results in Tables 1 to 5, further discuss whether a new more relaxed spurious emission requirement for NTN-UE to TN-UE coexistence compared to -50dBm/MHz, thereby allowing the NTN service to work effectively.

| Table 1 | Co-existence requirement = -50dBm/MHz |  |
| --- | --- | --- |
| N252 UL with full RBs Fc, CBW (MHz) | N252 A-MPR (dB) For N25 DL | N252 A-MPR (dB) For N2 DL |
| 2005, 10 | [13] | [5] |
| 2010, 10 | [5] | [1.5] |
| 2015, 10 | [1.5] | 0 |
| Table 2 | Co-existence requirement = -30dBm/MHz |  |
| N252 UL with full RBs Fc, CBW (MHz) | N252 A-MPR (dB) For N25 DL | N252 UL with full RBs Fc, CBW (MHz) |
| 2005, 10 | [5] | 2005, 10 |
| 2010, 10 | [0.5] | 2010, 10 |
| 2015, 10 | 0 | 2015, 10 |

| Table 3 |  | Co-existence requirement = -50dBm/MHz |  |  |
| --- | --- | --- | --- | --- |
| N252 UL with full RBs Fc, CBW (MHz) |  | N252 A-MPR (dB) For N25 DL |  | N252 A-MPR (dB) For N2 DL |
| 2002.5, 5 |  | [5.5] |  | [2] |
| 2007.5, 5 |  | [2] |  | 0 |
| 2012.5, 5 |  | 0 |  | 0 |
| 2017.5, 5 |  | 0 |  | 0 |
| Table 4 |  | Co-existence requirement = -30dBm/MHz |  |  |
| N252 UL with full RBs Fc, CBW (MHz) |  | N252 A-MPR (dB) For N25 DL |  | N252 A-MPR (dB) For N2 DL |
| 2002.5, 5 |  | [1] |  | 0 |
| 2007.5, 5 |  | 0 |  | 0 |
| 2012.5, 5 |  | 0 |  | 0 |
| 2017.5, 5 |  | 0 |  | 0 |
| Table 5 | Co-existence requirement = -50dBm/MHz |  |  |  |
| N252 UL Fc, CBW (MHz) | N252 UL with full RBs, N252 A-MPR (dB) for N25 DL |  | N252 UL with partial RBs allocation, N252 A-MPR (dB) for N25 DL |  |
| 2002.5, 5 | [5.5] |  | The 10 RBs are located on the right side inside CBW. [0dB] A-MPR |  |
| 2007.5, 5 | [2] |  | The half of total RBs is located on the left side inside CBW. [0dB] A-MPR |  |

5 Reference

- RP-241661, “New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)”, DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- R4-2417206, “WF on NTN and IoT-NTN bands”, Inmarsat