3GPP TSG-RAN WG4 Meeting #113	R4-2419523

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	6.19.1

Source:	MediaTek Inc.

Title:	Discussion on UE RF requirements for NR-NTN bands to support the Extended L-band and the combined MSS L-band and Extended L-band ranges

Document for:	Discussion

- Introduction

In RAN#104 plenary meeting, the spectrum WID on introduction of extended L-band and combined L-band was approved in [1]. One of the main objectives is to specify UE RF core requirements for fulfilling the urgent deployment demand.

In the contribution, the consideration of UE RF requirements for the new NR-NTN bands are provided for discussion.

- Discussion

2.1		Spectrum WI scope for the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz)

First, regarding UE RF requirements, the objective of the WI RF core part is shown below for reference.

- Specify a new NR NTN FDD band with a UE transmitting at 1668 – 1675 MHz and SAN transmitting at 1518 – 1525 MHz; - Support 5MHz channel bandwidth with 15kHz SCS - Specify new NR NTN FDD band(s) with SAN transmitting at 1518 – 1559 MHz and UE transmitting at 1626.5 – 1660.5 MHz or 1668 – 1675 MHz; - Support channel bandwidths and SCS as presented in Table 4.1-1 below: Table 4.1-1: DL Channel bandwidth and SCS system parameters NTN satellite band; SCS kHz; DL Channel bandwidth (MHz). 5; 10; 15; 20; 25; 30; 35; 40. 15; 5; 10; 15; 20; 25; 30; 35; 40. [n25x]; 30; 10; 15; 20; 25; 30; 35; 40. 60; 10; 15; 20; 25; 30; 35; 40. - Support Asymmetric UE TX-RX Channel BW configurations to allow leveraging the 41 MHz contiguous DL allocation with a smaller UL allocation. NOTE: UL channel allocations spanning across the 1660.5-1668 MHz gap shall not be allowed - For these bands, the following shall be specified: - Support Flexible TX-RX Separation - Mandatory support for Enhanced Channel Raster with raster points at step of 10kHz - Support UE Power class 3 (+23 dBm) as a starting point - Introduce the corresponding SAN and UE RF core requirements. - Introduce the corresponding RRM requirements. - Requirements for protection of the Radio Astronomy in the 1660-1670 MHz range shall be captured - Any relevant additional regional requirements and restrictions will also be captured. - NOTE1: The introduction of ETSI-specific requirements is expected to be addressed at a later stage, due to ongoing discussions within ETSI in respect to NTN - NOTE2: The 1668-1675 MHz range is presently not available for MSS in the continental United States. NOTE: The specification work in this WI shall leverage the studies and requirements for NR NTN n255, and the corresponding IoT NTN Work Item (IoT_NTN_extLband) for Band 253, where applicable. All UE requirements specified as part of this WI shall be Release-independent for the UE starting from Release-17.

2.2		Previous Meeting Agreements

In RAN4#112bis, the approved WF [2] provided the guidance for following works. The partial contents are listed below for reference.

Issue 3-2-1: UE Duplexer - Agreement: - Further discuss whether implementation with one duplexer is feasible, considering the requirements to protect Radio Astronomy in the 1660-1668MHz range between the two UL.

2.3		UE RF requirements

2.3.1	UE Duplexer

Table 1: NR-NTN satellite bands in FR1-NTN

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high |  | Duplex mode |
| --- | --- | --- | --- | --- |
| n253 | 1668MHz – 1675MHz | 1518MHz – 1525MHz |  | FDD |
| n251 | 1626.5MHz – 1660.5MHz |  | 1518MHz – 1559MHz | FDD |
| n250 | 1668MHz – 1675MHz |  | 1518MHz – 1559MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |  |

It is observed that n253 DL frequency range (i.e., 1518-1525 MHz) is within n250 DL frequency range (i.e., 1518-1559 MHz). Additionally, n253 UL frequency range is fully overlapping with n250 UL frequency range. Thus, it seems that one duplexer to cover bands n253 and n250 would be feasible.

Proposal 1: At least, it seems feasible to have one duplexer covering bands n253 and n250. Two options are as follows. Regarding these two options, further NW signal can be considered to control UL RB allocations or A-MPR to protect Radio Astronomy if necessary.

- Option 1: One duplexer covers bands n253 and n250. One duplexer covers band n251. Consider UL RB allocations or A-MPR to protect Radio Astronomy in the 1660-1668MHz range.

- Option 2: One duplexer covers bands n253, n250 and n251. Compared to Option 1, consider UL RB allocations or more A-MPR to protect Radio Astronomy in the 1660-1668MHz range.

2.3.2	Out-of-band Blocking for n253

If using same duplexer to cover n253 and n250 would be allowed, the out-of-band blocking requirements for n253 is needed to be specified accordingly.

Proposal 2: Regarding bands n253, n250 and n251, whether one duplexer covers 2 bands or 3bands, the out-of-band blocking requirements for n253 would be specified accordingly to enable such implementation.

3 Conclusion

In this paper, our views on UE RF requirements for the new NR-NTN bands are provided.

Proposal 1: At least, it seems feasible to have one duplexer covering bands n253 and n250. Two options are as follows. Regarding these two options, further NW signal can be considered to control UL RB allocations or A-MPR to protect Radio Astronomy if necessary.

- Option 1: One duplexer covers bands n253 and n250. One duplexer covers band n251. Consider UL RB allocations or A-MPR to protect Radio Astronomy in the 1660-1668MHz range.

- Option 2: One duplexer covers bands n253, n250 and n251. Compared to Option 1, consider UL RB allocations or more A-MPR to protect Radio Astronomy in the 1660-1668MHz range.

Proposal 2: Regarding bands n253, n250 and n251, whether one duplexer covers 2 bands or 3bands, the out-of-band blocking requirements for n253 would be specified accordingly to enable such implementation.

5 Reference

- RP-241666, “New WID proposal: Introduction of new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz)”, Inmarsat, Viasat, Thuraya

- R4-2417206, “WF on NTN and IoT-NTN bands”, Inmarsat