3GPP TSG-RAN WG4 Meeting #113	R4-2419524

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.27.2

Source:	MediaTek Inc.

Title:	Discussion on RF requirement impact for IoT NTN phase 3

Document for:	Discussion

- Introduction

In RAN#104 plenary meeting, the WID of “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” was approved in [1]. One of the main objectives is to enable multiplexing of multiple UEs via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH and to update RF requirements accordingly if needed.

In this contribution, our views on the RF requirements impact are provided for discussion.

- Discussion

2.1		RF requirement impact for IoT-NTN phase 3

Some of the main Rel-19 WID [1] objectives for IoT NTN are shown below for reference.

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account

In RAN4#112Bis meeting, the agreed WF [2] provides the following works for considering RF requirement impact. The contents are shown as follows.

RF core requirements NOTE1: Enhancements to enable multiplexing of multiple UEs in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH. NOTE2: For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs. NOTE3: For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB. NOTE4: “Agreement: No UE RF requirement impact from symbol-level for NPUSCH” in RAN4#112 R4-2413530 “Way Forward for [112][316] IoT_NTN_Ph3” Issue 1-1: UE RF requirement impact from 3.75 kHz single-tone NPUSCH with symbol-level OCC4 - Agreement: - Encourage companies to reconfirm whether previous agreement would be still valid. - RAN4#112 R4-2413530: “Agreement: No UE RF requirement impact from symbol-level for NPUSCH” Issue 1-2: UE RF requirement impact from 3.75 kHz single-tone NPUSCH with slot-level OCC2/OCC4 - Agreement: - Wait for further progress in RAN1 before further work on inter-slot OCC, if any. Issue 1-3: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2 - Agreement: - Encourage companies to study the potential RF impact. Issue 1-4: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC4 - Agreement: - Encourage companies to study the potential RF impact. Issue 1-5: UE RF requirement impact from NPRACH with OCC2/OCC4 feature - Agreement: - Wait for further progress in RAN1 before further work on inter-symbol/inter-slot OCC, if any.

Regarding Tdocs in RAN1 meetings, the agreed proposals [3-5] and useful evaluations [6-7] are provided below for reference.

RAN1#116bis “Session notes for 9.11.4 Non-Terrestrial Networks (NTN) for NR Phase 3” [3] Agreement For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs.

RAN1#118 R1-2407297 “FL Summary #2 for IoT-NTN” [4] contents below for reference: Agreement The following combinations are considered for further simulation in RAN1 for 3.75kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 2: OCC2, Symbol-level, CDM DMRS with new pattern - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 6: OCC4, Symbol-level, CDM DMRS with new pattern The following combinations are considered for further simulation in RAN1 for 15kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 5: OCC4, Symbol-level, TDM DMRS - Option 7: OCC4, Slot -level, TDM DMRS - Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern Note 1: For TDM, the legacy DMRS pattern, with DMRS symbols appropriately muted/blanked is used. Companies to report their assumption on whether spreading is applied to the legacy DMRS pattern for 15 kHz SCS. Note 2: Companies to report DMRS sequence applied. Agreement For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

RAN1#118bis R1-2409192 “FL Summary #1 for IoT-NTN” [5] contents below for reference: Agreement At least the following schemes are supported for single-tone: - For 3.75kHz SCS OCC for NPUSCH format 1: - OCC length 2, Symbol-level - FFS: DMRS pattern(s) - For 15kHz SCS OCC for NPUSCH format 1: - OCC length 2, Slot-level, CDM DMRS with legacy pattern - FFS: CDM details, e.g. with or without spreading

From SAN’s perspective, to avoid too much phase difference between UEs at maximum frequency offset (e.g., +/- 0.1ppm) is preferred when combing the limited number of UEs (e.g., 4 UEs).

RAN1#118bis R1-2408138 “Discussion on IoT-NTN uplink capacity/throughput enhancement” [6] The legacy DMRS on NPUSCH format 1 does not support UE multiplexing. Regarding this issue, RAN1 has discussed the following two DMRS schemes (e.g., TDM or CDM) for further study in RAN1#116b meeting. According to the current DMRS pattern of single-tone NPUSCH format 1, only one DMRS symbol is transmitted on 1 slot. When NSF UEs are multiplexed via OCC, at least NSF DMRS symbols on NSF slots are needed to ensure the orthogonality of channel estimation between the OCC-multiplexed UEs, whether for TDMed DMRS or CDMed DMRS. Table 1. The required SNR at 10% BLER for 15kHz SCS Non-OCC; -3.55dB. OCC scheme; OCC2; OCC4. TDM DMRS; Slot-level OCC; -2.65dB; *. Symbol-level OCC; -2.55dB; *. CDM DMRS; Slot-level OCC; -3.37dB; *. Fig 6. BLER and aggregated throughput performance evaluation for 15kHz SCS For 15kHz SCS, when OCC4 is applied, all the candidate OCC schemes cannot achieve 10% BLER. The BLER performance degradation is due to worse channel estimation.

RAN1#118bis R1-2408719 “IoT-NTN uplink capacity and throughput enhancement” [7] A ST 15 kHz is specified in TS 38.211 Table 10.1.2.3-1 to be NslotsUL=16 with a slot duration 0.5 ms and ST 3.75 kHz kHz is NslotsUL=16 with a slot duration 2 ms. The time duration of OCC applied to repetitions of NPUSCH Format 1 is 16 ms for OCC length 2 and 32 ms for OCC length 4 with ST 15 kHz, and 64 ms for OCC length 2 and 128 ms for OCC length 4 with ST 3.75 kHz Phase rotation over time duration X ms as summarize in Table 1can be calculated as - Δφ = 2π (rad) × timing drift (us/s) × X (ms) × channel bandwidth (Hz) /2 - Δφ = 2π (rad) × CFO × X (ms) × Fc/2 Where max time drift = 80 ppm and max CFO = ±0.1 ppm. Timing drift 80 ppm Max CFO ±0.1 ppm; Timing drift 80 ppm Max ΔΘ [deg] ST 15 kHz / ST 3.75 kHz; Max CFO ±0.1 ppm Max ΔΘ [deg] ST 15 kHz / ST 3.75 kHz. Slot-level OCC 2 Slot-level OCC 4; 0.216 / 0.864 0.864 / 3.456; 36 / 144 72 / 288. Symbol-level OCC 2 and 4; 0.108; 18. Table 1: Max phase rotation due to time drift and CFO with OCC schemes Observation 1: The maximum phase rotation due to CFO in slot-level OCC length 4 with NPUSCH Format 1 with single tone 15 kHz is not consistent with QPSK modulation. Observation 2: Symbol-level OCC length 2 and 4 with NPUSCH Format 1 with single tone 3.75 kHz and 15 kHz is robust to CFO and timing drift.

Based on references [5][6][7], there was reasonable understanding as follows:

    - Symbol-level OCC2/OCC4 worked at 3.75kHz SCS

    - Slot-level OCC may not work well at 3.75kHz SCS due to accumulated large phase error from CFO

    - Slot-level OCC2 worked at 15kHz SCS

| OCC schemes | Max CFO ±0.1 ppm Max ΔΘ [deg] ST 15 kHz / ST 3.75 kHz |
| --- | --- |
| Slot-level OCC 2 Slot-level OCC 4 | 36 / 144 72 / 288 |
| Symbol-level OCC 2 and 4 | 18 |

Table 1: Max phase rotation due to time drift and CFO with OCC schemes

Observation 1: Based on references [4-7], at least for 3.75kHz SCS single-tone (ST) transmission OCC for NPUSCH format 1, symbol-level OCC is feasible. But some companies consider 3.75kHz ST OCC4 would add larger complexity to eNB scheduler and a bottleneck is more likely to be created in DL which is not orthogonal.

Observation 2: Based on references [4-7] and evaluation, slot-level OCC2/OCC4 seems not feasible for 3.75kHz SCS ST transmission OCC for NPUSCH format 1.

Observation 3: Based on references [4-7] and evaluation, at least for 15kHz SCS ST transmission OCC for NPUSCH format 1, slot-level OCC2 feasible.

Observation 4: Regarding UL gap on OCC operation, there may be likely to be phase discontinuity on either side of an UL transmission gap – this may introduce loss of orthogonality between UEs. Whether such issue would exist needs to be studied.

It is observed that RAN1 thinks symbol-level OCC2 would be feasible for 3.75kHz SCS single-tone NPUSCH and slot-level OCC2 would be feasible for 15kHz SCS single-tone NPUSCH.

- For 3.75kHz SCS OCC for NPUSCH format 1:

    - OCC length 2, Symbol-level

- For 15kHz SCS OCC for NPUSCH format 1:

    - OCC length 2, Slot-level, CDM DMRS with legacy pattern

Proposal 1: RAN4 stops evaluating UE RF requirement impacts for slot-level OCC2/OCC4 of 3.75 kHz single-tone NPUSCH format 1 since RAN1 agreed to remove the options.

Proposal 2: For slot-level OCC2 of 15 kHz single-tone NPUSCH format 1, no UE RF requirement impact.

| OCC schemes | Max CFO ±0.1 ppm Max ΔΘ [deg] ST 15 kHz |
| --- | --- |
| Slot-level OCC 2 | 36 |

Proposal 3: RAN4 stops evaluating UE RF requirement impacts for slot-level OCC4 of 15 kHz single-tone NPUSCH format 1 since RAN1 agreed to remove this option.

Proposal 4: Regarding UL gap on OCC operation, clarify whether there would be phase continuity issue across gap.

3 Conclusion

In this paper, our views on RF requirements impact for NPUSCH with OCC feature are provided as follows:

Observation 1: Based on references [4-7], at least for 3.75kHz SCS single-tone (ST) transmission OCC for NPUSCH format 1, symbol-level OCC is feasible. But some companies consider 3.75kHz ST OCC4 would add larger complexity to eNB scheduler and a bottleneck is more likely to be created in DL which is not orthogonal.

Observation 2: Based on references [4-7] and evaluation, slot-level OCC2/OCC4 seems not feasible for 3.75kHz SCS ST transmission OCC for NPUSCH format 1.

Observation 3: Based on references [4-7] and evaluation, at least for 15kHz SCS ST transmission OCC for NPUSCH format 1, slot-level OCC2 feasible.

Observation 4: Regarding UL gap on OCC operation, there may be likely to be phase discontinuity on either side of an UL transmission gap – this may introduce loss of orthogonality between UEs. Whether such issue would exist needs to be studied.

Proposal 1: RAN4 stops evaluating UE RF requirement impacts for slot-level OCC2/OCC4 of 3.75 kHz single-tone NPUSCH format 1 since RAN1 agreed to remove the options.

Proposal 2: For slot-level OCC2 of 15 kHz single-tone NPUSCH format 1, no UE RF requirement impact.

| OCC schemes | Max CFO ±0.1 ppm Max ΔΘ [deg] ST 15 kHz |
| --- | --- |
| Slot-level OCC 2 | 36 |

Proposal 3: RAN4 stops evaluating UE RF requirement impacts for slot-level OCC4 of 15 kHz single-tone NPUSCH format 1 since RAN1 agreed to remove this option.

Proposal 4: Regarding UL gap on OCC operation, clarify whether there would be phase continuity issue across gap.

4 Annex

TS 36.211 Table 10.1.2.3-1: Supported combinations of , , and  for frame structure type 1.

| NPUSCH format |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3.75 kHz | 1 | 16 | 7 |
|  | 15 kHz | 1 | 16 |  |
|  |  | 3 | 8 |  |
|  |  | 6 | 4 |  |
|  |  | 12 | 2 |  |
| 2 | 3.75 kHz | 1 | 4 |  |
|  | 15 kHz | 1 | 4 |  |

Since the slot duration is 2ms, the RU length spans = 16 slots, which results in 32ms. Figure 1 depicts the slot format for NPUSCH Format 1 with 3.75 kHz SCS.

| CP+Data symbol (275 us) | CP+Data symbol (275 us) | CP+Data symbol (275 us) | CP+Data symbol (275 us) | CP+DMRS symbol (275 us) | CP+Data symbol (275 us) | CP+Data symbol (275 us) | Guard period (75 us) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Slot (2ms) |  |  |  |  |  |  |  |

Figure 1: slot format for NPUSCH Format 1 with 3.75 kHz SCS

Since the slot duration is 0.5ms, the RU length spans = 16 slots, which results in 8ms. Figure 2 depicts the slot format for NPUSCH Format 1 with 15 kHz SCS.

| CP+Data symbol | CP+Data symbol | CP+Data symbol | CP+DMRS symbol | CP+Data symbol | CP+Data symbol | CP+Data symbol |
| --- | --- | --- | --- | --- | --- | --- |
| Slot (0.5 ms) |  |  |  |  |  |  |

Figure 2: slot format for NPUSCH Format 1 with 15 kHz SCS.

TS 36.213 Table 16.5.1.1-3: Number of repetitions () for NPUSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |

RAN1#118bis R1-2409192, “FL Summary #1 for IoT-NTN”

BLER performance of schemes based on 3.75kHz SCS simulations:

| Option |  |  |  | Loss to baseline |
| --- | --- | --- | --- | --- |
| Option 1 | OCC2 | Symbol-level | TDM | HW: 2.31dB CMCC: 0.3dB ZTE: 1.0dB OPPO: 2.5dB (error floor) QC: 0.29dB |
| Option 2 | OCC2 | Symbol-level | CDM new pattern | HW: 4.84dB CMCC: 0.1dB ZTE: -0.1dB OPPO: 0.2dB (error floor) QC: 0dB |
| Option 3 | OCC2 | Slot-level | TDM | HW: 2.87dB CMCC: 1.1dB ZTE: 1.0dB Xiaomi: 2.0dB OPPO: 1.8dB (error floor) QC: 4.37dB |
| Option 4 | OCC2 | Slot-level | CDM legacy pattern | HW: poor performance CMCC: 0.7dB ZTE: 0.1dB OPPO: poor perf (BLER > 10%) QC: 3.32dB |
| Option 6 | OCC4 | Symbol-level | CDM new pattern | HW: poor performance CMCC: 1.1dB ZTE: 0.2dB QC: 0.47dB |

BLER performance of schemes based on 15kHz SCS simulations:

| Option |  |  |  | Loss to baseline |
| --- | --- | --- | --- | --- |
| Option 1 | OCC2 | Symbol-level | TDM | HW: 1.3dB Vivo: 3.21dB CMCC: 0.05dB ZTE: 0.6dB CATT: 0.7dB OPPO: 0.8dB |
| Option 3 | OCC2 | Slot-level | TDM | HW: 1.43dB Vivo: 2.49 dB CMCC: 0.21dB ZTE: 0.8dB CATT: 0.2dB OPPO: 0.8dB |
| Option 4 | OCC2 | Slot-level | CDM legacy | HW: 0.15dB CMCC: 0.09dB (at REP16) ZTE: 0.2dB Xiaomi: 3.5dB CATT: 0.7dB OPPO: 0.2dB QC: 0.37dB |
| Option 5 | OCC4 | Symbol-level | TDM | HW: 3.56dB CMCC: 4.16dB ZTE: 3.5dB CATT: 4.7dB OPPO: poor performance |
| Option 7 | OCC4 | Slot-level | TDM | HW: 4.44dB CMCC: 4.86dB ZTE: 4.4dB CATT: 4.7dB OPPO: poor performance |
| Option 8 | OCC4 | Slot-level | CDM legacy | HW: 1.33dB ZTE: 1.6dB Xiaomi: 4.7dB CATT: 4.4dB OPPO: poor performance QC: 2.82dB |

5 Reference

- RP-241624, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc.

- R4-2416565, “Way Forward for [112bis][315] IoT_NTN_Ph3”, MediaTek Inc.

- Session notes for 9.11.4 Non-Terrestrial Networks (NTN) for NR Phase 3, Agenda Item 9.11.4, Changsha, Hunan Province, China, April 15th – 19th, 2024.

- R1-2407297, “FL Summary #2 for IoT-NTN”, Sony

- R1-2409192, “FL Summary #1 for IoT-NTN”, Sony

- R1-2408138, “Discussion on IoT-NTN uplink capacity/throughput enhancement”, OPPO

- R1-2408719, “IoT-NTN uplink capacity and throughput enhancement”, MediaTek Inc.