3GPP TSG-RAN WG4 Meeting #113	R4-2419532

# Orlando, United States, 18 – 22 November 2024

Agenda item:		7.8.2.2.1

Source: 		Qualcomm Inc.

Title: 		NR NTN high power UE Tx and Rx requirements

Document for:		Approval

## 1	Introduction

The work item for enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved. One of the objectives is high power UE operation for NTN. In this contribution UE RF requirements and feasibility of NTN HPUE are discussed.

## 2	Discussion

Tx Requirements

For UE RF architecture, NR TN HPUE conclusions need further consideration. For NR, PC2 is allowed to use dual Tx (tx diversity) implementation, but PC1.5 is specified to always apply dual Tx. For NTN UL-MIMO has not been considered, as UL-MIMO typically needs good SNR to give benefits. For NR NTN up to PC1.5 is specified currently for TN for TDD bands. The same or similar hardware could be used to support PC1.5 for NTN. Re-use from TN would mean that PC1.5 is based on dual-Tx architecture.

With PC2 being commercialized in handheld devices, also component designs have been evolved to tolerate higher powers. PC2 single Tx measurement results are commonplace in A-MPR evaluations in TN side. Also duplexers have evolved and more duplexers capable of handling PC2 power levels are coming to market.

Looking at commercially available components is not however the best way to the consider feasibility, as often the development of those components really gathers speed only after the standard allowing the use of higher powers becomes available.

Observation 1: Single Tx PC2 PAs and duplexers are feasible.

Observation 2: Typically, the component availability improves once the standard is in place allowing the use of such components – without a market there is limited need to make such components commercially available.

Proposal 1: PC1.5 is feasible for NR NTN for handheld devices. At least 2 Tx solution is feasible for FDD.

For non-handheld devices PC1.5 solution could use similar hardware as for PC1. PC1 for TN is specified for multiple FDD bands and assumes single Tx. TN PC1 solution could be scaled down to support NTN PC1.5 with single Tx, alternatively used to support NTN PC1.

Similar to a handheld device, 2Tx device can also be used in non-handheld devices. There are multiple reasons why also 2Tx PC1.5 needs to be considered. Firstly, similar RF front-end solution for both handheld and non-handheld allows using the same components for both, and therefore taking advantages of the economies of scale. Secondly, there are specific use cases where 2Tx solutions are preferable.

For example, in automotive domain solutions are often 2Tx – this may be even required to have a higher likelihood for working Tx if there is a need to place an emergency call after a crash. Similarly telematics systems may use 2Tx. This again allows taking advantage of components developed for handheld UE ecosystem.

Observation 3: 2Tx PC1.5 solution allows taking advantage of economies of scale of handheld ecosystem.

Observation 4: 2Tx PC1.5 solution is preferred for automotive industry use cases

Proposal 2: PC1.5 is feasible for NR NTN non-handheld devices. Requirements shall be defined at least for 2Tx solution.

Rx Requirements

In previous meeting it was agreed to define RSD requirements similarly as in TN specifications. For n256 [0] dB RSD was agreed, but further study was requested on duplexer isolation and PA interference.

When n256 was developed, accommodations were made to specifically consider co-banding. Therefore, n1 PC2 requirements defined for TN are a good reference for n256. RSD for band n1 was studied in depth with results capture in TR 38.861. Requirements were later developed in a separate WI. It would be unlikely to find commercially available PC2 components designed to specifically address only n256, as even the standard does not yet enable PC2 for any NTN band.

Observation 5: n1 RSD requirements can be used to guide the n256 RSD requirement

Especially for L-band, situation is more challenging than for n256, as the duplex distance is much smaller. This means that both Tx leakage will be stronger, and Tx-Rx isolation is more challenging to reach without compromises to filter insertion loss. The filter passband may also be wider than just the single operating band, as currently many other L-bands are being specified, and co-banding these is likely.

Observation 6: It is highly likely that n255 can be co-banded together with the other L-bands being specified currently.

In addition to filter isolation and Tx noise, it is necessary to consider also the Rx non-linearity in the RSD evaluation. Recently, 54 dBm IIP2 is often used in RSD evaluations for TN bands. An example is provided below, which fully ignores the Tx leakage and is therefore focused to show the impact of Rx non-linearity

Observation 7: Depending on duplexer isolations, Rx IIP2 can cause refsens degradation and it needs to be considered in the RSD evaluation.

Proposal 3: In addition to Tx leakage to Rx, Rx non-linearity needs to be considered in RSD evaluation.

Final Tx leakage values are also dependent on what will be Tx ACLR. Therefore RSD requirements should be finalized only when Tx ACLR requirement is known.

Proposal 4: Decide Tx ACLR requirement first before finalizing RSD requirements.

## 3	Conclusion

In this NTN HPUE operation was discussed. Following observations and proposals were made:

Observation 1: Single Tx PC2 PAs and duplexers are feasible.

Observation 2: Typically, the component availability improves once the standard is in place allowing the use of such components – without a market there is limited need to make such components commercially available.

Proposal 1: PC1.5 is feasible for NR NTN for handheld devices. At least 2 Tx solution is feasible for FDD.

Observation 3: 2Tx PC1.5 solution allows taking advantage of economies of scale of handheld ecosystem.

Observation 4: 2Tx PC1.5 solution is preferred for automotive industry use cases

Proposal 2: PC1.5 is feasible for NR NTN non-handheld devices. Requirements shall be defined at least for 2Tx solution.

Observation 5: n1 RSD requirements can be used to guide the n256 RSD requirement

Observation 6: It is highly likely that n255 can be co-banded together with the other L-bands being specified currently.

Observation 7: Depending on duplexer isolation, Rx IIP2 can cause refsens degradation and it needs to be considered in the RSD evaluation.

Proposal 3: In addition to Tx leakage to Rx, Rx non-linearity needs to be considered in RSD evaluation.

Proposal 4: Decide Tx ACLR requirement first before finalizing RSD requirements.

## 4	References