3GPP TSG-RAN WG4 Meeting # 113	R4-2419538

Orlando, US, Nov 18 – Nov 22, 2024

Agenda Item: 7.8.2.1

Source: MediaTek Inc.

Title: Discussion on NTN coexistence study

Document for: Discussion

Introduction

This document provides simulation results focusing on adjacent channel interference for coexistence between NTN and TN based on the NTN coexistence study [1].

Simulation scenarios and assumptions

Simulation scenario

According to WID [2] and WF [3], RAN4 to evaluate the impact on both uplink (UL) and downlink (DL) performance of TN. The aggressor is high power UE (HPUE) for NTN (aggressor) in both UL and DL. The power class (PC) include PC2, PC1.5 and PC1. The Victim can be TN UL from base station for scenario 4 or TN DL from UE for scenario 5. From the following agreement in WF [3], we summarize the coexistence scenarios shown in Table 1. This document presents the simulation observations relating to the coexistence of NTN with TN for both scenario 4 and scenario 5.

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112 Agreement: - Consider both Scenario 4 and 5 for co-existence study - Prioritize scenario 4 since it is the worst case - Do not consider Scenario 1, 2, 3 and 6 Agreement: Prioritize Urban Macro for Scenario 4and Rural Macro for Scenario 5 Agreement: For the scenario, consider the scenarios in TR 38.863 as a baseline Prioritize GEO and LEO1200 for co-existence evaluation - LEO600 is not precluded for the requirements and the conclusion for LEO1200 can be applied to LEO600

Table 1: Simulation scenarios

| Satellite | Scenarios | Environment | Aggressor | Victim |
| --- | --- | --- | --- | --- |
| GEO | Scenario 4 | Urban Macro | NTN UL | TN UL |
| GEO | Scenario 5 | Rural Macro | NTN UL | TN DL |
| LEO1200 | Scenario 4 | Urban Macro | NTN UL | TN UL |
| LEO1200 | Scenario 5 | Rural Macro | NTN UL | TN DL |

The results presented are for urban/rural environments described in TR38.901, using both AAS and non-AAS antenna models in TR38.821.

For scenario 4 and scenario 5, we simulated all NTN UEs are transmitting at three set of PC level (31 dBm, 29 dBm and 26 dBm) as HPUE for both NR-NTN and IoT-NTN.

We reused the simulation procedure and assumption refer to TR 38.863 [1] as below in details.

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112 Agreement: Agree to use TR 38.863 and WF R4-2217473 for NR-NTN and IoT-NTN HPUE coex study assumptions and scenarios as starting point. The detailed modifications to these references will be discussed and agreed in case-by-case manner. TR 38.863 [1] 6.2.2 System parameters Table 6.2.2.4-1: Simulation assumptions of TN respectively based on NB-IoT and NR NB-IoT standalone; NR. Carrier frequency in GHz; 2; 2. Size of each nominal channel BW in MHz; 0.2; 20. Transmission bandwidth in MHz; 0.18; N/A. Environment; Urban macro Rural; Deployment scenario related, see Table 6.2.2.4-2.. Network layout; 19-sites [57 sectors] with wrap-around; 19-sites 57 sectors with wrap-around. Inter-site distance in meter; 500 for 2GHz band for UMA TBD For Rural; Deployment scenario related, see Table 6.2.2.4-2. System loading and activity; Full buffer 100%; See Table 6.2.1.1-1. Network location; TBD; See Table 6.2.1.1-1. DL subcarrier spacing; 15kHz; 15kHz. UL; See RP-152284[7]; OFDMA. DL power control; No; No. UL power control; TR 36.942[8] section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB; TR 36.942[8]. Frequency reuse; 1; 1. Number of scheduled UE per cell (DL); 1; 1. Number of scheduled UE per cell (UL); 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone; 3. UE antenna height in meter; 1.5; 1.5m. UE TX power in dBm; -40 to 23; -40 to 23. UE antenna gain in dBi; 0; 0. Building penetration loss; TR 45.820[9] Annex D.1; In pathloss model, TR 38.901[10]. Cell selection margin in dB; 3; 3. BS-MS min distance in meters; 35; 35. BS noise figure in dB; 5; 5. UE noise figure in dB; 9; 9. BS-UE path-loss model; TR 36.942[8] macro urban; TR 38.901[10]. Standard deviation of BS-UE log-normal shadow fading in dB; 10; Deployment scenario related, referring to TR 38.901[10]. Shadowing correlation; Inter-cell 0.5 Intra-cell 1; Inter-cell 0.5 Intra-cell 1. Link-level performance model; See Section 6.2.8 Throughtput-SINR mapping. UE distribution; Uniform. Evaluation metrics; SINR vs ACS (as victim); See Section 6.2.8 Throughtput or SNR loss criteria. Table 6.2.2.4-2: Deployment-related parameters of TN (2 GHz) Urban Macro; Rural Macro; Remarks. ISD in meters; 750; 7500; ITU-R Report M.2292[11]. BS Antenna height in meters; 25; 30. UE Outdoor/indoor; 100% Outdoor. UE height in meter; 1.5; 1.5; RP-200559 [12] 3GPP LS to ITU-R WP5D and ITU-R WP5D [IMT_Parameters] [13]. Table 6.2.2.1-2: Set-1 satellite parameters for co-existence study Satellite orbit; GEO; LEO-1200; LEO-600. Satellite altitude; 35786 km; 1200 km; 600 km. Payload characteristics for DL transmissions. Satellite EIRP density; 2GHz; 59 dBW/MHz; 40 dBW/MHz; 34 dBW/MHz. Satellite max TX power in dBm; BW (MHz); 5; 10; 15; 20; 5; 10; 15; 20; 5; 10; 15; 20. SCS 15kHz; 44.53; 47.71; 49.53; 50.81; 46.53; 49.71; 51.53; 52.81; 40.53; 43.71; 45.53; 46.81. SCS 30kHz; 43.98; 47.37; 49.36; 50.64; 45.98; 49.37; 51.36; 52.64; 39.98; 43.37; 45.36; 46.64. Satellite Tx max Gain; 51 dBi; 30 dBi; 30 dBi. Channel bandwidth; 5/10/15/20MHz; 5/10/15/20MHz; 5/10/15/20MHz. 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam; 0.4011 deg; 4.4127 deg; 4.4127 deg. ABS (Adjacent Beam Spacing) of adjacent beams from the central beam; 0.3474 deg; 3.8206 deg; 3.8206 deg. Satellite beam diameter; 250 km; 90 km; 50 km. Payload characteristics for UL transmissions. G/T; 2 GHz; 19 dB K-1; 1.1 dB K-1; 1.1 dB K-1. Satellite Rx max Gain; 51 dBi; 30 dBi; 30 dBi. - 6.2.6 Transmission power control model - 6.2.6.2 NTN UL TPC For the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted to align with UE UL power control parameters used in TR 38.821[6]. - 6.2.6.3 NTN DL TPC For downlink scenario, no power control scheme is applied. 6.3.1 Simulation procedure Adopt following simulation steps. - Step 1: Generate aggressor and victim networks. - NTN central beam is at satellite nadir, surrounded with 6 co-frequency beams. NTN FRFs higher than 1 need to be considered. Assume one NTN aggressor as default. - Deployment of TN network (19 cells with wraparound) refers to Table 6.2.1.1-1 - Step2: UE associations - TN UE are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss. - Deployment of NTN UE refers to Table 6.2.1.1-1. - Step 3: Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS/SAN-UE. This is done for both victim and aggressor networks. - Step 4: Throughput is computed in the victim systems without considering ACI as below: ThputNO ACIbpshz=fSINRICI=fSN+IICI, where: IICI is the inter-cell interference. For TN-NTN SINR calculation, the satellite receiver off angle should be considered in the satellite receiver gain calculation when calculating SINR. Note that such angle is not considered in TR 38.821[6] section 6.1.3 equations. Thus those equations should be used for SINR calculation. - Step 5: Throughput is computed considering ACI as below: ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where: IACI is the adjacent channel interference. - Step 6: RF parameters are determined based on the degradation cause by ACI as below: LossACI=1-ThputACIThputSINGLE

Network layout model

According to WF [3] and TR 38.863, the deployment on system level simulation is implemented as below.

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112 Agreement: - Consider isolation distance in scenario 4 and 5 for both NR-NTN HPUE coex and IoT-NTN HPUE coex. - Use the isolation distance 1.5km as the starting point - Other values for isolation distance are not precluded Agreement: Random dropping on circle, and to use uniform distribution for random. Agreement - Consider 9 UEs for 15kHz, 18 and 36UEs for 3.75kHz single tone as starting point - FFS whether to down scope. Agreement: Option 1 as baseline assumption, other options not precluded. (Network layout model) TR 38.863 [1] 6.2.1 Network layout model Figure 6.2.1.1-1: Isolation distance for Case 1 As defined in Figure 6.2.1.1-1, isolation distance is the distance between the blue-dotted line which represents TN cell boarder and the red line. No UEs deployed in the isolation region is assumed to reduce the calculation complexity. No.; Combination; Aggressor; Victim; Which NTN cell/UE to observe?; Which TN/UE to observe?; Which TN cells in a TN to observe?. 4; TN with NTN satellite; NTN UL; TN UL; NTN cell: Nadir point. NTN UE: NTN UEs dropped at the edge of TN clusters; TN randomly placed in this NTN beam; Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the TN sectors which have NTN UE(s) at their edges. Option 1 is the baseline and it is not precluded companies can follow Option 2 to bring results. 5; TN with NTN satellite; NTN UL; TN DL; NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters; TN clusters randomly placed in this NTN beam; All active TN clusters which has the NTN UE(s) at its edge.

In this deployment, we assumed there’re 3 TN UEs in each TN sector and total 9 NTN UEs in the system. According to agreement in WF [3], we assume the isolation is 1.5 km for scenario 4 shown in Figure 1(Urban for example). NTN UEs placed at the TN cluster edge; those are at the radius of inter-site distance (ISD)*2 metres (isolation) from the TN cluster edge. The same isolation distance (1.5 km) can be reused for scenario 5.

Figure 1: Urban Macro for NR-NTN deployment for NTN UE and TN UE (scenario 4, isolation=1.5km)

To observe more potential impact from NTN UL to TN UL, we assume the isolation is starting from 0m to 1.5 km for scenario 4 shown in Figure 2. The simulation result could be found in table 9 and table 10.

Figure 2: Urban Macro for NR-NTN deployment for NTN UE and TN UE (scenario 4, isolation=0~1.5km)

System parameters

According to the following agreement in WF [3], we list the parameters in system level simulation shown in Table 2.

Agreement - Consider 20MHz system bandwidth for NR-NTN and NR in co-existence study. Agreement: To use Set-1 as NTN SAN parameters. Agreement - Assuming all NR-NTN/IoT-NTN UEs are HPUE, i.e. 100% HPUE ratio as baseline. - FFS other ratio values Agreement - Baseline: Use NF as 9dB, and Tx power 26/29/31 dBm for PC2/1.5/1 NTN HPUE - FFS option of 7dB NF for PC2 and LEO600. Agreement - Consider 9 UEs for 15kHz, 18 and 36UEs for 3.75kHz single tone as starting point - FFS whether to down scope. Agreement - To update the existing uplink power control parameters considering different power classes and bandwidth (Samsung, Qualcomm, Huawei) - Update Pmax to 26/29/31 for PC2/1.5/1; - Update Rmin to -66/-69/-71 assuming Pmin as -40; - Update BW to corresponding transmission BW. - Agree on the following detailed parameters where: - Pmax = 26dBm for PC2, 29 dBm for PC1.5, 31 dBm for PC1. - Rmin = -66 dB for PC2, -69 dB for PC1.5, -71 dB, assuming -40dBm minimum output power, - CLx-ile and γ are set as following: - CLx-ile = 10*log10(Pmax) – (SNRtarget + 10*log10(kTB) + NF ), - Where: - SNRtarget and BW For NR-NTN HPUE: - Option 1: 3 dB SNR target and 5MHz UL BW (One company proposed for LEO only) - Option 2: 15 dB SNR target and 2RB UL BW as baseline assumption - NF is the SAN noise figure, i.e. 4.3dB. - 10log10(kT) = -174dBm/Hz. - γ = 1 For uplink scenario. Agreement - All power classes listed in WID will be considered for co-existence study.

Table 2: System level simulation parameters for NTN HPUE co-existence study

| Scenarios | DL bandwidth | TN UL bandwidth | NR-NTN UL bandwidth | IoT-NTN UL bandwidth | RX NF | NR-NTN Power control | NTN UE PC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Scenarios 4 | N/A | 0.18MHz | 0.36MHz (2RB) | 0.015MHz | 5 dB | Follow above agreement. (Option 2) | PC2 PC1.5 PC1 |
| Scenarios 5 | 20MHz | N/A | 0.36MHz (2RB) | 0.015MHz | 9 dB | Follow above agreement. (Option 2) | RC2 PC1.5 PC1 |

Simulation results

The key performance metric as highlighted in TR 38.863 [1] is the throughput loss ratio, thus simulation results in this document will focus on this.

Throughput Loss Ratio vs ACIR

The throughput loss ratio of each scenario is determined across different ACIR values to determine the minimum required ACIR of the victim system to prevent impact from interference.

Table 3: ACIR requirements for NR-NTN (scenario 4, GEO)

|  | BS Antenna type | ACIR Requirement @ 5%-tile Throughput Loss Ratio |  |  |
| --- | --- | --- | --- | --- |
| Satellite |  | GEO |  |  |
| Aggressor |  | NR NTN UL |  |  |
| Victim |  | TN UL (FDD) |  |  |
| PC level |  | PC1 (31 dBm) | PC1.5 (29 dBm) | PC2 (26 dBm) |
| Urban | Non-AAS | 18.24 dB | 16.24 dB | 13.49 dB |
|  | AAS | 24.88 dB | 22.88 dB | 19.77 dB |

Table 4: ACIR requirements for NR-NTN (scenario 4, LEO 1200)

|  | BS Antenna type | ACIR Requirement @ 5%-tile Throughput Loss Ratio |  |  |
| --- | --- | --- | --- | --- |
| Satellite |  | LEO 1200 |  |  |
| Aggressor |  | NR NTN UL |  |  |
| Victim |  | TN UL (FDD) |  |  |
| PC level |  | PC1 (31 dBm) | PC1.5 (29 dBm) | PC2 (26 dBm) |
| Urban | Non-AAS | 18.11 dB | 16.24 dB | 13.49 dB |
|  | AAS | 24.8 dB | 22.88 dB | 19.77 dB |

Table 5: ACIR requirements for IoT-NTN (scenario 4, GEO)

|  | BS Antenna type | ACIR Requirement @ 5%-tile Throughput Loss Ratio |  |  |
| --- | --- | --- | --- | --- |
| Satellite |  | GEO |  |  |
| Aggressor |  | NR NTN UL |  |  |
| Victim |  | TN UL (FDD) |  |  |
| PC level |  | PC1 (31 dBm) | PC1.5 (29 dBm) | PC2 (26 dBm) |
| Urban | Non-AAS | 3.89 dB | 3.89 dB | 3.89 dB |

Table 6: ACIR requirements for IoT-NTN (scenario 4, LEO 1200)

|  | BS Antenna type | ACIR Requirement @ 5%-tile Throughput Loss Ratio |  |  |
| --- | --- | --- | --- | --- |
| Satellite |  | LEO 1200 |  |  |
| Aggressor |  | NR NTN UL |  |  |
| Victim |  | TN UL (FDD) |  |  |
| PC level |  | PC1 (31 dBm) | PC1.5 (29 dBm) | PC2 (26 dBm) |
| Urban | Non-AAS | 0 dB | 0 dB | 0 dB |

Table 7: ACIR requirements for NR-NTN (scenario 5)

|  | BS Antenna type | ACIR Requirement @ 5%-tile Throughput Loss Ratio |  |  |
| --- | --- | --- | --- | --- |
| Satellite |  | GEO |  |  |
| Aggressor |  | NR NTN UL |  |  |
| Victim |  | TN DL (TDD) |  |  |
| PC level |  | PC1 (31 dBm) | PC1.5 (29 dBm) | PC2 (26 dBm) |
| Urban | Non-AAS | 0 dB | 0 dB | 0 dB |
|  | AAS | 0 dB | 0 dB | 0 dB |

Table 8: ACIR requirements for IoT-NTN (scenario 5)

|  | BS Antenna type | ACIR Requirement @ 5%-tile Throughput Loss Ratio |  |  |
| --- | --- | --- | --- | --- |
| Satellite |  | GEO |  |  |
| Aggressor |  | NR NTN UL |  |  |
| Victim |  | TN UL (FDD) |  |  |
| PC level |  | PC1 (31 dBm) | PC1.5 (29 dBm) | PC2 (26 dBm) |
| Urban | Non-AAS | 0 dB | 0 dB | 0 dB |

The above simulation results are based on isolation=1.5km, all the ACIR is less than 25dB. Even for the most stringent case (Scenario 4, GEO, PC1), the ACIR (24.88 dB) is near to 25dB. The ACIR requirement is determined based on the 5%-tile throughput loss which should be less than 5%. Plots are given in Annex 1, with TN as the victim.

Based on the results in above tables, the following observations can be made:

- No tighten ACIR requirement since NTN UL interference to TN UL with a required ACIR < 25dB for NTN HPUE (PC1, PC1.5, PC2).

- No tighten ACIR requirement since NTN UL interference to TN DL with a required ACIR = 0dB for NTN HPUE (PC1, PC1.5, PC2).

Observation 1: Based on the simulation results in tables:

- No tighten ACIR requirement since NTN UL interference to TN UL with a required ACIR < 25dB for NTN HPUE (PC1, PC1.5, PC2).

- No tighten ACIR requirement since NTN UL interference to TN DL with a required ACIR = 0dB for NTN HPUE (PC1, PC1.5, PC2).

Proposal 1: Based on the simulation results from table 2 to table 8, there is no interference impact or tighten ACIR requirement from NTN UL to TN UL or TN DL for both NR-NTN and IoT-NTN HPUE (PC 2, PC1.5, PC1).

The following table shows the simulation results on isolation = 0 and compare the ACIR with isolation (1.5 km) and without isolation. For both NR-NTN and IoT-NTN, the ACIR will be very stringent (NR-NTN: 38.56 dB, IoT-NTN:32.16 dB) based on isolation distance are zero. There’s 14 dB difference (38.56-24.88) when compared the worst case between isolation(=0) and isolation(=1.5km) in NR-NTN.

Proposal 2: Based on the simulation results from table 9 to table 10, zero isolation between NTN and TN systems operation would introduce the higher ACLR values than existing TN HPUE ACLR values, which is impractical for UE implementation.  To capture zero-isolation evaluation results in TR but do not use zero-isolation results to specify final NTN HPUE RF ACLR requirements.

Table 9: ACIR requirements for NR-NTN (scenario 4, GEO)

|  | BS Antenna type | ACIR Requirement @ 5%-tile Throughput Loss Ratio |  |  |
| --- | --- | --- | --- | --- |
| Satellite |  | GEO |  |  |
| Aggressor |  | NR NTN UL |  |  |
| Victim |  | TN UL (FDD) |  |  |
| PC level |  | PC1 (31 dBm) | PC1.5 (29 dBm) | PC2 (26 dBm) |
| Urban Isolation = 0 | Non-AAS | 37.79 dB | 35.79 dB | 33.11 dB |
|  | AAS | 38.56 dB | 36.56 dB | 33.46 dB |
| Urban Isolation = 1.5km | Non-AAS | 18.24 dB | 16.24 dB | 13.49 dB |
|  | AAS | 24.88 dB | 22.88 dB | 19.77 dB |

Table 10: ACIR requirements for IoT-NTN (scenario 4, GEO)

|  | BS Antenna type | ACIR Requirement @ 5%-tile Throughput Loss Ratio |  |  |
| --- | --- | --- | --- | --- |
| Satellite |  | GEO |  |  |
| Aggressor |  | NR NTN UL |  |  |
| Victim |  | TN UL (FDD) |  |  |
| PC level |  | PC1 (31 dBm) | PC1.5 (29 dBm) | PC2 (26 dBm) |
| Urban Isolation = 0 | Non-AAS | 32.16 dB | 32.16 dB | 32.16 dB |
| Urban Isolation = 1.5km | Non-AAS | 3.89 dB | 3.89 dB | 3.89 dB |

Annex – 1 Throughput Loss Ratio vs ACIR with TN as victim network

Figure 3: TN UL Throughput loss ratio with nAAS @ GEO NR-NTN

Figure 4: TN UL Throughput loss ratio with AAS @ GEO NR-NTN

Figure 5: TN UL Throughput loss ratio with nAAS @ LEO1200 NR-NTN

Figure 6: TN UL Throughput loss ratio with AAS @ LEO1200 NR-NTN

Figure 7: TN UL Throughput loss ratio with nAAS @ GEO IoT-NTN

Figure 8: TN UL Throughput loss ratio with nAAS @ LEO1200 IoT-NTN

Figure 9: TN UL Throughput loss ratio with nAAS @ GEO NR-NTN, isolation=0

Reference

[1] 3GPP TR 38.863 Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

[2] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112