3GPP TSG-RAN WG4 Meeting #113									                        R4- 2419549

Orlando, USA, Nov. 18 – 22, 2024

Agenda item:			7.10.2.3

Source:	Ericsson

Title:	Discussion on RF requirement of ATG UE UL MIMO

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, and the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, some agreements were made addressing the general concerns [2]. Part of the agreements regarding the UL MIMO are shown below:

Issue 3-1-1: ULFPTx mode for UL-MIMO

WF:

Consider whether and how to introduce this requirement.

- Option 1: ULFPTx mode 1/2/Full in the following table can be supported depending on UE PA implementation

- Table 2: Example of ATG ULFPTx (R-MOP = x dBm)

| ATG ULFPTx mode | PA1(dBm) | PA2(dBm) |
| --- | --- | --- |
| Mode2 | x | (x-3)~(x-1) |
| Mode-Full | x | x |
| Mode1 | (x-3)~(x-1) | (x-3)~(x-1) |

- Option 2: Current ULFPTx mode requirements for legacy UE in TS38.101-1 can be reused for ATG UE, and there is no other specification affect.

- Option 3: No need to extend ULFPTx mode to ATG UE.

Issue 3-1-2: whether preclude the single antenna port related requirement

WF:

- Option 1: Yes.

- Option 2: No.

Issue 3-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18

Agreement:

- If the legacy requirement for UL MIMO is defined at each antenna connector,

  - For ATG UE with antenna array, the requirement should be updated at each TAB connector

- If the legacy requirement for UL MIMO is defined per layer

  - for ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer

Issue 3-2-2: whether to apply coherent UL MIMO requirement for ATG UE

Agreement:

Since coherent UL MIMO is the optional requirement, it could be supported without specification impact.

Issue 3-3-1: How to test Rx requirements

Agreement:

No need to test the Rx requirements under the UL MIMO scenario for ATG UE, since general Rx requirements have been tested.

Issue 3-3-2: How to test Tx requirements

Agreement:

Further discuss the method of definition for general ATG Tx requirements referring to agreed CR R4-2414321 can be used.

Issue 3-3-3: Transmit modulation quality for ATG

Agreement:

### 6.4J.2 Transmit modulation quality for ATG The requirements for transmit modulation quality defined in clause 6.4.2 shall apply for ATG UE except for the phase continuity requirements for DMRS bundling in 6.4.2.5. And the requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM. For ATG UE, the requirements for Error Vector Magnitude, Carrier leakage and EVM equalizer spectrum flatness are defined per layer. The requirements for In-band emission are defined at each transmit antenna connector or each TAB connector.

There are still many open issues that need to be addressed. The following session will discuss the RF requirement for ATG UE UL MIMO.

## RF requirements for ATG UE UL MIMO

### ULFPTx mode for UL-MIMO

For ATG UE UL-MIMO, in the case of one antenna port being configured (single layer UL MIMO operation) but with two Tx antenna connectors, the uplink full power transmission (ULFPTx) mode is applicable. From Rel-16 onwards, with the support of ULFPTx mode, one can transmit at full power either by combining two connectors using a coherent precoder or by supporting a full-power PA in full power mode. This ensures that UE can transmit at full power, i.e., if the UE is configured with only a single antenna port, it should always transmit at full power.

An example of ATG UE ULFPTx mode was proposed in [3], which could be adopted into the specification.

Table 2.1: Example of ATG ULFPTx (R-MOP = 26dBm)

| ATG ULFPTx mode | PA1(dBm) | PA2(dBm) |
| --- | --- | --- |
| Mode2 | 26 | 23~25 |
| Mode-Full | 26 | 26 |
| Mode1 | 23~25 | 23~25 |

- ULFPTx mode should be supported for ATG UL-MIMO

### Single antenna port related requirement for ATG UE

Issue 3-1-2: whether preclude the single antenna port related requirement

Regardless of the conditions of the propagation channel, as stated in the previous session, The ATG UE UL-MIMO is possible to be scheduled for a single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission with precoding matrix W=1. Therefore, from the completeness perspective, the single antenna requirement still needs to be specified for ATG UE UL MIMO.

- Apply single antenna port-related requirements for ATG UE.

# Conclusion

In the previous sections, we made the following observations:

Based on the discussion in the previous sections, we propose the following:

Proposal 1	ULFPTx mode should be supported for ATG UL-MIMO

Proposal 2	Apply single antenna port-related requirements for ATG UE.

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2417121, WF on Rel-19 ATG UE RF requirements

- R4-2415897, ATG UE RF requirements for UL MIMO, LG Electronics.