3GPP TSG-RAN WG4 Meeting #113		 			                        R4-2419556
Orlando, USA, 18th – 22nd November, 2024
 
Agenda Item: 7.9.2

Source:	SKY Perfect JSAT

Title: Discussion on VSAT UE parameter for Ku-band co-existence study

Document for:	Discussion and Decision

- Introduction

At RAN4#112 and #112bis, parameters for the Ku-band co-existence study were discussed. Additionally, as SKY Perfect JSAT examined the RF requirements for the UE, we proposed that it is appropriate not to specify a maximum EIRP and TRP value as the values will vary depending on use cases and subject satellites [1], and we also assessed the required maximum EIRP for providing 5G NR services using Ku-band GEO satellites [2]. In this contribution, based on our analysis of the necessary maximum EIRP value, we propose UE parameter values to be used for the Ku-band co-existence study related to maximum EIRP.

- Analysis and Discussion

Analysis of required EIRP for Ku-band UE

SKY Perfect JSAT calculated link budget and estimate the link quality and the achievable throughput in case 5G NR waveform carrier is transmitted from Ku-band VSAT UE to GSO Software Defined Satellite and legacy GSO satellite. The details are provided in the contribution R4-2419558[2], but as a result of the link budget analysis, it has been determined that the maximum EIRP of 88.2 dBm is required for Ku-band VSAT UE.

Observation 1: A maximum EIRP of 88.2 dBm is required for Ku-band VSAT UE.

Status of parameter consideration for Ku-band co-existence study

For the point of co-existence study, Ku-band spectrum allocation and realistic interference scenarios was already posted[3];

2.2 Realistic Interference Scenarios With reference to Figure 4, we can observe that: •There is an existing radioastronomy, a scientific, and a radar bands adjacent to and within the Ku Band. •There are no allocated terrestrial band in the vicinity of the Ku Band and as such there will not be any terrestrial services in the foreseeable future around the Ku Band. •The only area that we can perform Ku Uplink Band co-existence study would be between 13.75-14.50 GHz and the IMT Services between 14.80-15.35 GHz to be studied 2027 (this band is currently under study and no services have yet been assigned). •The only area that we can perform Ku Downlink Band co-existence study would be in Region 2 and it would be between 10.7-12.7 GHz and the IMT Services between 10.0-10.5 GHz (assigned). •All current radioastronomy, scientific and military services to be protected. This follows that all scientific services adjacent to the planned IMT services between 10.0-10.5 GHz must be protected.

At RAN4 #112, the following agreement was reached regarding TN frequencies in the co-existence study[4].

Issue 2-10: TN Parameters As starting point, reuse the antenna parameters and BS/UE ACLR/ACS from: · The 10.0-10.5 GHz frequency range (TR 38.921) for the reference frequency of 11 GHz The 14.8-15.35 GHz frequency range (pending on conclusion of SI FS_NR_IMT_4400_7125_14800MHz) for the reference frequency of 14 GHz.

The co-existence study impacted by the UE's EIRP focuses on interference from the uplink NTN (reference frequency 14 GHz, actual allocation 14.0 (13.75)-14.5 GHz) to the TN operating in the 14.8-15.35 GHz range. There is a 300 MHz separation between the NTN and TN frequencies.

Observation 2: The co-existence study impacted by the UE's EIRP focuses on interference from the uplink NTN (reference frequency 14 GHz, actual allocation 14.0 (13.75)-14.5 GHz) to the TN operating in the 14.8-15.35 GHz range. There is a 300 MHz separation between the NTN and TN frequencies.

Additionally, at RAN#112bis, the discussions towards the co-existence study was held based on the following UE parameters:

- Sub-topic 2-9: VSAT equivalent antenna aperture Use 60 cm - Sub-topic 2-11: VSAT max Tx antenna gain Use 36.6 dBi (option 1 2W), 36.2 dBi (option 2 10W) - Sub-topic 2-16: VSAT max Tx power 40 dBm (10 W)

Calculating the VSAT maximum EIRP from the above parameters for the co-existence study, including the VSAT max Tx antenna gain and the VSAT max Tx power, results in 76.2 dBm.

UE parameters used in the co-existence study

Based on the results of the link budget analysis assuming Ku-band GEO legacy satellites and SDS satellites, we believe that a maximum EIRP of 88.2 dBm is required for the NTN VSAT UE. The UE's EIRP value will only affect to the co-existence study between the uplink NTN in the 14.0 (13.75)-14.5 GHz range and the TN in the 14.8-15.35 GHz range. Since there is a 300 MHz separation between the NTN and TN frequencies, it is expected that increasing the EIRP to 88.2 dBm would still be within a feasible level. Therefore, for the Ku-band co-existence study, the VSAT parameters should be set such that the maximum EIRP is 88.2 dBm, with the max Tx power set at 52 dBm.

Proposal 1: For the Ku-band co-existence study, the VSAT parameters should be set such that the maximum EIRP is 88.2 dBm, with the max Tx power set at 52 dBm.

- Conclusions

Observation 1: A maximum EIRP of 88.2 dBm is required for Ku-band VSAT UE.

Observation 2: The co-existence study impacted by the UE's EIRP focuses on interference from the uplink NTN (reference frequency 14 GHz, actual allocation 14.0 (13.75)-14.5 GHz) to the TN operating in the 14.8-15.35 GHz range. There is a 300 MHz separation between the NTN and TN frequencies.

Proposal 1: For the Ku-band co-existence study, the VSAT parameters should be set such that the maximum EIRP is 88.2 dBm, with the max Tx power set at 52 dBm.

- References

- R4-2415679, Ku-band VSAT UE RF requirement analysis; SKY Perfect JSAT, RAN#112bis

- R4-2419558, Ku band Maximum TRP and EIRP Analysis; SKY Perfect JSAT, RAN#113

- R4-2411507, Ku Band Co-existence: Intelsat, RAN#112

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General; Moderator (Eutelsat Group), RAN4#112