3GPP TSG-RAN Working Group 4 (Radio) Meeting #113	R4-2519587

# Orlando, USA, 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | <CR#> | rev | <Rev#> | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.141-2: Addition of spatial emission requirement to protect FSS UL within band n104 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Perf |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To capture the new requirement for band n104 to protect co-channel FSS UL service in conformance specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New clause 6.9 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the new requirement will not be captured in BS RF core specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | There are several other clause that need modifications in case this CR is endorsed. E.g. 4.8 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.9	OTA spatial emission

### 6.9.1	General

### 6.9.2 Protection of FSS UL

#### 6.9.2. 1	Definition and applicability

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions.

#### 6.9.2 2	Minimum requirement

The minimum requirement for BS type 1-O and BS type 1-H is in TS 38.104 [2], subclause 9.9.2.

#### 6.9.2.3	Test purpose

The test purpose is to verify that radiated power above the horizon does not exceed specified EEIRP limits.

#### 6.9.2.4	Method of test

##### 6.9.2.4.1	Initial conditions

The test environment: Normal, see TS 38.141-2, Annex B.2.

RF channels to be tested for single carrier: B and T; see TS 38.141-2, clause 4.9.1. It is sufficient to test only one carrier, out of the ones characterised by the highest supported output power spectral sensity, expressed in dBm/MHz .

Beams to be tested: For each declared CAR determine the set of test beams, (red points in example below) as follows:

- 4 points on the extremes of the largest elipse/circle inside the declared CAR region

- 7 points on each diagonal, equally distanced, but exclude the extremes of the CAR region.

Figure 6.9.2.4.1-1: Example of approximate test beam position in the declared CAR region (green contour)

The test object shall be placed on the positioner with mechanical tilt set to 0 degrees.

Since this requirement is targeting the power transmitted above the horizon for a specific site installation, it is required that the BS is mounted on the positioner such that blocking effects due to the positioner are minimized.

##### 6.9.2.4.2	Procedure

- Calibrate the test range, according to calibration described in TR 37.941, subclause 8.3.

- Place the test object on the positioner, such that blocking effect in the vertical domain shall be minimized.

- Align the manufacturer declared coordinate system orientation (D.2) of the test object with the coordinate system used by the test system.

- Configure the test object according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1, subclause 4.9.2.

- Set the positioner at location for measurement point (θn,φm) for m=1..M and n=1..N.

- Enable test beam  k for k=1..K.

- Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

- Repeat step 6 and 7 for all K test beams.

- Repeat step 5, 6, 7 and 8 for all M and N positioner locations.

- Calculate EEIRPi profile from measured EIRPkθn,φm patterns for i=1..7 as described in Annex XX.

#### 6.9.2.5	Test requirement

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6 425 to 7 075 MHz, shall not exceed the limits specified in table 6.9.2.5-1 for any declared coverage angular ranges (CAR).

Table 6.9.2.5-1: EEIRP limits as function of elevation above horizon

| Elevation angular range θL ≤ θ < θH (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- |
| 0<<5 | 27 |
| 5<<10 | 23 |
| 10<<15 | 19 |
| 15<<20 | 18 |
| 20<<30 | 16 |
| 30<<60 | 15 |
| 60<<90 | 15 |
| NOTE: The requirement shall apply to all supported mechanical tilts. |  |