# 3GPP TSG-RAN WG4 Meeting #113	R4-2419619

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.20.3.2

Source: 	Samsung

Title: 	Views on RRM requirements for Rel-19 SBFD operation

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 further discussed the potentail impact to RRM requirements with SBFD operation as agreements captured in the WF [2].

From RAN4 RRM requiremements perstive, follow SSB based and or CSI-RS based measurement requirements are specified as summarized in below table:

Table 1: SSB and or CSI-RS related measurement in TS 38.133

| Serving cell RRM measurement |  |  | Neighbouring cell measurement |
| --- | --- | --- | --- |
| L3 RRM measurement | L1-BM (L1-RSRP, L1-SINR) with in CSI framework | L1 RLM L1 BFD/BFR | L3 RRM measurement |

In this contribution, further analysis and proposals were provided for RRM measurement impact with SBFD operation.

2 RRM measurement impact for SBFD operation

2.1 SSB based RRM measurement.

Following agreements were reached in previous RAN1/RAN4 meeting:

RAN1 agreement:

RAN1#118bis Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

RAN4 agreement

RAN4#112bis #### Issue 1-2-1: Requirements for SSB based measurement Agreements (from online session) - Down-select one of the two options: - Option 1: - As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. - This can be revisited if it proves to be conflicting with further RAN1 agreements. - Option 2: - For inter-frequency measurement without gap, UL transmission is prioritized. - Follow option 1 in other cases.

RAN1 specified collision handling rule that serving cell SSB will be prioritized over than UL transmission in SBFD slots as existing collision rules. And RAN1 also agreed that SSB frequency occupation must be “self-contained” within an SBFD DL subband, i.e., SSB cannot overlap with DL and UL subband boundary.

For SSB based on neighbouring cell measurement, we believe this belongs to RAN4 scope. In legacy RAN4 RRM requirements, SSB based on measurement is always prioritized over than UL/DL transmission given RRM mobility performance shall be maintained and treated with high priority.

Legacy rules for neighbouring cell measurement can be applied i.e., L3 neighbouring cell RRM measurement shall be prioritized over than UL transmission.

For CSI-RS based on L3 measurement, similar scheduling availability rule is specified in existing specifications.

Proposal 1: Existing scheduling availability rule for intra-frequency/inter-frequency measurement can be applied for SBFD slots/symbols i.e.  SSB based L3 measurement is prioritized over configured uplink transmission.

One open issue, in SBFD slots/symbols when SSB/CSI-RS based measurement collided with scheduled DL transmission in different DL sub-bands, any restriction on scheduling availability rule or measurement restriction needs to be specified or not?

Following figures are descripted 3 cases:

- Case 1: Neighbour cell SSB is completely contained in one of the active DL sub-band.

- Case 2:  Neighbour cell SSB is not partially contained in one of the active DL sub-bands.

- Case 3: Neighbour cell SSB is out of active DL sub-band(s)

For case 1, we believe no additional scheduling availability is required for SBFD-aware UE to receive scheduled DL PDCCH/PDSCH/TRS/CSI-RS for CQI.

For case 2, and case 3 when Neighbour cell SSB or CSI-RS is not completely contained in active DL sub-bands e.g. partially overlapped or totally within UL sub-band, the scheduling availability rule is required.

Figure 1: SSB configuration in neighbouring cell

Proposal 2: For inter-frequency measurement w/o gap, additional scheduling availability rule shall be introduced when neighbouring cell SSB/CSI-RS based L3 measurement is not completely contained in active DL sub-bands on SBFD symbols/slots i.e.

- UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on the configured CSI-RS symbols/SSB symbols for RRM mobility measurement during SBFD symbols

2.3 CSI-RS based RRM measurement.

As discussed in last RAN4 meeting, there are 3 items in RAN1 which may have impact to RAN4 CSI-RS based measurement.

RAN4#112bis agreement: Issue 1-2-2: Requirements for CSI-RS based measurement · Agreements (from online session) o RAN4 to discuss whether and how L1 CSI-RS measurement requirements would be impacted by the following SBFD operations. - CSI-RS colliding with dynamically scheduled UL transmission - CSI-RS puncturing in frequency domain in SBFD symbols - CSI-RS distributed over SBFD and non-SBFD symbols o Align the understanding whether above SBFD operations are applicable for CSI-RS for RLM, BFR and L1-RSRP, or they are only applicable for CSI-RS for CSI. o RAN4 to discuss whether and how L3 CSI-RS measurement requirements would be impacted by SBFD operations, and further RAN1/2 conclusions can be considered.

Background: RAN4 CSI-RS based RRM measurement including following scenarios/cases

| Serving cell CSI-RS based RRM measurement |  |  | Neighbouring cell measurement (CSI-RS based) |
| --- | --- | --- | --- |
| L3 RRM measurement | L1-BM (L1-RSRP, L1-SINR) with in CSI framework | L1 RLM L1 BFD/BFR | L3 RRM measurement |

RAN1 specified following enhancement related CSI-RS resources

- Collision rule: CSI-RS colliding with dynamic UL transmission

Following agreement reached in RAN1 on collision rule for case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission)

RAN1#116bis agreements: If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported - FFS on dynamically scheduled UL transmission with repetition

This is applied for all DL reception which following legacy rule for dynamic scheduled UL transmission in flexible symbols.

Following collision rules is specified in TS 38.213 section 11.1.1 for flexible symbols:

### 11.1.1 UE procedure for determining slot format ....... For unpaired spectrum operation for a UE on a cell in a frequency band of FR1, and when the scheduling restrictions due to RRM measurements [10, TS 38.133] are not applicable, if the UE detects a DCI format indicating to the UE to transmit in a set of symbols, the UE is not required to perform RRM measurements [10, TS 38.133] based on a SS/PBCH block or CSI-RS reception on a different cell in the frequency band if the SS/PBCH block or CSI-RS reception includes at least one symbol from the set of symbols.

Above existing rules specified in RAN1 is only applied for neighbouring cell measurement under the condition that the scheduling restrictions for RRM measurements are not applicable.

In RAN4 RRM specification, scheduling restrictions are always applicable for CSI-RS based L3 RRM measurement (inter-frequency measurement with gap and intra-frequency measurement w/o gap) i.e., L3 RRM measurement is always prioritized over than UL transmission.

Following same rule, we shall avoid the impact to existing L3 RRM requirements and RRM mobility performance as much as possible under SBFD operation.

For L1 CSI-RS related measurement (L1-BM, RLM and BFR/BFD), above collision rules shall be applied.

Proposal 3: For CSI-RS collision with dynamic UL transmission:

- For L3 RRM CSI-RS measurement, reusing existing rules (scheduling restriction for intra-frequency measurement, and measurement gap for inter-frequency measurement) that L3 RRM measurement prioritized over than UL transmission.

- For L1 CSI-RS based on measurement (L1-BM, RLM, BFD), dynamic UL transmission shall be prioritized over than L1 CSI-RS based on measurement

  - RAN4 need to further discuss how to update the requirements with extended measurement period to collect sufficient valid measurement samples

- CSI-RS resource puncturing on SBFD symbols

Following agreements in RAN1 for CSI-RS resource punctured across DL sub-bands in SBFD slots:

RAN1#117 Working assumption: For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands

For this case, we need to discriminate CSI-RS from serving cell and CSI-RS from neighbour cell.  On serving cell case, we can follow above WA from RAN1.

For neighbour cell case, UE has no information on neighbour cell SBFD configurations. For CSI-RS based L3 inter-frequency measurement, measurement gap is always configured and neighbour cell operated on different frequency than serving cell, UE need to follow measurement gap to process measurement under dedicated time window. On this case, we can follow legacy UE behaviour and requirements which is transparent to serving SBFD configuration.

For serving cell CSI_RS measurement: SBFD aware-UE shall follow DL sub-band configuration during SBFD symbols. UE only measurement CSI-RS with DL sub-bands in SBFD symbols.

On measurement BW issue, RAN4 requirements are only applicable when measurement BW >=48 PRBs as specified in existing specification and in previous release we also have CHBW < 48 PRBs cases, and RAN4 requirements are not applicable.  For SBFD operation, we believe BWs for downlink sub-bands shall be larger than 48 PRBs under typical deployment scenario.

Proposal 4: For CSI-RS resource puncturing on SBFD symbols

- For L1 CSI-RS measurement (L-BM, RLM, BFR/BFD) and L3 intra-frequency measurement w/o gap on SBFD symbols, SBFD-aware UE follow DL sub-bands configuration

- For neighbouring cell L3 inter-frequency CSI-RS measurement with gap, UE follow measurement gap and legacy CSI-RS configuration.

- CSI-RS distributed over SBFD and non-SBFD symbols

RAN1 specified two configurations for DL reception and UL transmission as following:

RAN1#117 agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. · Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only · Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols · FFS granularity of the configuration, e.g. per UE, per channel/signal etc. FFS whether support of configuration 2 is subject to UE capability Agreement: #118b For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with Configuration 1 or Configurat 2 via RRC configuration. Select from the following options - Option 1: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis - Option 2: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis - Separate configurations for configured and dynamic transmission/receptions for a given channel/signal are not precluded. - Option 3: Configuration 1 or Configuration 2 is on a per cell basis. - No separate UE capabilities for Configuration 1 and 2. Agreement #118b For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

SBFD-aware UE is provided with Configuration 1 or Configuration 2 via RRC configuration for serving cell.  For CSI report, only single symbol type is used for CSI derivation.

Unlike CSI report which can be considered as single shot measurement. For L1 based RRM measurement, multiple samples measurement is assumed except (L1-RSRP/SINR measurement which can be configured with single shot measurement). Regarding L1 CSI_RS based measurement for RLM, BFR/BFD and L1-BM, it’s unclear whether above configuration 1 and 2 are applicable or not.

If configuration 2 applicable, then RAN4 need to further discuss whether UE need to combine the measurement across SBFD slots and non-SBFD slots or drop/skip part of measurement samples.

Proposal 5: For CSI-RS distributed over SBFD and non-SBFD symbols

- For L1 CSI-RS measurement (L-BM, RLM, BFR/BFD) and L3 intra-frequency measurement w/o gap, FFS wait for further progress from RAN1/RAN2

- For neighbouring cell L3 inter-frequency CSI-RS measurement with gap, no impact on existing requirements

- Conclusion

This contribution provides further analysis on RRM measurement impact with SBFD operation.

SSB base measurement

Proposal 1: Existing scheduling availability rule for intra-frequency/inter-frequency measurement can be applied for SBFD slots/symbols i.e.  SSB based L3 measurement is prioritized over configured uplink transmission.

Proposal 2: For inter-frequency measurement w/o gap, additional scheduling availability rule shall be introduced when neighbouring cell SSB/CSI-RS based L3 measurement is not completely contained in active DL sub-bands on SBFD symbols/slots i.e.

- UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on the configured CSI-RS symbols/SSB symbols for RRM mobility measurement during SBFD symbols

CSI-RS based measurement

Proposal 3: For CSI-RS collision with dynamic UL transmission:

- For L3 RRM CSI-RS measurement, reusing existing rules (scheduling restriction for intra-frequency measurement, and measurement gap for inter-frequency measurement) that L3 RRM measurement prioritized over than UL transmission.

- For L1 CSI-RS based on measurement (L1-BM, RLM, BFD), dynamic UL transmission shall be prioritized over than L1 CSI-RS based on measurement

  - RAN4 need to further discuss how to update the requirements with extended measurement period to collect sufficient valid measurement samples

Proposal 4: For CSI-RS resource puncturing on SBFD symbols

- For L1 CSI-RS measurement (L-BM, RLM, BFR/BFD) and L3 intra-frequency measurement w/o gap on SBFD symbols, SBFD-aware UE follow DL sub-bands configuration

- For neighbouring cell L3 inter-frequency CSI-RS measurement with gap, UE follow measurement gap and legacy CSI-RS configuration.

Proposal 5: For CSI-RS distributed over SBFD and non-SBFD symbols

- For L1 CSI-RS measurement (L-BM, RLM, BFR/BFD) and L3 intra-frequency measurement w/o gap, FFS wait for further progress from RAN1/RAN2

- For neighbouring cell L3 inter-frequency CSI-RS measurement with gap, no impact on existing requirements

| SBFD operation | Serving cell CSI-RS based RRM measurement |  |  | Neighbor CSI-RS based L3 RRM measurement |  |
| --- | --- | --- | --- | --- | --- |
|  | L3 RRM measurement | L1-BM (L1-RSRP, L1-SINR) with in CSI framework | L1 RLM L1 BFD/BFR | Intra-frequency w/o gap | Inter-frequency with gap |
| Collision rule: conflicting with dynamic UL transmission | Existing scheduling restriction applied No requirement update | UL transmission prioritized DL measurement dropped; Requirement update needed |  | Existing scheduling restriction applied No requirement update | Measurement gap prioritized No requirement update |
| CSI-RS resource puncturing on SBFD symbols | UE only measurement CSI-RS in DL sub-bands Update RAN4 spec | UE only measurement CSI-RS in DL sub-bands Update RAN4 spec |  | UE only measurement CSI-RS in DL sub-bands Update RAN4 spec | Follow legacy UE behavior No requirement update |
| CSI-RS distributed over SBFD and non-SBFD symbols | FFS pending on RAN1 further conclusion? | FFS pending on RAN1 decision which configuration applicable to UE? |  | Following serving cell L3 measurement conclusion | Follow legacy UE behavior No requirement update |

Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, HiSilicon

[2] R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon