# 3GPP TSG-RAN WG4 Meeting #113	R4-2419620

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.20.3.1

Source: 	Samsung

Title: 	Initial L1 CLI measurement simulation results

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 further discussed L1 CLI measurememt impact and simulation assumption for  L1-SRS-RSRP measurement was approved in [2].

- Simulation results

Initial simulation was provided in this section for AWGN channel under SNR 1 dB with 1 symbol, 48PRB SRS.

For Residual timing error, two cases were evaluated:

| Residual timing error (time offset between measurement timing at UE0 and arrival timing of SRR from UE 1) Note 2 | - Case 1: 4.67us for FR1, 3.67us for FR2 - Case 2: 1.67us for FR1, 0.67us for FR2 |
| --- | --- |

For measurement samples: {1 sample, 2, samples, 3 samples} were evaluated.

FR1-15kHz

| Figure-1 FR1 15kHz, SNR =1dB with TO =4.67us | Figure-2 FR1 15kHz, SNR =1dB with TO =1.67us |
| --- | --- |

FR1-30kHz

| Figure-3 FR1 30kHz, SNR =1dB with TO =4.67us | Figure-4 FR1 30kHz, SNR =1dB with TO =1.67us |
| --- | --- |

FR1-60kHz

| Figure-5 FR1 60kHz, SNR =1dB with TO =4.67us | Figure-6 FR1 60kHz, SNR =1dB with TO =1.67us |
| --- | --- |

FR2-60kHz

| Figure-7 FR2 60kHz, SNR =1dB with TO =3.67us | Figure-8 FR2 60kHz, SNR =1dB with TO =0.67us |
| --- | --- |

FR2-120kHz

| Figure-9 FR2 120kHz, SNR =1dB with TO =3.67us | Figure-10 FR2 120kHz, SNR =1dB with TO =0.67us |
| --- | --- |

Summary

Table 1 below summarized the absolute delta_RSRP w/o RF margin for target 90% accuracy:

Table 1: 90% absolute delta RSRP [dB]

| SCS/Channel | TO case 1 |  |  | TO case 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 sample | 2 samples | 3 samples | 1 sample | 2 samples | 3 samples |
| AWGN FR1-15kHz | 1.2 | 1.1 | 1.0 | 0.7 | 0.6 | 0.5 |
| AWGN FR1-30kHz | 2.1 | 1.9 | 1.7 | 1.0 | 0.8 | 0.8 |
| AWGN FR1-60kHz | 3.9 | 3.6 | 3.4 | 1.5 | 1.4 | 1.3 |
| AWGN FR2-60kHz | 3.1 | 2.7 | 2.6 | 0.9 | 0.7 | 0.6 |
| AWGN FR2-120kHz | 6.8 | 6.2 | 6.0 | 1.3 | 1.1 | 1.0 |

We have similar observations as Rel-16 L3 CLI-SRS-RSRP evaluation:

Observation 1: SRS-RSRP measurement accuracy is sensitive to residual timing error

- With residual timing error case 1 (R16 L3 CLI assumption), measurement accuracy will be degraded with numerologies (SCS) increased as ~1dB for 15kHz, ~2dB for 30kHz, ~3.5dB for FR1 60kHz, ~2.6 dB for FR1 60khz and ~6.0 dB for FR2 120kHz (with 3 samples).

- With residual timing error case 2 (w/o cell phase sync error), measurement accuracy improved a lot compared to case 1 especially under high numerologies as   0.7 dB for 15kHz, 1.0 dB for 30kHz, 1.5 dB for FR1-60kHz, 0.9dB for FR2-60kHz and 1.3 dB for FR2-120kHz.

Observation 2: With number of samples for measurement increased, the measurement accuracy improvement is limited with ~0.5dB improvement with 3 samples compared to 1 sample measurement and ~0.1dB improvement with 3 samples compared to 2 samples.

Figure 11: Absolute RSRP accuracy [dB] under AWGN with SNR =1dB, 48 PRB SRS BW

Above observations are also aligned with theoretical analysis as in below table we summarized the CP length with different numerologies.

Table 2: Normal CP length

| 15kHz | 4.7 us /5.2 us |
| --- | --- |
| 30khz | 2.35 us/2.6 us |
| 60kHz | 1.175 us/1.3 us |
| 120kHz | 0.5875 us/0.65us |

- With residual timing error case 1(legacy assumption with cell phase error), except 15kHz, timing error already exceed CP length, the ICI will be generated.

- With residual timing error case 2(no cell phase error with intra-cell measurement), except FR1-60kHz SCS, all other SCSs the timing error is smaller or close to CP length which can be tolerated by measurement performance.

Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, HiSilicon

[2] R4-2416910, Simulation assumptions for L1 SRS-RSRP measurement, Ericsson