3GPP TSG-RAN WG4 Meeting #113	R4-2419629

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	On the NGSO Satellite Motion Trajectory used in test cases

Agenda item:	7.8.4

Document for:	Discussion

Introduction

In RAN #101, RAN plenary has approved a simplification of the test framework for NGSO, amidst concerns of the time to complete the work item in RAN5 by the end of the Release 17. As a consequence, the specification work in RAN4 has concluded that the test cases for NGSO scenarios would be tested against a fixed (non-varying) propagation delay and fixed doppler delay.

As the speed of satellite in NGSO scenarios is a very relevant aspect of the NTN scenario that has not been covered by similar testing before and considering that NTN is a growing and relevant market, RAN has approved the WID RP-241985 [1] (NR_IoT_NTN_req_test_enh) to develop more meaningful and relevant test cases for this feature considering the motion trajectory of the satellite.

The following objectives were established for this WI:

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

The topic was initially discussed in RAN4 #112, and the discussion followed in RAN4 #113. The current list of open issues identified in this WI can be found in the WF captured in [2].

Discussion

In the present document, we will discuss some of the open issues presented in [2].

The first open issue is related to the methodology to generate the time-varying doppler shift and propagation delay modelling.

Issue 1-1-1 Methodology for Time varying Doppler Shift and propagation delay modelling - WF - RAN4 aims to introduce TE-emulated time varying doppler shift and propagation delay model matching a satellite motion with following option: - Satellite motion emulated using the two-body Keplerian propagator model, [i.e., spherical gravity model] - RAN4 further discuss how to derive the Doppler shift and propagation delay variation based on Keplerian model with the necessary parameters and details procedure - Companies are encouraged to provide the two-body Keplerian propagator model to be implemented in the TE, to emulate the Doppler shift and propagation delay, and to derive the ephemeris information.

For this discussion we propose the following steps to create a satellite orbit.:

- Choose a start location for the UE

- Based on the UE position, choose a starting point for the satellite according to the following rules:

  - The elevation angle between UE and the satellite equals to X degrees. Where

    - X= 30 (or other specific values, such as 90 degrees, might be chosen depending on test purpose and pre-existing test configurations)

  - Satellite altitude follows the agreed test configuration

- Based on the selected UE and satellite start positions, design a satellite orbit, using the Keplerian propagator model such that

  - UE is located in the satellite orbital plane

  - The satellite moves initially towards the UE when the test is initiated (unless stated otherwise, for example in the mobility test case)

- Update time and doppler conditions in the channel emulated TE based on current satellite and UE positions and velocities

Agree on the steps above to generate the UE position and satellite motion in the test setup.

The methodology proposed has the advantage to be flexible to different test setups while providing a reproductible test configuration.  The proposal above also addresses Issue 1-1-2-3 in [2]

Assumptions on Satellite and UE position (and velocity) configurations

There are the following options related to the position (altitude) of the satellite to be emulated and the position and velocity of the UE in the test setup:

Issue 1-1-2-1: Assumption of Satellite altitude - WF - Prioritize the LEO-600 orbit for TE emulated channel modelling - FFS on the LEO-1200 orbit for TE emulated channel modelling pending on the test purpose Issue 1-1-2-2: Assumption of UE motion & UE position - WF - Same UE location information should be set for both DUT and TE before the test start with the following initial position - Assuming UE is stationary during the demodulation and RF requirement testing - FFS on the assuming UE is in motion during the RRM requirement testing

First, in what regards the satellite altitude, we would like to highlight that there are some tests whose configuration consider a NGSO satellite with altitude of 1200 kms. In the old test setup, there are test configurations that include ephemeris information for satellites travelling at 1200 kms of altitude, such as indicated by TS 38.508-1:

5.6.2.0 Assumptions for Ephemeris generation This section provides the assumptions used to generate Ephemeris information for GSO and NGSO orbits: 1. Ephemeris files are generated for 3 types of satellites: GSO (inclination equal to 7º), LEO-600 and LEO-1200. 2. UE location is as defined in section 5.6.1. 3. PositionVelocity state vectors are used for EphemerisInfo signaling. 4. For GSO, an altitude of 35,786 km is considered. For NGSO, 2 different altitudes are considered: 600 km and 1200 km. 5. One-way delay between UE and satellite to be in the range of 119.375 ms to 128.79 ms for GSO and between 2 ms (lowest value for LEO orbit 600 km) and 6.67 ms (highest value for LEO orbit 1200 km) 6. Doppler (frequency) shift up to [0.93] ppm for GSO and up to 24ppm for NGSO (LEO-600).

There already some tests whose configuration shows satellite ephemeris utilizing a satellite altitude of 1200 kms.

Considering the above, it is necessary that the new TE-channel model enables the testing under the current pre-existing conditions, but do not add new configurations to be assessed in the different test setups.

The new TE-Model should enable all currently designed tests to be performed but do not introduce new test configurations that were not agreed in previous releases.

Some companies have demonstrated concerns that considering sallite altitude equal to 1200 kms would cause all test cases to be performed also for such altitude, but the goal of this WI is not to amend or create new test cases, but solely providing a working TE-Model with a good emulation of live operation conditions. Therefore, in order to tend to the concerns of these companies, but maintain the testability of all early agreed test cases we propose the following:

Both NGSO (LEO) 600km and NGSO (LEO) 1200km to be considered for this WI. The satellite configuration to be used in each individual performance test shall follow pre-existing conditions for altitude, and as a baseline, no new test cases introduced in this WID to cover for different satellite altitudes.

For the UE position there is currently an FFS on whether the UE shall be considered static or moving during the transmit timing test case.

It goes without saying that a test with a UE with updating position is more meaningful to the new mechanism for timing configuration introduced in NTN. As such, this should be the intended configuration for the performance evaluation of the UE. However, there might be reasonable concerns of feasibility or complexity of such test setup/TE channel model. Such change would also impact RAN5 work, as they are the more experienced group in the test setup where the UE is informed of its position and would understand better the constraints for position update.

Therefore, we propose:

Consider asking RAN5 whether it is feasible to implement/emulate UE motion exclusively during the transmit timing test case.

Assume UE in motion during the RRM transmit timing test case.

Other considerations

Update rate of Satellite ephemeris information.

The following way forward was agreed on the discussions related to the refresh rate of the satellite assistance information provided via SIB19 during the test case.

Issue 1-1-2-5: Assumption of updated granularity for Satellite ephemeris information - WF - Companies are encouraged to provide the view for RAN4#113 - Satellite ephemeris information will be updated at least every 10.24 seconds as the worst case according 38.331 - NOTE: other (smaller) value may be considered depending on the test purpose

It is very important to highlight that RRM test cases already included more tight specifications for the refresh rate than the one suggested as baseline above (10.24 s). For NGSO scenarios, current RRM definitions state that the test configurations are to be performed with an interval between adjacent epoch times equal to 2.56 s.

Also, 10.24 s is the maximum value expected for updates on the epoch time (cycle for repetition of SFN).

Therefore, in order to not create overlap or ambiguity with previous agreements, we suggest agreeing on the following:

Unless otherwise stated on existing test configurations, the TE is expected to refresh satellite ephemeris in SIB19 at least once every 10.24s.

On multiple satellite configurations

There are some mobility test cases for RRM that consider the deployment of two satellites. This is important because in NTN the UE behaviour (and consequently the requirements) change depending on whether the neighbor cell is associated with the same satellite as the current serving cell or to a different satellite.

If the new TE-Channel model is implemented and made generic for all test cases, including current and future mobility test cases, it will not be possible to test whether UE fullfil the requirements in a fair manner.

Some mobility test cases can only properly performed in the presence of at least two satellite configurations for the test case.

We understand that in the previous meeting some companies have considered this a risk of expanding the test cases, including demodulation test cases, to encompass the scenarios with two satellites, in order to avoid this risk, but maintain the testability of the mobility test cases we propose the following:

For RRM mobility test cases, when two satellites are considered in the test configuration, the two generated satellites might be in the same orbital plane.

Issue 1-1-2-7: Assumption of Multiple satellite scenario - WF - RAN4 focuses TE emulated channel modeling for one satellite FFS on the TE emulated channel modelling for multiple satellite scenario, considering the serving satellite changes

## Sub-topic 1-3 Inclusion in Specfication Issue 1-3-1 Specification of Channel Model - WF - RAN4 further discuss how to incorporate the channel model into specification pending on channel model discussion progress.

Inclusion in Specification

Following the discussion regarding how to define the channel model, one important aspect which should not be overlooked is how this channel model should be incorporated into the current 3GPP NTN specifications. Currently the NTN-TDL channels used within the performance specifications are contained within TR 38.811

Channel Models for NTN are currently contained within TR 38.811

However it is typical that a TR is stable once a release has been completed, and further we note that TR 38.811 is not listed in the effected documentation in the WID for Rel-19.

Currently it is not expected (within the WID) for TR 38.811 to be impacted.

Therefore, it could be considered whether the outcome of this work may have an impact to TR 38.811. Alternatively, the channel model itself could be included in the respective specification where the performance requirements are defined, as is the case for example with High Speed Train channels wit time varying doppler.

The varying doppler profiles could be captured in the appropriate performance specifications.

Thus, this leads to the following proposal.

RAN4 to incorporate the new TE-Channel model in TS 38.101-5 and reference it from RRM and demodulation specifications.

Conclusion

The paper has discussed the TE channel model for varying doppler and delay conditions under NGSO scenarios and the following set of observations and proposals were made:

Proposal 1: Agree on the steps above to generate the UE position and satellite motion in the test setup.

Observation 1: There already some tests whose configuration shows satellite ephemeris utilizing a satellite altitude of 1200 kms.

Observation 2: The new TE-Model should enable all currently designed tests to be performed but do not introduce new test configurations that were not agreed in previous releases.

Proposal 2: Both NGSO (LEO) 600km and NGSO (LEO) 1200km to be considered for this WI. The satellite configuration to be used in each individual performance test shall follow pre-existing conditions for altitude, and as a baseline, no new test cases introduced in thi WID to cover for different satellite altitudes.

Proposal 3: Consider asking RAN5 whether it is feasible to implement/emulate UE motion exclusively during the transmit timing test case.

Proposal 4: Assume UE in motion during the RRM transmit timing test case.

Proposal 5: Unless otherwise stated on existing test configurations, the TE is expected to refresh satellite ephemeris in SIB19 at least once every 10.24s.

Observation 3: Some mobility test cases can only properly performed in the presence of at least two satellite configurations for the test case.

Proposal 6: For RRM mobility test cases, when two satellites are considered in the test configuration, the two generated satellites might be in the same orbital plane.

Observation 4: Channel Models for NTN are currently contained within TR 38.811

Observation 5: Currently it is not expected (within the WID) for TR 38.811 to be impacted.

Observation 6: The varying doppler profiles could be captured in the appropriate performance specifications.

Proposal 7: RAN4 to incorporate the new TE-Channel model in TS 38.101-5 and reference it from RRM and demodulation specifications.

- References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, Samsung, RAN #105, Melbourne, Australia, September 9-12, 2024+

- R4-2413516, " Way Forward for [112][329] NTN_testing_NGSO_channel_model ", Samsung, RAN4 #112, Maastricht, Netherlands, Aug. 19-23, 2024.

- TS 38.1015-5 “NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements”, Release 18.7.0, 3GPP

- R1-2102360, “Considerations on UL timing and frequency synchronization in NTN”, Thales, RAN1 #104, e-Meeting, April 12th – April 20th, 2021