3GPP TSG-RAN WG4 Meeting #113	R4-2419633

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	On the specification impact of CB-Msg3 transmission without Msg1 and 2.

Agenda item:	7.27.3

Document for:	Discussion

Introduction

In RAN2 #126, RAN2 has reached agreements in the Rel-19 WI related to NB-IoT over NTN (IoT_NTN_Ph3-Core) which might have impact/be impacted by RAN4 UE requirements. This was indicated by RAN2 in their LS to RAN4 [1].

RAN2 has asked RAN4 to analyze whether it is possible for a UE in IDLE mode to skip MSG1 in the RACH procedure and still obtain timing synchronization before MSG3. Further inquiries regarding this subject were sent to RAN4 by RAN1 [3]. RAN4 has consistently replied to both inquiries that from the UE point of view the requirements are met, however, there are ongoing concerns that from the system point of view, such feature might cause UL disruption in the system.

RAN1 has followed with the specification work, and it might result on CB-MSG3  without MSG1/2 being adopted as a feasible solution for standards.

Discussion

Assuming that the transmission of MSG3 without MSG1/2 in initial access is a feature being considered by RAN1 and likely to be adopted, RAN4 needs to discuss the specification impact caused by such feature.

Specification impact of CB-MSG3

Timing Requirements

Assuming contention-based MSG3 is approved, the timing requirements must be updated to include the situation where MSG3 is transmitted without initialization of timing advance commands. Currently, clause 7.20A.2 in TS 36.133 specifies that:

The UE initial transmission timing error shall be less than or equal to ±Te where the timing error limit value Te is specified in Table 7.20A.2-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA_Ref for NPRACH is defined as 0. NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.22A was applied. NTA_Ref for other channels is not changed until next timing advance is received.

The part highlighted in green was included to cover for the case where the UE has to initiate a transmission without initialized values for NTA_Ref. Previously this was only possible during NPRACH transmissions. But now, RAN4 needs to make unambiguous which value the UE shall adopt as NTA_Ref  also in case of CB-MSG3 transmission. Considering the lack of previous information for NTA_Ref , the most reasonable might be 0, unless specified otherwise by RAN1.

If the UE performs CB-MSG3 transmission, the value of NTA_Ref is to be considered equal to zero, unless otherwise agreed by RAN1.

Conclusion

The paper has provided our views on the CB MSG 3 transmission and we have proposed the following:

Proposal 1: If the UE performs CB-MSG3 transmission, the value of NTA_Ref is to be considered equal to zero, unless otherwise agreed by RAN1.

- References

- R2-2405769, “LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, ZTE, RAN2#126, Fukuoka, Japan, May 20 – May 24, 2024.

- R4-2414114, “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”,

- R1-2407548 “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, ZTE, RAN1 #118, Maastricht, NL, August 19th – 23rd, 2024

- TS 36.133 “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”, 3GPP, Release 18.6.0

- TS 36.211 “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”, 3GPP, Release 18.6.0