# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2419637

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	7.1.1.1.2

Source: 	Huawei, HiSilicon

Title:	Discussion on the Intra-band CA with PC1.5

Document for:	Approval

# 1	Introduction

This paper discusses the remaining open issues for PC1.5 intra-band CA as captured in the WF [1], especially the Pcmax issue.

## 1.1 MPR assumption

Agreement:

- Keep the following last meeting’s agreement for intra-band non-contiguous ULCA

- Define MPR/A-MPR requirements based on equal PSD assumption for intra-band contiguous ULCA

  - It does not preclude companies to bring evaluation for unequal PSD

- MPRc = MPR, which means no change of spec

## 1.2 PCMAX,f,C and PCMAX

Agreement:

- For a contiguous intra-band combination with PC1.5 capability supporting by txDiversity-r16

  - The PCMAX,f,c and the total configured output power PCMAX for the band combination are given by clause 6.2A.4.1.1 for intra-band contiguous CA in the current version of 38.101-1, no change for PC1.5

- For a non-contiguous intra-band combination with PC1.5 capability supporting dualPA-Architecture

  - For PCMAX,f,c, keep the following last meeting’s agreement, and the detailed wording can be discussed based on the CR in future

  - For PCMAX, FFS

# 2	Discussion

In the current specs, MPRc = MPR is specified for both contiguous and non-contiguous intra-band ULCA. Use contiguous CA as an example, which is shown below:

For PC1.5 ULCA, the following can be derived based on MPRc = MPR:

Table 1: The configured max output power for CA and component carriers

|  | Ppowerclass,CA (dBm) | Ppowerclass,c (dBm) | Pcmax,CA (dBm) | Pcmax,c1 (dBm) | Pcmax,c2 (dBm) | Pcmax,c1+Pcmax,c2 (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| 2Tx | 29 | 29 | 29-MPR | 29-MPR | 29-MPR | 32-MPR |
| DualPA | 29 | 26 | 29-MPR | 26-MPR | 26-MPR | 29-MPR |

It can be seen that:

Observation 1:

- For 2Tx, if both CCs are transmitted at the configured max power per cc Pcmax,c, the total power will exceed the configured max power for CA Pcmax,CA;

- For DualPA, both CCs have to be transmitted at the configured max power per cc Pcmax,c, in order for the total power to meet the configured max power for CA Pcmax,CA, which means equal power per CC regardless of RB numbers.

In summary, it’s not feasible for UE to meet the configured maximum power per CC Pcmax,c during ULCA operations.

Proposal 1: A UE is not required to meet the configured maximum power per CC Pcmax,c during intra-band ULCA operations, and Pcmax,c is calculated only for PHR purpose.

Regarding PH reporting, the following requirement has been specified for intra-band ULCA:

“For PH reporting the following exception applies: if the UE is configured with multiple uplink serving

cells, the power PCMAX,c used for the purpose of PH reporting on first serving cell c = c1 does not consider for

computation of the PH report transmissions on a second serving cell c2 as exempted in subclause 7.7.1 in [8].”

It can be seen that:

Observation 2: Based on the current specification, the PHR for intra-band ULCA does not consider power sharing between component carriers.

Starting from equal PSD for transmission, the CC that has large number of RBs allocated will transmit at much higher power than the CC that has small number of RBs. Since the configured maximum power per CC Pcmax,c is the same for both CCs, the PHR will be much bigger for the CC that has small number of RBs.

Observation 3: For intra-band ULCA, the configured max power the Pcmax,c is the same per CC, while PHR per CC varies with the number of RBs allocated per CC.

For 2Tx architecture, both CCs that are transmitted from the same PA go through the same RF path. Hence, it’s infeasible to perform individual power control per CC.

Observation 4: For intra-band ULCA, it’s not feasible for a UE to perform independent power control per CC and still meet the MPR requirements.

Furthermore, it’s worth noting that the equal PSD condition is specified in the requirements for relative power tolerance. For example, for intra-band contiguous ULCA:

Proposal 2: Consider to add the equal-PSD condition as in the relative power tolerance to the configured transmitted power requirements for intra-band ULCA.

Lastly, for PCMAX:

Proposal 3: Consider Ppowerclass,c = 29dBm for 2Tx and Ppowerclass,c = 26dBm for DualPA, and Ppowerclass,CA=29dBm as the reference power level for MPR for both PA architectures when defining the configured max output power PCMAX requirements for PC1.5 intra-band ULCA.

# 3	Conclusion

In this paper, we have shared our views on the configured output power requirements for PC1.5 intra-band ULCA.

The following observations and proposals are made:

Observation 1:

- For 2Tx, if both CCs are transmitted at the configured max power per cc Pcmax,c, the total power will exceed the configured max power for CA Pcmax,CA;

- For DualPA, both CCs have to be transmitted at the configured max power per cc Pcmax,c, in order for the total power to meet the configured max power for CA Pcmax,CA, which means equal power per CC regardless of RB numbers.

Proposal 1: A UE is not required to meet the configured maximum power per CC Pcmax,c during intra-band ULCA operations, and Pcmax,c is calculated only for PHR purpose.

Observation 2: Based on the current specification, the PHR for intra-band ULCA does not consider power sharing between component carriers.

Observation 3: For intra-band ULCA, the configured max power Pcmax,c is the same per CC, while PHR per CC varies with the number of RBs allocated per CC.

Observation 4: For intra-band ULCA, it’s not feasible for a UE to perform independent power control per CC and still meet the MPR requirements.

Proposal 2: Consider to add the equal-PSD condition as in the relative power tolerance to the configured transmitted power requirements for intra-band ULCA.

Proposal 3: Consider Ppowerclass,c = 29dBm for 2Tx and Ppowerclass,c = 26dBm for DualPA, and Ppowerclass,CA=29dBm as the reference power level for MPR for both PA architectures when defining the configured max output power PCMAX requirements for PC1.5 intra-band ULCA.

# References

- R4-2417098 WF on HPUE RF requirements, Samsung, Skyworks, Nokia, RAN4#112bis