# 3GPP TSG-RAN WG4 Meeting #113	R4-2419639

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	5.9.1

Source: 	Huawei, HiSilicon

Title:	Discussion on NTN NB-IoT inband operation

Document for:	Approval

# 1	Introduction

At RAN4#112bis meeting, it was agreed to study the open issues related to NB-IoT inband operation within a NR carrier for NTN [1].

- Inband or guard band operation at network for NR NTN

  - Option 1: inband

- In-band/guard band operation should be mandatory or optional feature

  - Option 1: mandatory

  - Option 2: optional

- From which release the in-band/guard band operation mode should be mandatory supported by NTN NB-IoT UE

  - Option 1: Rel-18

  - Option 2: Rel-19

  - Option 3: Not a mandatory feature

- From which release the in-band/guard band operation mode should be optional supported by NTN NB-IoT UE

  - Option 1: Rel-18

  - Option 2: other release

- Signaling aspect and other potential cross group impact

  - Issue 1: When configured with non-anchor carrier, NTN NB IoT UE needs to apply M_DL offet differently for standalone mode and in-band mode, hence NTN NB-IoT needs to know which deployment mode the configured anchor carrier is:

    - Further check if the NTN NB-IoT UE can use the MIB IE below to identify the In-band/guard band and standalone mode.

  - Further check impact and/or consequencies in RAN1 specs if in-band mode is indicated

MasterInformationBlock-NB field descriptions

  - operationModeInfo

    - Deployment scenario (in-band/guard-band/standalone) and related information. See TS 36.211 [21] and TS 36.213 [23].

    - Inband-SamePCI indicates an in-band deployment and that the NB-IoT and LTE cell share the same physical cell id and have the same number of NRS and CRS ports.

    - Inband-DifferentPCI indicates an in-band deployment and that the NB-IoT and LTE cell have different physical cell id.

    - guardband indicates a guard-band deployment.

  - Issue 2: whether to define the UE capability for in-band operation

    - Option 1: Yes

    - Option 2: No

- Which CR category should be used for TEI CR?

    - Option 1: Category C

    - Option 2: Category B

- Which wording for the draftCR adding the F_DL for in-band/guard band operation:

  - Option 1: Adding the same  F_DL forumular with in TS 36.101

    - i.e option 1 in draftCR R4-2416114

  - Option 2: keep the same formular in TS 36.102 and modifying the M_DL offset in a note for inband operation.

  - Option 3: other option not excluded

- Other specification impact

- SAN TS 38.108 impact

  - Evaluating NTN NB-IoT in-band operation RF specificaiton in next meeting

- SAN TS 36.108 impact

  - EARFCN to adopt the same with 36.102

- SAN TS 38.181 impact

  - Declaration of the NTN NB-IoT operation

  - Related test for core requirement in TS 38.108

# 2	Discussion

During the discussion in last meeting, it was raised that guard-band mode might be needed from the UE perspective. However, as described in [2], the potential use case happens when refarming the LTE carrier to NR carrier with the existence of downlink anchor guard-band NB-IoT operations.

### 5.3.2	Refarming consideration

When refarming the LTE carrier to NR carrier with the existence of downlink anchor guard-band NB-IoT operations, keeping same NB-IoT carrier frequency, NB-IoT might partially operate in NR in-band and partially in NR guard band as shown in figure 5.3.1.

Figure 5.3.1: Downlink anchor guard-band NB-IoT operation for 10MHz LTE channel bandwidth refarming to 15kHz SCS with 100kHz channel raster NR

For NTN, there’s no reframing need to be considered.

Proposal 1: Do not consider NB-IoT guard-band operation for NR NTN.

Generally speaking, a new feature has to be optional when it’s added to a frozen release so as to avoid NBC issues.

Proposal 2: The inband operation mode should be optional if added to Rel-18 as it’s already frozen. And it can be mandatory from Rel-19.

As seen from Figure 1, the NB-IoT DL during inband operation mode will avoid using the resource elements that might be used by the LTE PDCCH or CRS.

Figure 1: Illustration of the LTE-NB Frame Structure for In-Band Deployment (Even Radio Frame) (source: www.sharetechnote.com/html/Handbook_LTE_NB_FrameStructure_DL.html)

Observation 1: If operating inband with NR, there is no LTE CRS for NB-IoT DL to avoid.

Furthermore, the existing opertionModeInfo IE conveyed by the MIB may indicate inband-SamePCI or inband-DifferentPCI as shown below.

Figure 2: The existing operationModeInfo IE in the MIB.

Observation 2: The “eutra-NumCRS-Ports” information contained in Inband-DifferentPCI is irrelevant for a NR carrier.

When Inband-SamePCI is received, the NB-IoT UE may follow the procedure for acquiring cell-specific reference signal

sequence and raster offset as described in TS 36.213.

## 16.8	UE procedure for acquiring cell-specific reference signal sequence and raster offset

If the higher layer parameter operationModeInfo indicates inband-SamePCI for a cell, the UE may derive cell-specific reference signal sequence and raster offset from the higher layer parameter eutra-CRS-SequenceInfo according to Table 16.8-1, where E-UTRA PRB index  is defined as .

Table 16.8-1: Definition of eutra-CRS-SequenceInfo

| eutra-CRS- SequenceInfo | E-UTRA PRB index for odd number of | Raster offset | eutra-CRS- SequenceInfo | E-UTRA PRB index for even number of | Raster offset |
| --- | --- | --- | --- | --- | --- |
| 0 | -35 | -7.5 kHz | 14 | -46 | +2.5 kHz |
| 1 | -30 |  | 15 | -41 |  |
| 2 | -25 |  | 16 | -36 |  |
| 3 | -20 |  | 17 | -31 |  |
| 4 | -15 |  | 18 | -26 |  |
| 5 | -10 |  | 19 | -21 |  |
| 6 | -5 |  | 20 | -16 |  |
| 7 | 5 | +7.5 kHz | 21 | -11 |  |
| 8 | 10 |  | 22 | -6 |  |
| 9 | 15 |  | 23 | 5 | -2.5 kHz |
| 10 | 20 |  | 24 | 10 |  |
| 11 | 25 |  | 25 | 15 |  |
| 12 | 30 |  | 26 | 20 |  |
| 13 | 35 |  | 27 | 25 |  |
|  |  |  | 28 | 30 |  |
|  |  |  | 29 | 35 |  |
|  |  |  | 30 | 40 |  |
|  |  |  | 31 | 45 |  |

Observation 3: The UE procedure defined for inband-SamePCI is designed for up to 20MHz LTE carriers, which might not be applicable for NR carriers.

In Rel-16, the synchronisation between NR and NB-IoT was studied for TN [2], and it was concluded that there should be no interference issue due to TAGs misalignment between NB-IoT and NR if 15kHz SCS is used in the NR system.

Case 1: the timing misalignment scenario with NR SCS 15KHz

For NR UE transmit timing, if uplink subcarrier spacing is configured with 15KHz and SSB SCS is configured with 15KHz which is specified with worst transmit timing error for uplink SCS 15KHz, the maximum transmit timing error should be 12*64*Tc+256*Tc+8*64*Tc equal to (12+4+8)*64*Tc equal to 24*Ts which is the same as the maximum LTE UE transmitting error. Based on the above considerations, maximum arrival timing difference between NR carrier and NB-IoT carrier should be 4.0798s as shown in Figure 5.5-1 which is less than the NR CP length 4.96us if SCS 15KHz is configured for NR.

Figure 5.5-1. the maximum arrival timing difference between NR and NB-IoT in Uplink

However, for NTN, the timing error requirements for both NR-NTN and IoT-NTN are relaxed in comparison to those of TN. Hence, the maximum arrival timing difference between NR carrier and NB-IoT carrier could exceed the length of CP.

Observation 4: The timing misalignment between NR-NTN and NB-IoT could exceed the length of CP.

Proposal 3: Further study how to reuse the existing operationModeInfo IE to signal the raster offset for inband deployment as well as the impact of UL timing misalignment between NR-NTN and NB-IoT NTN.

As per the meeting guidelines from the chairman, Cat-B CRs should be used to introduce a new feature.

- For TEI, if you plan to trigger a new topic which has no corresponding WI code and have to submit CR(s) with TEI-xx as the work item code for this topic, please contact session Chairs first, because TEI topics are under monitoring by RAN

  - Please submit the CRs by providing a TEI identifier and include it in the title of CRs.

    - Example of TEI identifier: [n77_Canada], which should be put in the tail of the tdoc title, e.g., UE RF requirements for … [n77_Canada]

    - TEI identifier should be provided for all the CRs with TEI18/TEI17 as WI code, otherwise the CRs cannot be approved officially

    - If CRs correspond to the previous release but the agreement in the group is to change it from Rel-18/17, the WI code of the previous release WID plus TEI18/TEI17 should be used as WI code, e.g., Work item code: NR_pos_enh-Core, TEI18, for which no TEI identifier is needed.

    - If TEI17 Cat-F CR was approved, the WI code for its Rel-18 Cat-A CR should be TEI17 rather than TEI18.

  - The first CRs of one TEI topic to introduce a new should be prepared as Cat-B CRs. The CRs which correct the specification for the previous TEI topics should be submitted as Cat-F or Cat-A.

  - Please refer to RP-240858 for the rule of TEI.

  - Proponents of TEI CRs shall explicitly check during the quarter that all relevant work is completed in all RAN WGs before asking approval from RAN plenary. (Conclusions of RP-241618)

Proposal 4: When all relevant work for introducing the inband mode is completed in all RAN WGs, submit all necessary Cat-B CRs as a package to RAN.

Regarding the modification to the carrier frequency offset, option 1 of [3] is used as the baseline and some revisions are needed.

### 5.4B.2	Channel raster, carrier frequency and EARFCN

The channel raster of UE category NB1/NB2 shall be as defined in clause 5.4A.2, and the channel raster per-frequency band shall be as defined in table 5.4A.2-1.

The carrier frequency of UE category NB1/NB2 in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, -0.5, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD. In standalone operation, the relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.4A.2-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL)

The carrier frequency of UE category NB1/NB2 in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.4A.2-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

NOTE 1:	For the carrier including NPSS/NSSS for stand-alone operation, MDL = 0.

=========================================================

<Option 1a>

NOTE 2:	For the carrier supporting in-band operation, the following equation is used.

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1)

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL+1)

<Option 2a>

NOTE 2:	For the carrier supporting in-band operation, in-band MDL = MDL + 0.5 and MUL = MUL + 0.5.

=========================================================

NOTE 3	For the carrier supporting in-band operation, MDL = -0.5 is not applicable for in-band operation.

NOTE 4	For the carrier including NPSS/NSSS for in-band operation, MDL is selected from {-2, -1, 0, 1}.

NOTE 5	Additional UE capability report for in-band operation would be required and introduced.

Proposal 5: Consider both option 1a and 2a as shown above for determining FDL and FUL for inband operation. And note that option 2a could cause confusion that the addition in NOTE 2 is also applicable to the MDL/MUL value when it’s signalled via higher layer messages at the network side.

# 3	Conclusion

In this paper, we have discussed the open issues related to adding inband mode for NB-IoT in NR-NTN carrier and made the following proposals:

Proposal 1: Do not consider NB-IoT guard-band operation for NR NTN.

Proposal 2: The inband operation mode should be optional if added to Rel-18 as it’s already frozen. And it can be mandatory from Rel-19.

Observation 1: If operating inband with NR, there is no LTE CRS for NB-IoT DL to avoid.

Observation 2: The “eutra-NumCRS-Ports” information contained in Inband-DifferentPCI is irrelevant for a NR carrier.

Observation 3: The UE procedure defined for inband-SamePCI is designed for up to 20MHz LTE carriers, which might not be applicable for NR carriers.

Observation 4: The timing misalignment between NR-NTN and NB-IoT could exceed the length of CP.

Proposal 3: Further study how to reuse the existing operationModeInfo IE to signal the raster offset for inband deployment as well as the impact of UL timing misalignment between NR-NTN and NB-IoT NTN.

Proposal 4: When all relevant work for introducing the inband mode is completed in all RAN WGs, submit all necessary Cat-B CRs as a package to RAN.

Proposal 5: Consider both option 1a and 2a as shown above for determining FDL and FUL for inband operation. And note that option 2a could cause confusion that the addition in NOTE 2 is also applicable to the MDL/MUL value when it’s signalled via higher layer messages at the network side.

# References

- R4-2417184 WF for NTN NB-IoT Inband operation, RAN4#112bis

- TR 37.824 NR and E-UTRA coexistence between NB-IoT and NR, v16.0.0

- R4-2416114 (TEI18) Draft CR to 36.102 In-band NB-IoT NTN deployment with NR, MediaTek, RAN4#112bis