3GPP TSG-RAN4 Meeting #113	R4-2419645

# Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on IoT-NTN HPUE Tx requirements and SAR

Source:	Samsung

Agenda item:	7.8.2.3.1

Document for:	Approval

1	Introduction

In the RAN4#112 meeting, the WF R4-2417205 [1], the way forwards of IoT-NTN HPUE Tx requirements were captured especially on feasibility of power classes and the SAR topics.

In this paper, we will further discuss the Tx requirements and SAR for NTN HPUE handheld devices.

2	IoT-NTN HPUE Tx requirements

Similar as the NR-NTN HPUE Tx requirement discussions, most Tx requirements are pending on the co-ex study conclusions. And considering the agreement in co-ex study in last meeting [2], that the ACLR for IoT-NTN HPUE would also take the co-ex study preliminary observation as a starting point, and to further check the AMPR, additional SE, and also considering the implmentation difficulties taking into account the state-of-art duplexer and PA.

Issue 1-8: NR-NTN ACIR/ACLR Agreement: - Consider following observations can be used as a starting point for RF requirements analysis/studies: - Candidate ACLR value for NR-NTN PC2/PC1.5/PC1, based on initial co-ex study results received in #112bis meeting, is 30 dBc - The current TN requirements are more stringent than the observations from co-existence results. Overlapping geographic coverage between NTN and TN is discussion in RAN4 in other WIs. - FFS for final ACLR requirements Issue 1-9: IoT-NTN ACIR/ACLR Agreement: - Take same approach in issue 1-8

Following the WF shown above, we have proposed a separate tdoc carrying all the submitted co-ex results before this #113 meeting. And from the collected co-ex results, we have following observations from co-ex study point of view.

Observation 1: For PC2, most sources support ACLR smaller than 10 dBc if isolation distance of 1.5km is considered.

Observation 2: For PC2, most sources support ACLR can be around 25 dBc if no isolation distance is considered.

Observation 3: For PC1, most sources support ACLR can be smaller than 20 dBc if isolation distance of 1.5km is considered.

Observation 4: For PC1, most sources support ACLR can be around 30 dBc if no isolation distance is considered.

Proposal 1: To adopt similar approach as agreed in #112bis WF [2], to study the Tx requirements considering the ACLR observations in Observation 1~4 in parallel with the co-ex study discussions.

3	SAR for handheld

In last meeting, the SAR solutions for handheld devices were slightly discussed. And it was agreed to take P-MPR as baseline and continue the effort to study other potential solutions in addition to P-MPR solution.

In last meeting, we proposed the meeting to further discuss whether the duty cycle can be applicable for FDD band considering the NTN scenario.

The NTN FDD bands did not come with the TDD-like UL/DL semi-static configurations, and the scheduling is totally up to the network and cannot be justified for the UE for SAR compliance when addressing its SAR to the regulators.

In order to justify UE’s SAR conformance to the regulator, and considering to adopt a method similar to duty-cycle, it’s better to have a semi-static ’duty cycle’ like configuration, and it should be applicable to NTN FDD bands.

Considering the nature of the NTN network, which needs to serve multiple beams in a TDM manner, the serving time ratio per beam/coverage seems to be something that likely to be the ’duty cycle’ in NTN FDD bands operations.

Observation 5: The NTN SAN has to serve different beams in a TDM manner. And the serving time ratio of each beam from a NTN SAN can be something to close the gap between the TDD duty-cycle solution to the NTN FDD bands.

Proposal 2: To consider the serving time of each beam, or the beam service time ratio of the UE, as a potential upper bound of the UL ’duty-cycle’ for the NTN HPUE.

In last meeting, some companies, including us, proposed the NTN UE usually need to perform critical communications especially during the emergency situations.

Observationt 6: The previous duty-cycle solution target to reduce the maximum power of a HPUE, which does not fulfill the demand of having HPUE in NTN scenario for critical services especially under natural disasters.

Proposal 3: To further discuss the duty-cycle solution, and to check if it can be altered for NTN scenario to allow UE to stay in its declared power class as much as possible, which helps NTN HPUE to close the link in emergencies and compensate the rain/cloud fading during natural disasters.

4	Conclusion

For the IoT NTN HPUE Tx requirements and SAR, we propose the following.

Observation 1: For PC2, most sources support ACLR smaller than 10 dBc if isolation distance of 1.5km is considered.

Observation 2: For PC2, most sources support ACLR can be around 25 dBc if no isolation distance is considered.

Observation 3: For PC1, most sources support ACLR can be smaller than 20 dBc if isolation distance of 1.5km is considered.

Observation 4: For PC1, most sources support ACLR can be around 30 dBc if no isolation distance is considered.

Proposal 1: To adopt similar approach as agreed in #112bis WF [2], to study the Tx requirements considering the ACLR observations in Observation 1~4 in parallel with the co-ex study discussions.

Observation 5: The NTN SAN has to serve different beams in a TDM manner. And the serving time ratio of each beam from a NTN SAN can be something to close the gap between the TDD duty-cycle solution to the NTN FDD bands.

Proposal 2: To consider the serving time of each beam, or the beam service time ratio of the UE, as a potential upper bound of the UL ’duty-cycle’ for the NTN HPUE.

Observationt 6: The previous duty-cycle solution target to reduce the maximum power of a HPUE, which does not fulfill the demand of having HPUE in NTN scenario for critical services especially under natural disasters.

Proposal 3: To further discuss the duty-cycle solution, and to check if it can be altered for NTN scenario to allow UE to stay in its declared power class as much as possible, which helps NTN HPUE to close the link in emergencies and compensate the rain/cloud fading during natural disasters.

5	Reference

[1] R4-2417205, WF on HPUE for IoT NTN, vivo

[2] R4-2417122, WF on co-existence study and HPUE for NR-NTN, Samsung