3GPP TSG-RAN4 Meeting #113	R4-2419646

# Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on IoT-NTN HPUE Rx requirements

Source:	Samsung

Agenda item:	7.8.2.3.2

Document for:	Approval

1	Introduction

In the RAN4#112bis meeting, the WF R4-2417205 [1] has agreed on the RSD form and band 256 RSD starting point, similar as in NR-NTN PC2 discussions.

2	Reference Sensitivity Degradation

In WF[1], the RSD were agreed as follows:

RSD (for full duplex eMTC based) Issue 2-1-2: RSD definition Agreement: - Follow NR approach for RSD as much as possible. Issue 2-1-3: PC2 RSD - WF - Reuse NR agreements: - For 256: [0dB] RSD for PC2 for 1Tx and 2Tx (if considered) - FFS on the duplexer isolation and PA interference Issue 2-1-4: PC1 RSD - Proposals - Proposal 1: For PC1 1Tx for full duplex eMTC based Iot-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB (for n254/n255/n256). - Recommended WF - FFS

In our evaluation, given the small CBW as for IoT and the high Tx-Rx duplex frequency separation, for band 254 it can follow similar approach as band 256, which is to have [0] dB RSD for PC2.

Proposal 1: To consider [0] dB RSD for PC2 for band 254 IoT-NTN UE.

3	Conclusion

For the IoT-NTN Rx requirements, we propose the following.

Proposal 1: To consider [0] dB RSD for PC2 for band 254 IoT-NTN UE.

4	Reference

[1] R4-2417205, WF on HPUE for IoT NTN, vivo