3GPP TSG-RAN4 Meeting #113	R4-2419647

# Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on Ku-band co-ex study

Source:	Samsung

Agenda item:	7.9.2

Document for:	Approval

1	Introduction

In the RAN4#112bis meeting, the WF R4-2416593 [1] had the following agreements on most NTN Ku-band system parameters for co-ex study.

This paper is to discuss and contribute the rest assumptions for co-ex study.

2	VSAT characteristics

In last meeting, the WF[1] captured agreements on VSAT Tx characteristics for co-ex study as follows:

Sub-topic 2-9: VSAT equivalent antenna aperture Use 60 cm Sub-topic 2-11: VSAT max Tx antenna gain Use 36.6 dBi (option 1 2W), 36.2 dBi (option 2 10W) Sub-topic 2-16: VSAT max Tx power 40 dBm (10 W)

VSAT antenna:

Given the WF[1] only captured the agreement to re-use the antenna model in Section 6.4.1 of TR 38.811 for Ku-band SAN, the antenna model for Ku-band VSAT is still pending and was not decided for both phased array and parabolic antenna. As there’s a significant portion of assumptions has been decided, or at least been provided the starting point, we would like to suggest the co-ex study to consider the parabolic antenna VSAT as starting point, and at the same time further disucss the candidate phase array VSAT antenna modeling.

For a VSAT with parabolic antenna, we suggest to consider to re-use the antenna model in Section 6.4.1 of TR 38.811 as for Ka-band in Rel-18 NR-NTN studies. And to adopt the equation from TR 38.811 with the equivalent aperture agreed as 60cm, the antenna model would be as follows:

Figure 2-1 Ku-band VSAT antenna gain with equivalent aperture of 60cm

Proposal 1: For VSAT antenna model, to progress the co-ex study with the VSAT equipped with parabolic antenna first. And to further discuss the phased array (flat-panel) antenna modeling in parallel.

Proposal 2: For VSAT with parabolic antenna, to reuse the Section 6.4.1 of TR 38.811 model with the agreed equivalent aperture as 60cm as starting point. The derived antenna model is shown in Figure 2-1 in this paper for reference.

Proposal 3: For VSAT with phased-array or flat panel, consider to re-use the AAS antenna model from TR 38.803 and to encourage VSAT vendors to provide the input parameters of array size (# of elements), element beamwidth, element gain, element spacing, front-to-back ratio, and sub-array size if applicable.

Level; Parameter; Symbol; Unit. Element; Front to back ratio; Am; dB. Side lobe suppression; SLAv; dB. Horizontal half power beamwidth; 3dB; Degrees. Vertical half power beamwidth; 3dB; Degrees. Array element peak gain; GE,max; dBi. Sub-array; Number of element rows in sub-array; Msub; Integer. Vertical element separation; dv,sub; m. Electrical pre-set sub-array down-tilt angle; subtilt; Degrees. Array; Number of elements/sub-array rows; M; Integer. Number of elements columns; N; Integer. Horizontal element separation; dh; m. Vertical element/sub-array separation; dv; m. Electrical down-tilt angle; etilt; Degrees. Electrical scan angle; escan; Degrees.

VSAT EIRP:

Given the max Tx power and max gain are agreed in [1], the max EIRP can be simply derived from the existing agreements as following:

Proposal 4: The VSAT max EIRP can be assumed by agreed max gain and Tx power as following: 76.2 dBm (10W), and 69.6 dBm (2W) for 10W and 2W opitons respectively. The insertion loss is not considered and can be added to the max EIRP based on discussion.

And by combining the antenna models we proposed in Proposal 2, for parabolic antenna VSAT, the off-axis EIRP can be derived as following:

Figure 2-2: Parabolic VSAT eirp with 60cm aperture and 10 W Tx power

Figure 2-3: Parabolic VSAT eirp with 60cm aperture and 2 W Tx power

Observation 1: The eirp of VSAT having 10 W and 2 W Tx power is shown in Figure 2-2 and Figure 2-3 above.

3	TN parameters

In WF [1], it was agreed to use the TR 38.922 v0.3.0 for the TN network as follows:

Sub-topic 2-22: TN parameters NTN to NTN won’t be considered. Continue with the TN - NTN coexistence study noting that the applicability of the VSAT ACS simulation result may have to be relaxed for the core requirement derivation. Use the IMT parameters from TR38.922 version 0.3.0 for the IMT study on 15 GHz as a starting point.

However, in the TR 38.922 v0.3.0, the TN network has many options, and we’d like to express our views on the priority for this Ku-band to accelerate the work.

Based on the previous study, the following options from TR 38.863 [2] can be prioritized from others:

Table 6.3.2-1: Selected option for each scenario Scenario; Aggressor system; Victim system; Environment; Contributing. 1; TN DL; NTN GEO DL; Urban; NTN UE ACS. 2; TN UL; NTN GEO UL; Urban; NTN SAN ACS. 3; NTN LEO-600 DL; TN DL; Rural; NTN SAN LEO ACLR. NTN GEO DL; TN DL; Rural; NTN SAN GEO ACLR. 4; NTN GEO UL; TN UL; Urban; NTN UE ACLR. 5; NTN GEO UL; TN DL; Rural; NTN UE ACLR. 61; NR-TN DL; NTN LEO-600 UL; Rural 2; NTN SAN ACS. NR-TN DL; NTN GEO UL; Rural 2; NTN SAN ACS. 7; HAPS DL; TN DL; Rural; HAPS ACLR. 8; TN UL; HAPS UL; Rural; HAPS ACS. NOTE 1: Agreed representative case for Scenario 6. NOTE 2： The initial results suggested that the NR-NTN SAN would suffer more interference in urban deployment scenario. It is agreed that a more relevant environment for case 6 is a mixture of Urban and Rural environment (e.g., urban area with a 50km diameter inside a GEO beam with a 250km diameter). Further studies based on the mixed urban environment could be considered. As compromise, rural only scenario was then selected..

Proposal 5: For scenario 1, 2, 4, the Urban deployment parameters from TN network can be prioritized over Rural and Dense Urban in TR 38.922.

Proposal 6: For scenario 3, 5, 6, the Rural deployment parameters from TN network can be prioritized over Urban and Dense Urban in TR 38.922.

4	Conclusion

For the NTN Ku-band co-ex studies, we propose the following.

Proposal 1: For VSAT antenna model, to progress the co-ex study with the VSAT equipped with parabolic antenna first. And to further discuss the phased array (flat-panel) antenna modeling in parallel.

Proposal 2: For VSAT with parabolic antenna, to reuse the Section 6.4.1 of TR 38.811 model with the agreed equivalent aperture as 60cm as starting point. The derived antenna model is shown in Figure 2-1 in this paper for reference.

Proposal 3: For VSAT with phased-array or flat panel, consider to re-use the AAS antenna model from TR 38.803 and to encourage VSAT vendors to provide the input parameters of array size (# of elements), element beamwidth, element gain, element spacing, front-to-back ratio, and sub-array size if applicable.

Proposal 4: The VSAT max EIRP can be assumed by agreed max gain and Tx power as following: 76.2 dBm (10W), and 69.6 dBm (2W) for 10W and 2W opitons respectively. The insertion loss is not considered and can be added to the max EIRP based on discussion.

Observation 1: The eirp of VSAT having 10 W and 2 W Tx power is shown in Figure 2-2 and Figure 2-3 above.

Proposal 5: For scenario 1, 2, 4, the Urban deployment parameters from TN network can be prioritized over Rural and Dense Urban in TR 38.922.

Proposal 6: For scenario 3, 5, 6, the Rural deployment parameters from TN network can be prioritized over Urban and Dense Urban in TR 38.922.

5	Reference

[1] R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Eutelsat Group

[2] TR 38.863

[3] TR 38.922 v0.3.0