3GPP TSG-RAN WG4 Meeting #113									                        R4-2419668

Orlando, US, November 18 – 22, 2024

Agenda item:			7.8.2.2.1

Source:	Ericsson

Title:	On UE RF Tx requirements for NR-NTN HPUE

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for NR-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the open issues on UE RF Tx requirements for NR-NTN HPUE from the approved WF in RAN4#112-bis [2].

# Discussion

## General considerations

Issue 2-3: UE architecture/type considerations for NR-NTN PC1.5 UE

Agreement:

- Both 1Tx and 2Tx for PC1.5 non-handheld UE can be considered as starting point for requirements discussions:

- FFS whether PC1.5 non-handheld UE need to be specified if PC1 non-handheld UE be specified

- FFS whether PC1.5 handheld UE can be specified or not

For TN [3], for all FDD bands which support PC1, specification of PC1.5 is “skipped”. The similar can be done for the NTN as well.

Proposal 1: For non-handheld UEs, for a given operating band PC1.5 does not need to be specified if PC1 is specified.

Issue 2-4: UE architecture/type considerations for NR-NTN PC1 UE

Agreement:

- Confirm feasibility of PC1 1Tx for non-handheld UE.

- For PC1, at least 1Tx to be considered for requirements.

- Further discuss consideration of PC1 1Tx, 2Tx, 3Tx for handheld UE.

- Further discuss consideration of PC1 2Tx, 3Tx for non-handheld UE.

In the ongoing Rel-19 WI “NR MIMO Phase 5”, RAN1 has specified only non-coherent TPMI precoders, and full-power transmission modes 1 and 2 are precluded from the WID. Full-power mode 0 is also unlikely to be specified for PC1.5, so only “Rel-15 behavior” will be available. PC1 is out of the scope (not even discussed). It would be pre-mature to consider PC1 with 3Tx for either handheld or non-handheld UEs in Rel-19.

Proposal 2: For TN, in Rel-19 only PC1.5 is being considered for 3Tx (not yet confirmed to be feasible), so 3Tx for PC1 should not be considered in this Release for NTN (for both handheld and non-handheld UEs).

Since in the last meeting companies have agreed that PC1.5 with 1Tx is feasible for NTN UEs, as well as PC1 with 1Tx, PC1 with 2Tx should also be feasible since its complexity is equivalent to having two times PC1.5 with 1Tx.

Proposal 3: PC1 with 2Tx should be feasible for non-handheld NTN UEs, since PC1/PC1.5 with 1Tx is feasible.

Issue 2-8: SEM

Agreement:

- To re-use existing SEM requirements from PC3 to other PCs as starting point.

SEM requirements are specified based on the regulatory requirements and should not depend on the outcome of the co-existence study. In TN UE RF requirements, TS38.101 [3], SEM requirements do not depend on the Power Class. In TS38.101-5, the SEM requirements follow the corresponding ones from TS38.101-1, and those should also not depend on the Power Class.

Proposal 4: The existing SEM requirements from PC3 should be re-used for other PCs as well, as these requirements are specified based on the regulatory requirements.

Issue 2-9: MOP tolerance

Agreement:

- Re-use NR TN TS 38.101-1 nominal MOP and +2/[-3] tolerance for PC2 for NR-NTN Non-Redcap HPUE

In the last meeting, we were the proponents of this proposal, and we still have the same view. We are however open to discuss the tightening of lower ([-3]) tolerance.

Proposal 5: Re-use NR TN TS 38.101-1 nominal MOP and +2/[-3] tolerance for PC2 for NR-NTN. Tightening of the lower bound could be considered.

## SAR related

Regarding the solution which should be specified for SAR compliance, the similar solution can be considered for both NR-NTN and IoT-NTN, where we have made our proposals under AI “7.8.2.3.1 Tx requirements” for “IoT-NTN HPUE RF requirements” [4], since the discussion is taking place there. We will copy our proposals to this contribution as well:

Proposal 6: Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value.

Proposal 7: If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details:

- If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

  - DPC = ‘00’: the criteria to report DPC is not met

  - DPC = ‘01’: ΔPPowerClass = 0 dB

  - DPC = ‘10’: ΔPPowerClass = 3 dB

  - DPC = ‘11’: ΔPPowerClass = 6 dB

- If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

  - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

  - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB

  - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB

  - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB

# Conclusion

Based on the discussion in the previous sections we have made the following proposals:

Proposal 1: For non-handheld UEs, for a given operating band PC1.5 does not need to be specified if PC1 is specified.

Proposal 2: For TN, in Rel-19 only PC1.5 is being considered for 3Tx (not yet confirmed to be feasible), so 3Tx for PC1 should not be considered in this Release for NTN (for both handheld and non-handheld UEs).

Proposal 3: PC1 with 2Tx should be feasible for non-handheld NTN UEs, since PC1/PC1.5 with 1Tx is feasible.

Proposal 4: The existing SEM requirements from PC3 should be re-used for other PCs as well, as these requirements are specified based on the regulatory requirements.

Proposal 5: Re-use NR TN TS 38.101-1 nominal MOP and +2/[-3] tolerance for PC2 for NR-NTN. Tightening of the lower bound could be considered.

Proposal 6: Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value.

Proposal 7: If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details:

- If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

  - DPC = ‘00’: the criteria to report DPC is not met

  - DPC = ‘01’: ΔPPowerClass = 0 dB

  - DPC = ‘10’: ΔPPowerClass = 3 dB

  - DPC = ‘11’: ΔPPowerClass = 6 dB

- If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

  - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

  - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB

  - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB

  - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB

# References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#105.

- R4-2417122, “WF on co-existence study and HPUE for NR-NTN”, Samsung, RAN4#112-bis.

- 3GPP TS 38.101-1 V18.7.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- R4-2419722, “On UE RF Tx requirements for IoT-NTN HPUE”, Ericsson, Sony, RAN4#113.