3GPP TSG-RAN WG4 Meeting #113															R4-2419683

Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 		Further discussion on CLTA aspects

Agenda Item:	7.11.3

Document for:	Discussion

# Introduction

In this contribution, we provide further inputs to the topics covered in the CLTA WF in R4-2416574. WF content is extracted below for reference:

## Sub-topic 1-1: CLTA definition Way forward: Potential outcomes of discussion on co-location reference antenna concept to be considered as alternatives to CLTA, rather than replacing it in Rel-19 OTA test specifications. ## Sub-topic 1-2: BS-to-BS isolation analysis Way forward: BS-to-BS isolation analysis can be further discussed in this WI, with no additional objectives needed. ## Sub-topic 1-3: Beam steering consideration for co-location scenario Way forward: Evaluation of the beam steering impacts is considered as applicable to the co-location scenarios analyses. Applicable frequency range within FR1 is FFS. ## Sub-topic 1-4: TX IMD co-location Way forward: Continue analysis of potential consequences due to TX IMD co-location requirement removal from Rel-19 specification. Applicable frequency boundary for TX IMD requirement removal (within FR1) is FFS.

# Discussion

## Sub-topic 1-1: CLTA definition

This topic was dealing with the issue of NR_BS_RF_req_evo-Perf work item outcomes potentially be treated as a replacement of the CLRA/CLTA concept. WF text has clarified that potential outcomes of discussion on co-location reference antenna concept to be considered as alternatives to CLTA, rather than replacing it in Rel-19 OTA test specifications.

Furthermore, it shall be highlighted that TS 38.141-2 clause 4.12.2.2 already captures the following sentence:

Translation of the requirements to other test antennas are not precluded but suitable translations between the co-location reference antenna and test antenna must be provided to demonstrate that the method is within the specified MU. NOTE: The below defined CLTAs are suitable for testing BS type 1-O implemented with a planar antenna array. The method for testing BS with other antenna array implementations is not covered by the present release of this specification.

With the above it is confirmed that there were also other test antennas considered during past CLTA discussions, and it has been reflected in the NR BS test specification. Therefore, concluding proposal is formulated:

Proposal 1: While CLRA antenna concept is out of scope of this work, other (than previously defined CLTA) test antennas are already allowed by legacy OTA test specifications to be used for the OTA co-location requirements testing.

Therefore, no further discussion on this sub-topic is needed.

## Sub-topic 1-2: BS-to-BS isolation analysis

This topic was addressing measurement results and observations brought by multiple companies. It was found to be common understanding that for higher bands there will be higher isolation due to pathloss.

Related proposals were captured in clause 2.4 for TX IMD.

## Sub-topic 1-3: Beam steering consideration for co-location scenario

This sub-topic was based on the discussion paper in R4-2415674, where wideband horn antennas and the translation with co-location reference antenna was discussed leading to the following proposal: Co-location scenarios for higher FR1 frequencies need to consider beam steering. Related WF was saying that evaluation of the beam steering impacts is considered as applicable to the co-location scenarios analyses.

Referring to the existing co-location requirements in OTA specifications, where are not details provided on the beams configuration. There were no assumptions on the default beam selection captured in the TS (it seems that boresight is currently silently assumed for those co-location tests).

Based on initial analysis of beam steering consideration for co-location scenario, number of high-level questions were identified:

- Investigation on the set of impacted specifications (core, test, or both?),

- Depending on the steering range supported, different couplings would be expected to be observed, leading to question on the underlying core requirement values, impacts BS classes, etc.,

- Investigation on beam steering applicability during the test: to the EUT (to potentially steer the beam towards CLTA), CLTA (to mimic AAS aggressor during the test), or both,

- Investigation on the set of impacted OTA co-location test requirements,

- At this point it is unclear whether all co-location requirements are actually applicable for such analysis. While beam steering is expected to have impact on the coupling levels between EUT and CLTA, we need to first clarify whether this is actually translating into impact on the OTA test requirements.

- Applicable frequency range.

It is felt, that clarification and consensus on the above aspects needs to be reached before progressing further.

Proposal 2: Before proceeding with further work on beam steering consideration for co-location scenario, number of open issues needs to be clarified first, e.g.:

- Investigation on the set of impacted specifications (core, test, etc.),

- Impact of supported steering range on the underlying core requirement values,

- Investigation on beam steering applicability during the test: EUT, CLTA, or both,

- Investigation on the set of impacted OTA co-location test requirements,

- Considered deployments to be modelled (horizontal antennas stacking, vertical stacking, etc.),

- Applicable frequency range.

## Sub-topic 1-4: TX IMD co-location

This sub-topic was related to discussion on potential removal of the TX IMD co-location requirements for high frequency bands. The WF has stated to continue analysis of potential consequences due to TX IMD co-location requirement removal from Rel-19 specification. Applicable frequency boundary for TX IMD requirement removal was kept FFS.

In our previous contribution in [2], we were proposing to remove TX IMD CLTA test for FR1 bands above 4.2 GHz. During previous meeting, there were some concerns raised on the need for analysis of potential consequences related to the removal of some core requirements. In particular, it was highlighted, that there may be some legal issues related to the Japan market. In order to address this concern, we suggest to introduce possibility to still keep the legacy approach (i.e. TX IMD being applicable to all the bands without any restriction) based on the Regional requirements approach.

Proposal 3: TX IMD CLTA test requirements for operating bands above 4.2 GHz to be as regional requirements (declaration based).

With the above, core specifications are not seen to require related updates.

Motivated by the attempt to progress and try to close this topic already this meeting, related Draft CRs to NR BS, AAS BS, IAB radiated test specifications with example implementation of the above proposals were submitted in [3 - 5].

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: While CLRA antenna concept is out of scope of this work, other (than previously defined CLTA) test antennas are already allowed by legacy OTA test specifications to be used for the OTA co-location requirements testing.

Proposal 2: Before proceeding with further work on beam steering consideration for co-location scenario, number of open issues needs to be clarified first, e.g.: Investigation on the set of impacted specifications (core, test, etc.),

- Impact of supported steering range on the underlying core requirement values,

- Investigation on beam steering applicability during the test: EUT, CLTA, or both,

- Investigation on the set of impacted OTA co-location test requirements,

- Considered deployments to be modelled (horizontal antennas stacking, vertical stacking, etc.),

- Applicable frequency range.

Proposal 3: TX IMD CLTA test requirements for operating bands above 4.2 GHz to be as regional requirements (declaration based).

# References

[1]		R4-2416574	Way Forward for [112bis][303] NR_BS_RF_Part2_CLTA, RAN4#112-bis

[2]		R4-2416279	TX IMD test for high frequency band in FR1, RAN4#112-bis

[3]		R4-2419684	Draft CR to TS 37.145-2: removal of TX IMD for high frequency bands

[4]		R4-2419685 	Draft CR to TS 38.141-2: removal of TX IMD for high frequency bands

[5]		R4-2419686	Draft CR to TS 38.176-2: removal of TX IMD for high frequency bands