3GPP TSG-RAN WG4 Meeting #113															R4-2419688

Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	Further inputs on the 15 GHz BS parameters

Agenda Item:	7.2.4.2

Document for:	Discussion

# Introduction

WF on 15GHz BS parameters was approved last meeting in [1].

In this contribution, we provide further inputs on open issues related to 15 GHz BS parameters.

# Discussion

## BS antenna parameters

WF in [1] has captured that the LS reply to ITU-R would capture for Urban Macro an antenna array of [3072 (16*24, SA size 4)]. [] were kept to double-check that for any potential coverage issues. In case of any coverage issues found for [3072], BS antenna array size of 4096 would be selected.

Proposal 1: For the LS reply on 15 GHz BS parameters, confirm BS antenna array size of 3027 (while the size of 4096 is also acceptable as alternative).

## Emissions categories

WF captured that “[category B] and category A“ limits to be covered in the LS reply in 15 GHz parameters for the emission mask as well as for the spurious emissions. Referring to LS replies for the other two lower frequency ranges, both Categoy A and Category B limits were provided, as those were already defined for existing operating bands. Still, we think that there was no need to provide Category B limits for the purpose of ITU-R studies.

In case of 15 GHz evaluations, situation is slightly different, as there are no existing bands defined in this frequency range. If RAN4 would provide some guestimate on the expected future Category B emission limits, there is some risk that regulators may take such inputs for their future evaluations, potentially leading to incorrect conclusions. With this, it is suggested to provide only Category A limits for both emission mask as well as for spurious emissions for 15 GHz range.

Proposal 2: For the LS reply on 15 GHz BS parameters, provide Category A limits only for the emission mask.

Proposal 3: For the LS reply on 15 GHz BS parameters, provide Category A limits only for the spurious emissions.

## ACLR

Based on related co-existence study simulation results provided, it is proposed to define BS ACLR as 30 dB.

Proposal 4: For the LS reply on 15 GHz BS parameters, define ACLR as 30 dB.

# Conclusions

Based on the above discussion, the following proposals were formulated:

Proposal 1: For the LS reply on 15 GHz BS parameters, confirm BS antenna array size of 3027 (while the size of 4096 is also acceptable as alternative).

Proposal 2: For the LS reply on 15 GHz BS parameters, provide Category A limits only for the emission mask.

Proposal 3: For the LS reply on 15 GHz BS parameters, provide Category A limits only for the spurious emissions.

Proposal 4: For the LS reply on 15 GHz BS parameters, define ACLR as 30 dB.

# References

[1]		R4-2417107	WF on 15GHz BS parameters, RAN4#112-bis