# 3GPP TSG-RAN WG4 Meeting # 113                                                         R4-2419714

# Orlando, USA, 18 – 22 November 2024

# Title:	Improvements to valid uplink HPUE configurations

# Agenda Item:	9.1.1.4

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document updates key proposals from [1], and describes new opportunities for additional improvements in case the proposals to simplify the 1UL HPUE MSD requirements are agreed upon in [2], or in case the HPUE MSD requirements are no longer specified, as captured in Option 3 of Way Forward (WF) [3].

# Discussion

At the last meeting, we presented in [1] the results of parsing the TS 38.101-1 Rel. 18.7.0, and discovered several inconsistencies between the specified UL-CA HPUE power classes in the CA configuration tables of clause 5, and the specified power classes defined in the various tables of clause 6. Necessary corrections corresponding to Proposal 2 of [1] are presented at this meeting. The corrections are not reproduced in this document. In this paper, we focus on processes that are designed to improve both the methods used to specify the valid 1UL HPUE configurations, and the valid ULCA HPUE configurations.

## Solutions to simplify the valid 1UL HPUE configurations

Table 1 summarizes the footnotes related to valid 1UL HPUE configurations that are specified in the clause 5 CA configuration tables. We identify a total of 12 different footnotes, across four sets of configuration tables. All footnotes can be accessed in the Annex.

Table 1: Overview of Rel-18 1UL HPUE footnotes in the two, three, four, five, and six-band BCS tables.

| 1UL PC, footnote, Number of Tx |  | two-band BCS Tables 5.5A.3.1-1a ~ .1-1n | three-band BCS Table 5.5A.3.2-1a | four-band BCS Table 5.5A.3.3-1a | five-band BCS Table 5.5A.3.4-1a | six-band BCS Table 5.5A.3.5-1a |
| --- | --- | --- | --- | --- | --- | --- |
| Default PC3 / footnote PC <> PC3 |  | Note 10 | Note 6 | Note 4 | Note 2 | no HPUE request |
| PC2 | Footnote | Note 8 | Note 7 | Note 5 | Note 3 |  |
|  | Number of Tx | up to 2Tx | Not specified |  |  |  |
| PC1.5 | Footnote | Note 9 | Note 9 | Note 6 | Note 5 |  |
|  | Number of Tx | 2Tx | Not specified |  |  |  |

On the subject of valid 1UL HPUE configurations, such as 1UL PC2 for Band n3 in DL CA_n3-n78, our understanding is that capturing these configurations in the clause 5 CA configuration tables was necessary to ensure that the RF requirements are complete. This is particularly important respecting the DL CA MSD requirements. RAN4 is currently evaluating methods to simplify these MSD requirements [3]. One proposal consists of “automatically” specifying  the 1UL HPUE MSD for inter-band CA by using look-up tables (LUTs) [2]. Other proposals include removing all HPUE MSD requirements (option 3 [3]). Thus, the DL CA MSD requirements would either be “automatically” completed for any 1UL power class (except PC1, which is not valid for DL-CA) [2], or would no longer be required (option 3 [3]). If such agreements were to be reached, there would be no need to continue capturing the valid 1UL HPUE configurations in the CA configuration tables. Additionally, multiple footnotes could be removed and/or simplified. Also, the 1UL HPUE operation would automatically be supported for all DL CA band combinations.

Proposal 1: In case the 1UL HPUE MSD requirements are simplified [2], or are no longer specified (Option 3 [3]), we consider simplifying the Rel-19 1UL HPUE configuration tables, by removing from the clause 5 tables:

- The valid 1UL HPUE configurations:

- from Tables 5.5A.3.1-1a ~ 5.5A.3.1-1n:

  - In Note 8, text “or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination”

  - Note 9 and Note 10

- from Tables 5.5A.3.2-1a:

  - Note 6

  - In Note 7, text “… or single uplink carrier in this downlink/uplink combination”

  - Note 9 – this note may be erroneous, see Observation 2

- from Tables 5.5A.3.3-1a:

  - Note 4

  - In Note 5 and Note 6, text “… or single uplink carrier in this downlink/uplink combination”

- from Tables 5.5A.3.4-1a:

  - Note 2 and Note 5

  - In Note 3, text “… or single uplink carrier in this downlink/uplink combination”

The CA configuration tables would then only capture the valid HPUE ULCA configurations. The 1UL HPUE operation would, de-facto, be supported for all DL-CA combinations.

An example of these simplifications is shown in yellow for CA_n3A-n78A in Table 2. For simplification, entries for BCS1 and BCS4/5 are removed.

Table 2: Proposal 1 on simplifying the valid 1UL HPUE configurations for the example of CA_n3A-n78A.

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n78A | n38 n788,9 CA_n3A-n78A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |

NOTE 8:	Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna 		connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 9:	Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 10: 	Only single uplink carriers with power class other than PC3 are listed.

## Solutions to simplify the valid UL-CA configurations

On the topic of valid ULCA HPUE (2UL) configurations, most of the errors reported in [1] may be linked to:

- The use of footnotes in Clause 5 configuration tables. The footnotes not only specify the valid UL ULCA HPUE power-class, but also the number of Tx per band. It is easy to add a HPUE footnote to an UL-CA configuration, without necessarily always checking if, for example, the corresponding UL-CA requirements are complete.

- The use of a different set of footnotes in the Clause 6 ULCA HPUE power class tables. This circumstance  makes it difficult to determine how the HPUE power class, declared in clause 5, maps to the power class of clause 6. In addition, the use of a different set of footnotes to capture the number of Tx per band in clause 5, versus clause 6, may create a condition that is prone to errors.

### Options for the HPUE footnotes of the clause 5 CA configuration tables

The complexity of the Rel-18 footnote handling is summarized in the overview of Table 3, and listed in the Annex.

Observation 1: The valid UL HPUE power classes are captured using a total of  nine footnotes across four sets of CA configuration tables.

Observation 2: Footnotes for PC1.5 are not identical from table to table. For example, Note 9 in Table 5.5A.3.2-1a does not specify that PC1.5 is a valid configuration for this uplink combination, and PC1.5 is valid only for single uplink carriers.

Table 3: Overview of Rel-18 UL-CA power class footnotes in the two, three, four, five, and six-band BCS tables.

| UL CA PC, footnote, Number of Tx |  |  | two-band BCS Tables 5.5A.3.1-1a ~.1-1n |  | three-band BCS Table 5.5A.3.2-1a |  | four-band BCS Table 5.5A.3.3-1a |  | five-band BCS Table 5.5A.3.4-1a |  | six-band BCS Table 5.5A.3.5-1a |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default power class = PC3 |  |  |  |  |  |  |  |  |  |  |  |  |
| PC2 2Tx | Footnote | Note 8 |  | Note 7 |  | Note 5 |  | Note 3 |  | no HPUE request |  |
|  | Number of Tx | 1Tx + 1Tx |  | Not specified |  |  |  |  |  |  |  |
| PC2 3Tx | Footnote | Note 13 |  | Not captured |  |  |  |  |  |  |  |
|  | Number of Tx | 1Tx+2Tx |  |  |  |  |  |  |  |  |  |
| PC1.5 | Footnote | Note 14 |  | PC1.5 not specified as valid for ULCA ? |  | Note 6 |  | PC1.5 not specified as valid for ULCA ? |  |  |  |
|  | Number of Tx | 1Tx+2Tx |  |  |  | Not specified |  |  |  |  |  |

The parsing of Rel-18 TS 38.101-1 illustrates that, despite all the RAN4 efforts, this footnote approach is prone to errors – see Observation 2 [1]. The current process method produces a TS where it is difficult to identify which power class is supported for a given DL CA band combination. With the additional new Rel-19 power class options, we believe RAN4 should consider simplifying this footnote approach, to ensure a future-proof manner of working.

We consider two simplification options.

Proposal 2: Consider either Option 1 or Option 2 to simplify the ULCA HPUE footnotes of the clause 5 tables:

  - Option 1: “Improvements to the current process method”

    - Retain the two-band and three-band CA Rel-18 footnotes to capture the ULCA power class, but remove all text that defines the supported number of Tx per band. This proposal is justified by:

      - Information that may become difficult to maintain for new Rel-19 PC options.

      - The number of Tx inherently captured in the ULCA power class tables, in clause 6.

    - Remove Note 13 from Table 5.5A.3.1-1a ~ 1-1n, since it can be assumed that the DL CA MSD requirements for ULCA account for the worst case PC2 UL TX configuration.

    - Remove the ULCA power class footnotes from the four-band and five-band CA configuration tables.

    - Modify Note 9 to ensure that ULCA PC1.5 is a valid configuration for three-band DL CA.

    - New Rel-19 HPUE options that need to be captured, with additional footnotes.

  - Option 2: If the HPUE MSD requirements are simplified, or are no longer specified [2, and 3], consider to:

  - Continue specifying the supported ULCA configurations in the CA configuration tables, and

  - Remove all footnotes related to ULCA power classes, since the impact on DL CA MSD will be either “automatically specified” (Option 1 [3]), or will no longer be specified (Option 3 [3]).

Option 1 improvements are summarized in Table 4 (in yellow). Option 1a is emphasized with blue highlighted text on top of the Proposal 1 (yellow text) for Note 8.

Table 4: Footnote simplification Proposal 2, Option 1. Deleted footnotes are highlighted in yellow, modified in blue.

| UL CA PC, footnote, Number of Tx |  |  | two-band BCS Tables 5.5A.3.1-1a ~.1-1n | three-band BCS Table 5.5A.3.2-1a | four-band BCS Table 5.5A.3.3-1a | five-band BCS Table 5.5A.3.4-1a | six-band BCS Table 5.5A.3.5-1a |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Default power class = PC3 |  |  |  |  |  |  |  |  |
| PC2 2Tx | Footnote | Note 8 |  | Note 7 | Remove Note 5 | Remove Note 3 | no HPUE request |
|  | Number of Tx | Remove the number of Tx from Note 8 |  | Not specified |  |  |  |
| PC2 3Tx | Footnote | Remove Note 13 |  | Not captured |  |  |  |
|  | Number of Tx |  |  |  |  |  |  |
| PC1.5 | Footnote | Note 14 |  | Modify Note 9 to indicate 2UL PC1.5 | Remove Note 6 | Remove Note 5 |  |
|  | Number of Tx | Remove the number of Tx from Note 14 |  |  |  |  |  |

NOTE 8:	Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna 		connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

### Options for the HPUE footnotes of the Clause 6 ULCA power class tables

In clause 6, various sets of footnotes are used across several power class tables. We anticipate that this condition will worsen with the new Rel-19 power class options. At this stage, it may be advisable to:

- Remove footnotes that specify how a ULCA power class is supported by number of Tx per band, and,

- replace these footnotes with a power class LUT that clearly indicates how a given HPUE PC is supported per band, and the supported number of Tx per band. This new LUT may become easier to maintain and may promote clarity to the supported HPUE ULCA configuration. The LUT format may be modelled using the Rel-19 WID HPUE request excel sheet template.

Proposal 3: Consider clarifying the supported UL configuration for a given power class by introducing a LUT that clearly defines the power class per band, and the number of Tx per band. This LUT may replace the current footnotes from the clause 6 power class tables, and may be modelled using the Rel-19 WID HPUE excel template below.

# Conclusion

Through each release, the number of footnotes used to capture the valid HPUE UL configurations has increased in complexity, making the TS difficult to read and prone to errors. With the new power class options being studied in Rel-19, we anticipate that this footnote approach may become even more difficult to manage. We, therefore, submit Proposals 1, 2, and 3 for consideration. The objective is to simplify and improve the Rel-19 TS.

Proposal 1: In case the 1UL HPUE MSD requirements are simplified [2], or are no longer specified (Option 3 [3]), we consider simplifying the Rel-19 1UL HPUE configuration tables, by removing from the clause 5 tables:

- The valid 1UL HPUE configurations:

- from Tables 5.5A.3.1-1a ~ 5.5A.3.1-1n:

  - In Note 8, text “or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination”

  - Note 9 and Note 10

- from Tables 5.5A.3.2-1a:

  - Note 6

  - In Note 7, text “… or single uplink carrier in this downlink/uplink combination”

  - Note 9 – this note may be erroneous, see Observation 2

- from Tables 5.5A.3.3-1a:

  - Note 4

  - In Note 5 and Note 6, text “… or single uplink carrier in this downlink/uplink combination”

- from Tables 5.5A.3.4-1a:

  - Note 2 and Note 5

  - In Note 3, text “… or single uplink carrier in this downlink/uplink combination”

The CA configuration tables would then only capture the valid HPUE ULCA configurations. The 1UL HPUE operation would, de-facto, be supported for all DL-CA combinations.

Proposal 2: Consider either Option 1 or Option 2 to simplify the ULCA HPUE footnotes of the clause 5 tables:

  - Option 1: “Improvements to the current process method”

    - Retain the two-band and three-band CA Rel-18 footnotes to capture the ULCA power class, but remove all text that defines the supported number of Tx per band. This proposal is justified by:

      - Information that may become difficult to maintain for new Rel-19 PC options.

      - The number of Tx inherently captured in the ULCA power class tables in clause 6.

    - Remove Note 13 from Table 5.5A.3.1-1a ~ 1-1n since it can be assumed that the DL CA MSD requirements for ULCA account for the worst case PC2 UL TX configuration.

    - Remove the ULCA power class footnotes from the four-band and five-band CA configuration tables.

    - Modify Note 9 to ensure that ULCA PC1.5 is a valid configuration for three-band DL CA.

    - New Rel-19 HPUE options that need to be captured, with additional footnotes.

  - Option 2: If the HPUE MSD requirements are simplified, or are no longer specified [2, and 3], consider to:

  - Continue specifying the supported ULCA configurations in the CA configuration tables, and

  - Remove all footnotes related to ULCA power classes, since the impact on DL CA MSD will be either “automatically specified” (Option 1 [3]) or will no longer be specified (Option 3 [3]).

Proposal 3: Consider clarifying the supported UL configuration for a given power class by introducing a LUT that clearly defines the power class per band, and the number of Tx per band. This LUT may replace the current footnotes from the clause 6 power class tables, and may be modelled using the Rel-19 WID HPUE excel template below.

# References

- R4-2416253, Valid Uplink-CA Configurations, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, Skyworks Solutions, Inc.

- R4-2419699 1UL HPUE NRCA MSD Simplifications for Rel-19, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions, Inc.

- R4-2417098, WF on HPUE RF requirements, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, Samsung, Skyworks, Nokia.

# Annex: HPUE footnotes in TS 38.101-1 CA configuration tables

Two-band BCS Tables 5.5A.3.1-1a to 5.5A.3.1-1n

NOTE 8:	Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna 		connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 9:	Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 10: 	Only single uplink carriers with power class other than PC3 are listed.

NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band.

NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band.

Three-band BCS Tables 5.5A.3.2-1a

NOTE 6: 	Only single uplink carriers with power class other than PC3 are listed.

NOTE 7:	Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination

NOTE 9:	Minimum requirements for Power Class 1.5 are applicable for single uplink carrier in this downlink/uplink combination

Four-band BCS Tables 5.5A.3.3-1a

NOTE 4: 	Only single uplink carriers with power class other than PC3 are listed.

NOTE 5:	Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination.

NOTE 6:	Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination.

Four-band BCS Tables 5.5A.3.4-1a

NOTE 2:	Only single uplink carriers with power class other than PC3 are listed.

NOTE 3:	Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination.

NOTE 5:	Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination.