# 3GPP TSG-RAN WG4 Meeting # 113                                                         R4-2419718

# Orlando, USA, 18 – 22 November 2024

# Title:	CR companion for DC_(n)8AA and MSD clean-up

# Agenda Item:	6.2.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents our views on the MSD test point for DC_(n)8AA. This contribution serves as a follow up to the latest agreed upon Way Forward (WF) [1] and to our previous contribution [2] on cleaning-up the intra-band contiguous EN-DC MSD test points. A companion CR [3] is provided to capture the RAN4 agreements.

# Discussion

At RAN4 meeting #112, it was agreed that for DC_(n)8AA only single switched uplink (UL) operation is supported and companies were invited to evaluate the following two MSD test points [4]:

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | FFS | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | FFS |  |
|  | 8 | 910 | 10 | 25 (RBstart = 25) | 955 | FFS |  |
|  | n8 | N/A | 20 | N/A | 940 | FFS |  |
| NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. |  |  |  |  |  |  |  |

At RAN4 meeting #112bis, we observed that [2]:

- the test point with single-uplink transmission in E-UTRA Band 8 does not comply to the clause 7.3B.2.1 core requirements on the placement of downlink (DL) MCG/SCG relative to the UL band, and

- this issue is not specific to DC_(n)8AA as several previously agreed test points also fail to meet these requirements.

In the WF [1], companies were invited to verify the impact of the proposed text changes on the NE-DC MSD requirements, and to come back with an MSD test point proposal for DC_(n)8AA.

## Impact of MSD clean-up proposals on NE-DC

We reproduce in Figure 1 the WF [1] agreement on fixing the non-compliant intra-band contiguous MSD test points.

Figure 1: WF 1 and WF2 [1].

The solution proposed in WF1 consists in changing the placement rule of the MCG/SCG by swapping the relative position of the E-UTRA and NR DL carriers so that the specified MSD test point verifies only the MSD of the LTE anchor. WF2 cleans-up the remaining non-compliant 1UL MSD test point which currently verify the NR SCG MSD. During meeting #112bis, one company raised some concerns to check if WF2 may impact the NE-DC MSD requirements.

The NE-DC RF requirements were recently clarified with the following text from TS 38.101-3 clause 4.2:

“A terminal which supports NE-DC configurations shall meet the minimum requirements for corresponding EN-DC configuration, unless otherwise specified.”

To our understanding, this clarification means that an NE-DC UE may be verified using the EN-DC RF requirements, i.e. after the WF2 clean-up, the NE-DC MSD of the LTE downlink carrier may be verified. We therefore believe that the WF2 agreement does not constitute any issue.

Observation: The WF2 clean-up proposal does not constitute an issue for NE-DC devices.

Proposal 1: Adopt the following changes to the core requirements of clause 7.3B.2.1 and implement the following changes to the Table 7.3B.2.1-1 MSD test points [1,2].

“For EN-DC configurations where uplink is not available in either the MCG or the SCG or for EN-DC configurations where the UE only supports single uplink operation, reference sensitivity requirements are verified with the downlink carrier(s) from the cell group without uplink configured closer to the uplink operating band than any of the downlink carriers from the cell group with uplink.”

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | 6.2 | FDD |
|  | n3 | 1770.0 | 30 | [10 (RBstart = 150)] | 1865.0 | N/A |  |
| DC_(n)5AA | 5 | 826.5 | 5 | N/A | 871.5 | 5.2 | FDD |
|  | n5 | 839 | 20 | 20 (RBend = 105) | 884 | N/A |  |
| DC_(n)5AA | 5 | 829 | 10 | N/A | 874 | 5.2 |  |
|  | n5 | 841.5 | 15 | 20 (RBend = 78) | 886.5 | N/A |  |
| DC_(n)5AA | 5 | 831.5 | 5 | N/A | 876.5 | 5.2 |  |
|  | n5 | 841.5 | 15 | 20 (RBend = 78) | 886.5 | N/A |  |
| DC_(n)5AA | 5 | 834 | 10 | N/A | 879 | 1.5 |  |
|  | n5 | 844 | 10 | 25 (RBend = 51) | 889 | N/A |  |
| DC_(n)12AA | 12 | 703.5 | 5 | N/A | 733.5 | 4.5 | FDD |
|  | n12 | 711 | 10 | 20 (RBend = 51) | 741 | N/A |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 151 | 15 (RBstart = 0) | 6321 | 2.5 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 51 | 5 (RBstart = 0) | 6371 | 2.2 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 101 | 10 (RBstart = 0) | 634.51 | 2.5 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 101 | 10 (RBend = 51) | 634.51 | 29.1 |  |
| DC_(n)71AA3 | 71 | N/A | 10 | N/A | 642.0 | [21.3] | FDD |
|  | n71 | 673.0 | 20 | [5 (RBstart = 0)] | 627.0 | N/A |  |
| DC_(n)71AA3 | 71 | N/A | 15 | N/A | 639.5 | [5.4] |  |
|  | n71 | 670.5 | 15 | [5 (RBstart = 0)] | 624.5 | N/A |  |
| DC_(n)71AA4 | 71 | 680.5 | 5 | N/A | 639.5 | [6.8] |  |
|  | n71 | 670.5 | 151 | [5 (RBstart = 2)] | 627.0 | N/A |  |
| DC_(n)71AA5 | 71 | 680.5 | 5 | N/A | 634.5 | [6.4] |  |
|  | n71 | 670.5 | 15 | [5 (RBstart = 0) | 624.5 | N/A |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. NOTE 4: Applicable only to BCS 1. NOTE 5: Applicable only to BCS 0. |  |  |  |  |  |  |  |

## MSD test point for DC_(n)8AA

We reproduce the 1UL band 8 MSD proposal based on our previous measurements results [2].

Proposal 2: Adopt the following 1UL MSD requirements for B8 in DC_(n)8AA.

| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | 19 | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | N/A |  |

# Conclusion

This paper captures our views on cleaning-up the 1UL intra-band EN-DC MSD test points and on the DC_(n)8AA MSD test point. The following proposals are captured in a change request at this meeting.

Observation: The WF2 clean-up proposal does not constitute an issue for NE-DC devices.

Proposal 1: Adopt the following changes to the core requirements of clause 7.3B.2.1 and implement the following changes to the Table 7.3B.2.1-1 MSD test points [1,2].

“For EN-DC configurations where uplink is not available in either the MCG or the SCG or for EN-DC configurations where the UE only supports single uplink operation, reference sensitivity requirements are verified with the downlink carrier(s) from the cell group without uplink configured closer to the uplink operating band than any of the downlink carriers from the cell group with uplink.”

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | 6.2 | FDD |
|  | n3 | 1770.0 | 30 | [10 (RBstart = 150)] | 1865.0 | N/A |  |
| DC_(n)5AA | 5 | 826.5 | 5 | N/A | 871.5 | 5.2 | FDD |
|  | n5 | 839 | 20 | 20 (RBend = 105) | 884 | N/A |  |
| DC_(n)5AA | 5 | 829 | 10 | N/A | 874 | 5.2 |  |
|  | n5 | 841.5 | 15 | 20 (RBend = 78) | 886.5 | N/A |  |
| DC_(n)5AA | 5 | 831.5 | 5 | N/A | 876.5 | 5.2 |  |
|  | n5 | 841.5 | 15 | 20 (RBend = 78) | 886.5 | N/A |  |
| DC_(n)5AA | 5 | 834 | 10 | N/A | 879 | 1.5 |  |
|  | n5 | 844 | 10 | 25 (RBend = 51) | 889 | N/A |  |
| DC_(n)12AA | 12 | 703.5 | 5 | N/A | 733.5 | 4.5 | FDD |
|  | n12 | 711 | 10 | 20 (RBend = 51) | 741 | N/A |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 151 | 15 (RBstart = 0) | 6321 | 2.5 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 51 | 5 (RBstart = 0) | 6371 | 2.2 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 101 | 10 (RBstart = 0) | 634.51 | 2.5 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 101 | 10 (RBend = 51) | 634.51 | 29.1 |  |
| DC_(n)71AA3 | 71 | N/A | 10 | N/A | 642.0 | [21.3] | FDD |
|  | n71 | 673.0 | 20 | [5 (RBstart = 0)] | 627.0 | N/A |  |
| DC_(n)71AA3 | 71 | N/A | 15 | N/A | 639.5 | [5.4] |  |
|  | n71 | 670.5 | 15 | [5 (RBstart = 0)] | 624.5 | N/A |  |
| DC_(n)71AA4 | 71 | 680.5 | 5 | N/A | 639.5 | [6.8] |  |
|  | n71 | 670.5 | 151 | [5 (RBstart = 2)] | 627.0 | N/A |  |
| DC_(n)71AA5 | 71 | 680.5 | 5 | N/A | 634.5 | [6.4] |  |
|  | n71 | 670.5 | 15 | [5 (RBstart = 0) | 624.5 | N/A |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. NOTE 4: Applicable only to BCS 1. NOTE 5: Applicable only to BCS 0. |  |  |  |  |  |  |  |

Proposal 2: Adopt the following 1UL MSD requirements for B8 in DC_(n)8AA.

| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | 19 | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | N/A |  |

# References

- R4-2417156 WF on DC_(n)8AA test point and MSD test point clean-up, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, Skyworks, Murata.

- R4-2416262 CA_n40-n41 Simultaneous RxTx, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, Skyworks Solutions, Inc.

- R4-241xxxx CR to Rel-18 38.101-3 on intra-band CA MSD clean-up, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, Skyworks Solutions, Inc, Murata Corporation Ltd.

- R4-2414274 WF on introduction of PC2 and 40MHz CBW in NR band n28, 3GPP TSG-RAN WG4 Meeting # 112, Maastricht, The Netherlands, Skyworks Solutions, Inc., Murata Manufacturing Co. Ltd.