3GPP TSG-RAN WG4 Meeting #113                                            R4-2419720

# Orlando, USA, 18th – 22nd of November 2024

Title: 	Discussion on the First Introduction of an NTN NB-IoT TDD Frequency Band

Agenda Item:	7.28.1 - General aspects and work plan

Release:	Rel-19

Source: 	THALES

Document for:	Discussion

# For information

As described in the approved WID RP-242415 for New WID on introduction of IoT-NTN TDD mode (Iridium, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024), the study and work objectives assume the following:

- LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763)

- Target the 1616-1626.5 MHz MSS allocated band

- Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT)

- Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band.

This work item includes the following objectives:

- Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4]

  - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2]

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

- Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4].

  - Specify band numbering

  - Specify SAN and UE RF characteristics

  - Specify DL and UL channelization.

  - Specify channel bandwidth as 200 kHz

  - Note1: No NTN-NTN coexistence study needed.

  - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

# Discussion on the NB-IoT current frequency bands

Previously, the following NB-IoT frequency bands have been introduced (see WF on Rel-19 FR1 NTN bands, R4-2414287, Inmarsat).

#### Issue 2-2-1: Band Numbering

Agreement: Adopt 252 as the band number for the new IoT NTN S-band.

#### Issue 2-2-2: Band Plan

Agreement:

  - Adopt the proposed band plan as follows

E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 2532 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. |  |  |  |  |  |  |  |

Therefore a new TDD NTN NB-IoT frequency band introduction with 1616-1626.5 MHz can use 251 numbering.

Proposal 1: Use “251” E-UTRA operating band number for the new TDD 1616-1626.5 MHz NTN NB-IoT frequency band introduction.

Proposal 2: Adopt the proposed band plan as follows:

E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 2532 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| 251 | 1616 MHz | - | 1626.5 | 1616 MHz | - | 1626.5 | TDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. |  |  |  |  |  |  |  |

Observation 1: For a better representation, the reader shall use the following figure for information purposes:

Figure 1: Current NTN NB-IoT L-band representation

Proposal 2: Unless stated otherwise, reuse TS 36.102 NTN IoT 254 L-band Tx requirements also for 251 band in TS 36.102.

Proposal 3: Unless stated otherwise, reuse TS 36.108 NTN IoT 254 L-band Rx requirements also for 251 band in TS 36.108.

Proposal 4: Unless stated otherwise, reuse TS 36.102 NTN IoT 255 L-band Rx requirements also for 251 band in TS 36.102.

Proposal 5: Unless stated otherwise, reuse TS 36.108 NTN IoT 255 L-band Tx requirements also for 251 band in TS 36.108.

Proposal 6: Re-introduce specific requirements for TDD mode (e.g. transmit ON/OFF power Clauses 6.4 and 9.5 applicable at least to TS 36.108).

Proposal 7: Consider a gap between previous introduced EARFCN FDD NTN (starting from 2^18-1=262143 for NUL and 2^18-2^15-1=229375 for NDL) and new (first) introduced EARFCN TDD NTN.

For the NTN E-UTRA EARFCN number, consider to start the upper range from 2^18-2^16-1=196607 or2^18-2^14-1=245759.

For more details, please check 2022 discussion (Email discussion summary for [104-bis-e][143] LTE_NBeMTC_NTN_UERF, 10th-19th of October 2022, e-meeting):

Issue 2-3-1: Huawei: prefer option 2. The rationale behind option 2 is as follows:

- Start from the max channel number and continue in descending order as per the agreed WF;

- Leave the space counting from B65 untouched (i.e. from 2^16 to 2^17+2^16-1);

- Derive the remaining unused space and split it equally for both DL/UL (i.e. from 2^17+2^16 to 2^18-1).

The figure below illustrates the design:

Proposal 8: For the NTN E-UTRA TDD EARFCN number, consider to start the NUL/NDL upper range from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL) or 2^18-2^14-1=245759 (in-between FDD NTN NOffs-DL and FDD NTN NOffs-UL).

Proposal 9: Use a channel raster of 100 kHz and a Step size of <1>.

Proposal 10: RAN4 to discuss/agree the following table:

Table 1: SAN RF requirements for the new IoT-NTN TDD band (TS 36.108)

| NR BS RF Tx/Rx requirement | Proposal for the new IoT-NTN TDD band (TS 36.108) |
| --- | --- |
| 6.2 Satellite Access Node output power | Not band specific --> No specification impact. |
| 6.3.2 RE power control dynamic range | Not band specific --> No specification impact. |
| 6.3.3 Total power dynamic range | Not band specific --> No specification impact. |
| 6.4 Transmit ON/OFF power | Add this specification to SAN starting from Rel-19 |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Not band specific --> No specification impact. |
| 6.6.4 Out-of-band emissions | Band specific --> Specification impact. Consider reuse of out-of-band emissions requirement for NTN IoT 251 similar as for NTN IoT 255 L-band Tx requirements (TS 36.108) |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable to SAN. |

Proposal 11: RAN4 to discuss/agree the following table:

Table 2: SAN RF requirements for the new IoT-NTN TDD band (TS 36.108)

| NR BS RF Tx/Rx requirement | Proposal for the new IoT-NTN TDD band (TS 36.108) |
| --- | --- |
| 7.2 Reference sensitivity level | Not band specific --> No specification impact. Consider reuse of reference sensitivity level for NTN IoT 251 similar as for NTN IoT 254 L-band Rx requirements (TS 36.108) |
| 7.3 Dynamic range | Not band specific --> No specification impact. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Not band specific --> No specification impact. |
| 7.4.2 In-band blocking | Not applicable for SAN. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Not band specific --> No specification impact. |

# Conclusion

Proposal 1: Use “251” E-UTRA operating band number for the new TDD 1616-1626.5 MHz NTN NB-IoT frequency band introduction.

Proposal 2: Adopt the proposed band plan as follows:

E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 2532 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| 251 | 1616 MHz | - | 1626.5 | 1616 MHz | - | 1626.5 | TDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. |  |  |  |  |  |  |  |

Observation 1: For a better representation, the reader shall use the following figure for information purposes:

Figure 1: Current NTN NB-IoT L-band representation

Proposal 2: Unless stated otherwise, reuse TS 36.102 NTN IoT 254 L-band Tx requirements also for 251 band in TS 36.102.

Proposal 3: Unless stated otherwise, reuse TS 36.108 NTN IoT 254 L-band Rx requirements also for 251 band in TS 36.108.

Proposal 4: Unless stated otherwise, reuse TS 36.102 NTN IoT 255 L-band Rx requirements also for 251 band in TS 36.102.

Proposal 5: Unless stated otherwise, reuse TS 36.108 NTN IoT 255 L-band Tx requirements also for 251 band in TS 36.108.

Proposal 6: Re-introduce specific requirements for TDD mode (e.g. transmit ON/OFF power Clauses 6.4 and 9.5 applicable at least to TS 36.108).

Proposal 7: Consider a gap between previous introduced EARFCN FDD NTN (starting from 2^18-1=262143 for NUL and 2^18-2^15-1=229375 for NDL) and new (first) introduced EARFCN TDD NTN.

Proposal 8: For the NTN E-UTRA TDD EARFCN number, consider to start the NUL/NDL upper range from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL) or 2^18-2^14-1=245759 (in-between FDD NTN NOffs-DL and FDD NTN NOffs-UL).

Proposal 9: Use a channel raster of 100 kHz and a Step size of <1>.

Proposal 10: RAN4 to discuss/agree the following table:

Table 1: SAN RF requirements for the new IoT-NTN TDD band (TS 36.108)

| NR BS RF Tx/Rx requirement | Proposal for the new IoT-NTN TDD band (TS 36.108) |
| --- | --- |
| 6.2 Satellite Access Node output power | Not band specific --> No specification impact. |
| 6.3.2 RE power control dynamic range | Not band specific --> No specification impact. |
| 6.3.3 Total power dynamic range | Not band specific --> No specification impact. |
| 6.4 Transmit ON/OFF power | Add this specification to SAN starting from Rel-19 |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Not band specific --> No specification impact. |
| 6.6.4 Out-of-band emissions | Band specific --> Specification impact. Consider reuse of out-of-band emissions requirement for NTN IoT 251 similar as for NTN IoT 255 L-band Tx requirements (TS 36.108) |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable to SAN. |

Proposal 11: RAN4 to discuss/agree the following table:

Table 2: SAN RF requirements for the new IoT-NTN TDD band (TS 36.108)

| NR BS RF Tx/Rx requirement | Proposal for the new IoT-NTN TDD band (TS 36.108) |
| --- | --- |
| 7.2 Reference sensitivity level | Not band specific --> No specification impact. Consider reuse of reference sensitivity level for NTN IoT 251 similar as for NTN IoT 254 L-band Rx requirements (TS 36.108) |
| 7.3 Dynamic range | Not band specific --> No specification impact. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Not band specific --> No specification impact. |
| 7.4.2 In-band blocking | Not applicable for SAN. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Not band specific --> No specification impact. |