# 3GPP TSG-RAN WG4 Meeting # 113                                                          R4-2419723

# Orlando, USA, 18 – 22 November 2024

# Title:	n28 RSD

# Agenda Item:	6.20.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents the 1Tx and 2Tx Reference Sensitivity Degradation (RSD) for Band n28 as a follow up to the agreed-upon Way Forward (WF) [1], and to our previous RSD proposals [2 3].

# Discussion

In this contribution, the Band n28 RSD is calculated based on the following agreements from WF [1]:

- Band n28 40 MHz REFSENS level of -66.3 dBm.

- Band n28 40 MHz 1Tx PC2 RSD is 3 dB, 2Tx PC2 RSD is 8.2 dB.

RSD is calculated based on the method presented in [4, 5] by reverse engineering the interference levels that correspond with the agreed-upon 1Tx PC3 REFSENS levels, within 0.1 dB. This method is proposed because the legacy agreed-upon REFSENS may be the result of a compromise between proponents and our power amplifier measurements. This often leads to a lower MRC combined REFSENS than those agreed-upon in the specification. For RSD, we assume the following:

- For CBW>3 MHz, the ideal RB scaled REFSENS is calculated from the 5 MHz CBW level of -98.5 dBm. We show in the Annex Table 3 that the agreed-upon REFSENS for 5, 10, 15 MHz CBW is lower, by ~ 0.5 dB, compared to the ideal RB scaled REFSENS, calculated from the 3 MHz REFSENS of -100.2 dBm. We propose, to account for the “mismatch” between the legacy agreed-upon REFSENS, and the new 3 MHz REFSENS, by flooring the RSD at 0.5 dB.

- For 1Tx RSD, we assume a 9 dB interference imbalance between the Main and Diversity antenna ports. Based on the WF 1Tx RSD agreement, we assume that the Power Class 2 (PC2) interference is 3 dB higher than the Power Class 3 (PC3) interference.

- For 2Tx RSD, we assume 0.3 dB reverse IMD.

The details of the RSD calculations can be found in the Annex. This is evaluated for all CBWs supported in Band n28.  This includes the new lowest 3 MHz CBW, as highlighted in blue in Proposal 2 and Proposal 3 below.

Proposal 1: Adopt the 1Tx RSD requirements of Table 1

Table 1: Band n28 RSD from PC3 to PC2 for UE not supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 0 | 0.5 | 0.5 | 0.5 | 1.0 | 2.1 | 2.7 | - | 3.0 |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: Adopt the 2Tx RSD requirements of Table 2

Table 2: Band n28 RSD from PC3 to PC2 for UE supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 0.3 | 0.8 | 0.8 | 0.8 | 1.9 | 6.2 | 7.6 | - | 8.2 |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 |  |  |  |  |  |  |  |  |  |  |  |

# Conclusion

This paper captures our views on the Band n28 RSD.

Proposal 1: Adopt the 1Tx RSD requirements of Table 1

Table 1: Band n28 RSD from PC3 to PC2 for UE not supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 0 | 0.5 | 0.5 | 0.5 | 1.0 | 2.1 | 2.7 | - | 3.0 |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: Adopt the 2Tx RSD requirements of Table 22

Table 2: Band n28 RSD from PC3 to PC2 for UE supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 0.3 | 0.8 | 0.8 | 0.8 | 1.9 | 6.2 | 7.6 | - | 8.2 |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 |  |  |  |  |  |  |  |  |  |  |  |

# References

- R4-2417082 WF on PC2 and 40MHz CBW for n28, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, CMCC.

- R4-2416227 Requirements for 40MHz CBW in Band n28, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, Skyworks Solutions, Inc.

- R4-2413062 n28 40MHz PC2 PC3 REFSENS, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, The Netherlands, Skyworks Solutions, Inc.

- R4-2405956 Guidelines for PC2 FDD Dual-TX MSD, 3GPP TSG-RAN WG4 Meeting #110bis, Changsha, China, Skyworks Solutions, Inc.

- R4-2419699 1UL HPUE NRCA MSD Simplifications for Rel-19, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions, Inc.

# Annex

Table 3: Band n28 agreed REFSENS vs ideal RB scaled REFSENS from the 3 MHz CBW REFSENS

| CBW (MHz) | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agreed n28 REFSENS (dBm) | -100.2 | -98.5 | -95.5 | -93.5 | -90.8 | -84.2 | -78.5 |  | -66.3 |
| Ideal RB scaled from the agreed 3MHz REFSENS level of -100.2dBm (dBm) | -100.2 | -98.0 | -94.8 | -93.0 | -91.7 | -90.7 | -89.9 |  | -88.6 |
| [Agreed - Ideal (3MHz)] (dB) | 0 | -0.5 | -0.7 | -0.5 | 0.9 | 6.5 | 11.4 |  | 22.3 |
| Ideal RB scaled from the agreed 5MHz REFSENS level of -98.5dBm (dBm) |  | -98.5 | -95.3 | -93.5 | -92.2 | -91.2 | -90.4 |  | -89.1 |
| [Agreed - Ideal (5MHz)] (dB) |  | 0 | -0.2 | 0 | 1.4 | 7.0 | 11.9 |  | 22.8 |

Figure 1: Band n28 40 MHz RSD from WF [1] agreements.

Figure 2: Band n28 30 MHz RSD

Figure 3: Band n28 25 MHz RSD

Figure 4: Band n28 20 MHz RSD

Figure 5: Band n28 15MHz RSD

Figure 6: Band n28 10 MHz RSD. The agreed REFSENS is 0.2 dB lower than the RB scaled REFSENS

Figure 7: Band n28 5 MHz RSD

Figure 8: Band n28 3 MHz RSD