# 3GPP TSG-RAN WG4 Meeting # 113                                                         R4-2419725

# Orlando, USA, 18 – 22 November 2024

# Title:	Input to PRD clause 5.5 and TP on rules for harmonic MSD

# Agenda Item:	9.1.1.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents our views on the structure of the PRD clause 5.5 on MSD [1] and provides a text proposal (TP) to capture the latest rules on specifying the MSD test points due to harmonic interference [2].

# Discussion

The first version of the RAN4 PRD [1] contains a clause dedicated to MSD rules that is currently “TBD”. In this document we present our views on the structure of that clause and provide a TP on rules for 1UL MSD test points, i.e. MSD due to uplink (UL) harmonic, Rx harmonic mixing. We intend to propose rules cross-band isolation at the next meeting.

  - Proposals for the structure of clause 5.5 “MSD”

## At this stage, we only consider capturing rules on MSD test points for carrier-aggregation (CA). Rules for single-band REFSENS may rules at this stage. Our preference for the hierarchy of clause 5.5 is to follow a structure which, by order of descending level captures rules based on a root that separates first the number of DL bands, i.e.

- Intra-band CA = 1 DL band (by definition)

- Inter-band CA

  - 2 DL bands

  - 3 DL bands

## Then within each DL band group, consider expanding the structure by number of UL bands, i.e.:

- Intra-band CA = 1 DL band + 1UL band (by definition) – no level change

- Inter-band CA

  - 2 DL bands

    - 1UL band

    - 2UL bands

  - 3 DL bands

    - 2UL bands

## Then within each UL band group, consider expanding the structure by number of component carrier (#CC) per UL band, i.e.:

- Intra-band CA = 1 DL band

  - 1 CC

  - 2 CCs

- Inter-band CA

  - 2 DL bands

    - 1UL band

      - 1 CC

      - 2 CCs

    - 2UL bands

      - 1CC in each UL band, i.e. 1CC+1CC

      - 1CC in one UL band and 2CCs in the other band, i.e. 1CC+2CCs

  - 3 DL bands

    - 2UL bands

      - 1CC in each UL band, i.e. 1CC+1CC

      - 1CC in one UL band and 2CCs in the other band, i.e. 1CC+2CCs

## and finally, the last level consists in expanding on the number of Tx per CC, hereby covering all HPUE implementations. Details can be further discussed we would like to propose an initial 1UL text proposal for the above-mentioned structure.

## Proposal 1: For clause 5.5 on rules for MSD, focus on rules for CA and consider adopting the following hierarchy. This structure allows future / more complex UL configurations to be accounted for, eg. accommodate more than three UL CCs. The rules for each type of MSD (harmonic interference, cross-band isolation etc..) are to be captured in a sub-level of this hierarchy (see proposal 2 example).

## 5.1 Intra-band CA

## 5.1.1: 1 UL CC

## 5.1.2: 2 UL CCs

## 5.2 Inter-band CA

## 5.2.1 Two DL band

## 5.2.1.1 One UL band

## 5.2.1.1.1 One UL CC

## 5.2.1.1.2 Two UL CCs

## 5.2.1.2 Two UL bands

## 5.2.1.2.1 One UL CC in each band

## 5.2.1.2.2 One UL CC in one band and two UL CCs in the other band

## 5.2.2 Three DL bands

## 5.2.2.1 Two UL bands

## 5.2.2.1.1 One UL CC in each band

## 5.2.2.1.2 One UL CC in one band and two UL CCs in the other band

## FFS whether and how to introduce another sub-level to capture MSD rules specific to the number of Tx to account for new Rel-19 and future Releases power class options.

  - TP on rules for 1UL MSD test points.

With regards to proposal 1, rules for one UL MSD test points are meant to be captured in sub-clause 5.2.1.1.1. Change marks are not used since the clause 5.5 is currently empty and that RAN4 first needs to agree upon a clause structure (eg. proposal 1).

Proposal 2: Consider the following text proposal based on hypothetical agreement of proposal 1

## ---------------------------------------------------- start of TP ----------------------------------------------------

## 5.1 Intra-band CA

## 5.1.1: 1 UL CC

## 5.1.2: 2 UL CCs

## 5.2 Inter-band CA

## 5.2.1 Two DL band

## 5.2.1.1 One UL band

## 5.2.1.1.1 One UL CC

## 5.2.1.1.1.1 Rules MSD due to harmonic interference

## This set of rules apply to both MSD due to uplink harmonic interference and to MSD due to receiver (Rx) 	harmonic mixing.

## 5.2.1.1.1.1.1 Rules for the uplink (UL) band configuration

## 5.2.1.1.1.1.1.1 UL band channel bandwidth (CBW) and SCS configuration

## The UL band should be configured with the smallest UL CBW and the smallest SCS supported by the UL band for 	both the 1st and 2nd MSD test points whenever a 2nd test point is specified.

## 5.2.1.1.1.1.1.2 UL band RB allocation configuration

## For direct-hit MSD test points:

## The RB allocation length “LCRB” scales with the harmonic order according to the following rule for SCS 15kHz / 	SCS 30 kHz, respectively:

- For the UL1/DLx test point, Lcrb=25RB / 12RB,

- For the UL2/DLx test point, Lcrb=12RB / 6RB,

- For the UL3/DLx test point, Lcrb=8RB / 4RB,

- For the UL4/DLx test point, Lcrb=6RB / 3RB,

- For the UL5/DLx test point, Lcrb=5RB / 2RB.

For near-miss MSD test points:

The UL band should be configured with the full RB allocation that can be supported by DFT-s-OFDM waveforms 	(e.g. Lcrb=50RB for CBW 10MHz, SCS 15kHz)

The uplink RB allocation “RBstart” is specified in the form of an equation which is captured in the first applicable 	generic footnote as:

“The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing.”

This equation ensures the configured UL RBs are centered in the middle of the uplink channel.

## 5.2.1.1.1.1.2 Rules for the downlink (DL) band configuration

## 5.2.1.1.1.1.2.1 DL band CBW and number of MSD test points

For direct-hit MSD test points:

A maximum of two test points can be specified:

- A first “mandatory” test point to capture the MSD of the lowest DL band CBW, this test point is mandatory.

- A second “optional” test point to capture the MSD of the highest DL band CBW. Other DL band CBW configurations are not precluded.

For near-miss MSD test points:

- When the DL band may be affected by an UL2/DL1 direct-hit collision, the near-miss MSD test point is not needed. The footnote associated to the UL2/DL1 source of interference informs RAN5 that the DL band may be affected by near-miss interference for which the MSD is not specified.

- When the DL band may not be affected by a direct-hit collision, one near-miss MSD test point may be considered whenever applicable, and the MSD should be specified for the lowest DL band CBW.

## 5.2.1.1.1.1.2.2 Handling of the DL band “new lowest/highest” CBW

- The DL band “lowest” and the “highest” CBW shall not exceed the CBW specified in the BCS configuration tables of clause 5.5A.

- The guidelines of WF [3] on handling of requests which introduce a new lowest or a new highest DL band CBW are the most up-to-date applicable guidelines.

## 5.2.1.1.1.1.3 Rules on specifying the UL/DL carrier frequency “Fc” condition

- Direct-hit UL/DL fc conditions remain unchanged and are captured with equations in dedicated footnotes.

- Near-miss UL/DL fc conditions for UL harmonic interference are different than the conditions for Rx harmonic mixing:

- The UL harmonic interference near-miss MSD test points should be evaluated according to the updated UL/DL fc conditions below:

“The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from  in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively”.

- The Rx harmonic mixing near-miss MSD test points should be evaluated with the following UL/DL fc conditions:

“The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the

highest NR ARFCN of the UL (higher) band.”

## 5.2.1.1.1.1.4 Rules on specifying the UL/DL harmonic order.

- The UL/DL harmonic order is captured using the notation “ULn/DLm” and “direct-hit” or “near-miss”, where “n” and “m” denote respectively the UL and the DL harmonic order.

---------------------------------------------------- End of TP ----------------------------------------------------

# Conclusion

This paper captures our views the clause structure of the RAN4 PRD clause 5.5 on MSD, cf. proposal 1. We tentatively propose to introduce our previous set of rules for MSD due to harmonic interference into the proposal 1 clause structure.

The TP for proposal 2 is not reproduced in this conclusion section for sake of simplicity.

## Proposal 1: For clause 5.5 on rules for MSD, focus on rules for CA and consider adopting the following hierarchy. This structure allows future / more complex UL configurations to be accounted for, eg. accommodate more than three UL CCs. The rules for each type of MSD (harmonic interference, cross-band isolation etc..) are to be captured in a sub-level of this hierarchy (see proposal 2 example).

## 5.1 Intra-band CA

## 5.1.1: 1 UL CC

## 5.1.2: 2 UL CCs

## 5.2 Inter-band CA

## 5.2.1 Two DL band

## 5.2.1.1 One UL band

## 5.2.1.1.1 One UL CC

## 5.2.1.1.2 Two UL CCs

## 5.2.1.2 Two UL bands

## 5.2.1.2.1 One UL CC in each band

## 5.2.1.2.2 One UL CC in one band and two UL CCs in the other band

## 5.2.2 Three DL bands

## 5.2.2.1 Two UL bands

## 5.2.2.1.1 One UL CC in each band

## 5.2.2.1.2 One UL CC in one band and two UL CCs in the other band

## FFS whether and how to introduce another sub-level to capture MSD rules specific to the number of Tx to account for new Rel-19 and future Releases power class options.

# References

- 3GPP RAN4 Permanent Reference Document (PRD) Rules, guidelines and ways of working for introduction of band combinations in NR and LTE (Release 19 and later releases, v0.1.0.

- R4-2416254 RAN4 PRD input on Guidelines for harmonic MSD test points, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, Skyworks Solutions, Inc., Qualcomm France, Nokia.

- R4-2310321 WF on guidelines on checks/specification updates required when BCS4/5 introduces a new smallest or largest CBW, 3GPP TSG-RAN WG4 Meeting #107, Incheon, Skyworks Solutions, Inc.