3GPP TSG-RAN WG4 Meeting#113							         		R4-2419739

Orlando, Florida, USA, 18 – 22 November 2024

Title: 	Inputs to PCmax for PC1.5 intra-band ULCA

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.1.1.2 Intra-band contiguous and non-contiguous UL CA with PC1.5 [NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN#112bis, we provided a preliminary input in [1] and [2] to the required back-off for PC1.5 intra-band contiguous and non-contiguous ULCA based on measurements. Unfortunately, with the limited time between the two meetings, we have not been able to conduct additional measurements to further refine our views on the required MPR. However, based on the data available in [1] and [2] and the discussion in [3], we can address some of the issues listed in way forward [4]. In this contribution, we further elaborate on PCmax and PSD imbalance aspects.

# Discussion

## 2.1 Inputs from way forward

The way forward [3] inputs relevant for PC1.5 non-contiguous intra-band ULCA are copied bellow in italic.

- Keep the following last meeting’s agreement for intra-band non-contiguous ULCA: MPR studies for PC1.5 non-contiguous intra-band ULCA focusses on dualPA architecture with two LO and may account for PSD imbalance with up to 6dB.

  - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB.

- For a non-contiguous intra-band combination with PC1.5 capability supporting dualPA-Architecture

  - For PCMAX,f,c, keep the following last meeting’s agreement, and the detailed wording can be discussed based on the CR in future: For intra-band UL NC CA with dualPA architecture. The upper bound for PCMAX,f,c is 26dBm

  - For PCMAX, FFS

In this contribution we will further elaborate on how PSD imbalance can be accounted for in the MPR and PCmax requirements.

## On PSD imbalance

Based on the measurements presented in Table 2 in [1, 2], if equal power under RB allocation imbalance has too much impact on MPR, accounting for 6dB imbalance in the back-off assessment results in an increased back-off for both contiguous and non-contiguous ULCA as follows:

- For the intra-band contiguous case:

  - Almost no impact on contiguous and non-contiguous inner

  - Up to 0.8dB impact on contiguous outer

  - Up to 2.2dB impact for non-contiguous outer

- For the intra-band non-contiguous case:

  - Up to 1.5dB on -13dBm/MHz IMD3 but for cases where the MPR is already large.

  - Up to 1.8dB on -30dBm/MHz IMD3 but for cases where the MPR is even larger.

Since accounting for 6dB PSD imbalance has a small impact on the allocations that have low MPR we believe is useful to account for it in the MPR evaluation and in the PCmax for dualPA architecture. The allocations with low MPR are:

- Inner contiguous and inner non-contiguous for contiguous ULCA

- Large RB BW > 10MHz for non-contiguous ULCA

Proposal: If less than a 0.5dB impact is confirmed for 6dB imbalance for allocations with low MPR, the PC1.5 intra-band ULCA MPR and dualPA PCmax should account for 6dB imbalance. Allocations (QPSK) with low MPR are allocations such as:

- Inner contiguous for contiguous ULCA.

- Inner non-contiguous for contiguous ULCA.

- Large RB BW > 10MHz for non-contiguous ULCA.

## 2.3 On PC1.5 intra-band ULCA PCmax for dualPA architecture.

Based on the above proposal, PCmax for dualPA should account for up to 6dB PSD imbalance.

For PC1.5 using two Tx there is no impact since PCmax,C = PCmax total = 29dBm independently from any PSD imbalance.

For PC1.5 using dualPA, if we understand that PCmax,C = 26dBm is important for the NW in terms of PHR report, it is not useful for the UE that applies MPR based on the total power.

Based on our work in [3] the following, Ptotal can be achieved for equal PSD or 6dB imbalance where RB1 ≥ RB2:

Ptotal(equalPSD) [dBm] = 26+10*log(1+RB2/RB1)

Ptotal(6dB imbalance) [dBm] = min{29,26+10*log(1+4*RB2/RB1)}

Further, making it generic for any allocation ratio and allowing different SCS in each CC, the following equations are provided for PC1.5 intra-band ULCA with dual PA:

PPowerClass,CA (equalPSD) [dBm] = 26+10*log(1+min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))

PPowerClass,CA (6dB imbalance) [dBm] = min{29,26+10*log(1+4*min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))}

Where RB1 and RB2 are the number of active RBs in CC1 and CC2 respectively and SCS1 and SCS2 are the SCS used in CC1 and CC2 respectively.

Based on the above and allowing up to 6dB PSD imbalance, we make the following proposal to amend the ULCA PCmax equations for PC1.5 ULCA.

Proposal on PC1.5 PCmax accounting for dualPA architecture limitations and allowing up to 6dB PSD imbalance:

For a UE supporting PC1.5 intra-band contiguous or non-contiguous ULCA and:

- Not signalling dualPA IE,

  - PCMAX,c = 29dBm

  - PPowerClass,CA = 29dBm

- Signalling DualPA IE,

  - PCMAX,c = 26dBm

  - PPowerClass,CA = min{29, 26+10*log(1+4*min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))} dBm

  - Where RB1 and RB2 are the number of active RBs in CC1 and CC2 respectively and SCS1 and SCS2 are the SCS used in CC1 and CC2 respectively.

# Conclusion

In this contribution, based on the measurement in [1] and [2] and the PCmax calculations in [3], we make the following proposals.

Proposal: If less than a 0.5dB impact is confirmed for 6dB imbalance for allocations with low MPR, the PC1.5 intra-band ULCA MPR and dualPA PCmax should account for 6dB imbalance. Allocations (QPSK) with low MPR are allocations such as:

- Inner contiguous for contiguous ULCA.

- Inner non-contiguous for contiguous ULCA.

- Large RB BW > 10MHz for non-contiguous ULCA.

Proposal on PC1.5 PCmax accounting for dualPA architecture limitations and allowing up to 6dB PSD imbalance:

For a UE supporting PC1.5 intra-band contiguous or non-contiguous ULCA and:

- Not signalling dualPA IE,

  - PCMAX,c = 29dBm

  - PPowerClass,CA = 29dBm

- Signalling DualPA IE,

  - PCMAX,c = 26dBm

  - PPowerClass,CA = min{29, 26+10*log(1+4*min(RB1*SCS1,RB2*SCS2)/max(RB1*SCS1,RB2*SCS2))} dBm

  - Where RB1 and RB2 are the number of active RBs in CC1 and CC2 respectively and SCS1 and SCS2 are the SCS used in CC1 and CC2 respectively.

# References

[1] R4-2415645 Preliminary input on DualPA PC1.5 non-contiguous ULCA MPR based on measurements, Skyworks Solutions Inc., RAN4#112bis

[2] R4-2415644 Preliminary input on 2Tx PC1.5 contiguous ULCA MPR based on measurements, Skyworks Solutions Inc., RAN4#112bis

[3] R4-2415275 On PC1.5 ULCA PCmax and UE behavior under large PSD imbalance, Skyworks Solutions Inc., RAN4#112bis

[4] R4-2417098 WF on HPUE RF requirements, Samsung, Skyworks, Nokia, RAN4#112bis

R4-2415275 On PC1.5 ULCA PCmax and UE behavior under large PSD imbalance Skyworks Solutions Inc.