3GPP TSG-RAN WG4 Meeting #113	R4-2419744

Orlando, US, November 18 – 22, 2024

Agenda Item:	7.15.3

Source:	MediaTek Inc.

Title:	Discussion on the RRM requirements for fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

In this paper, we continue to discuss the RRM core requirements for achieving fast SCell activation for UE supporting R18 EMR as captured in the WID below [1].

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

For the UE supporting Rel-18 EMR, the UE can be configured to perform RRM measurements in idle/inactive mode and then check the validity of these measurements before reporting them in the connected mode. The validity of Rel-18 EMR can be checked based on whether the measurement is performed within [X] seconds before msg1 transmission for RRC resume/setup request.

In order to utilize Rel-18 EMR reporting for fast SCell activation, RAN4 needs to discuss the required conditions that ensure the applicability of using R18 EMR towards enhancing SCell activation delay.

Proposal 1: RAN4 to discuss the conditions required to ensure the applicability of using R18 EMR reporting to achieve fast SCell activation.

In the last meeting, RAN4 discussed the applicability of fast SCell activation delay requirements as captured in the WF [1].

#### Issue 1-1-1: Applicability of fast SCell activation delay requirements Proposals: - Option 1: (vivo, CATT, CMCC, LGE, Apple, ZTE, MTK) - RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured. - Option 2: (OPPO, Nokia, Samsung, Ericsson) - RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR. - Option 2a: (OPPO) - Consider the following two approaches to support the case: - 1) a new indication of valid measurement results of idle/inactive mode can be introduced, or - 2) reusing the R18 validity check rule with fulfilling a default value of ValidityDuration. - Option 2b: (Samsung) - RAN4 not to include the case when measIdleValidityDuration-r18 is not configured - RAN4 to include the case when measReselectionValidityDuration-r18 is not configured

For Issue 1-1-1, if UE support R18 EMR but is not configured with the validity check, this can result in UE reporting some old measurement results from the IDLE mode to the NW when connection is resumed/setup. Therefore, these results cannot reflect the current status of the SCell to be activated and should not be applicable to fast SCell activation. In addition, according to the WID, it is clear that fast SCell activation is applicable when UE send valid EMR reporting, which in our view, should meet the validity check of R18 EMR and the measurement accuracy as well.

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

Proposal 2: RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured.

One of the main issues that RAN4 need to address is that whether the reported Rel-18 EMR in connected mode is fresh enough to be considered for SCell activation delay enhancement. One scenario can be, for example, the measurements performed in idle/inactive mode are valid (based on validity check of R18 EMR) but happened at the beginning of X window used for validity check, where X value can be very large according to R18 Specs. Then if these measurement results are reported in connected mode after some time, RAN4 need to study whether the report can still be used for SCell activation enhancement. In our view, RAN4 need to introduce a new time window Y to represent the validity of R18 EMR for SCell activation delay reduction.

Figure 1: Fast SCell activation for UE supporting Rel-18 EMR

If the above condition is met, then the unknown SCell can be activated by applying the activation delay used for the known SCell scenario, i.e., significant delay reduction can be achieved with respect to the unknown SCell activation delay. Otherwise, if the above condition is not met, legacy SCell activation shall apply. This can be achieved by updating the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command as it was agreed in the last meeting captured below.

#### Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting Agreement: - RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception. - For FR1, - [Y] equals to [5s] for direct SCell activation - FFS: Y for other SCell activation case. - For FR2, - Reuse the existing value of Y in direct SCell activation - FFS: Y for other SCell activation case.

For the FSS part shown above, the Y value for other SCell activation can reuse the corresponding FR1 and FR2 values defined in clause 8.3.2.

Proposal 3: The Y value for other SCell activation can reuse the corresponding FR1 and FR2 values defined in clause 8.3.2.

RAN4 has also made further agreement for Issue 1-2-1a as shown below.

#### Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting … Agreement: - the SSB detectability applies from the earliest possible eEMR measurement (e.g., the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting). - the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode. For continued discussion in next meeting: - For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured: - For FR1: - use existing SSB based known SCell activation delay requirement with [2] SSB samples. - For FR2: - Option1: The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X] - Option 1 delay components: 1 SSB sample as legacy known case (Apple with short window, OPPO with X=5, MTK, HW) - Option 2: (Ericsson, Nokia) No need to have such side condition in option 1. Need more discussion. - Option 3: use existing SSB based known SCell activation delay requirement with 2 SSB samples and don’t need side condition in option 1 - Option 3 delay components: [2] SSB samples * beam sweeping +1 SSB sample (QC, Apple, OPPO, ZTE, Samsung, VIVO, MTK when the validityduration is configured) - Option 4 (combine option 1 and option 2) delay components:[ K] SSB samples * beam sweeping +1 SSB sample (QC, ZTE, Nokia, HW, CATT) - K = 0 or 1 if when the “validityduration” is smaller than X - K = 1 or 2 otherwise - Option 4a: delay components:[ K] SSB samples * beam sweeping +1 SSB sample (Samsung) - K = 0 or 1 if when the “validityduration” is applied with smaller than X - K = 1 or 2 otherwise - FFS how to test Other proposals: - Proposal 6: For a UE supporting measValidationReportEMR or measValidationReportReselectionMeasurements, the legacy known conditions with deactivated SCell L3 measurement still apply. - Proposal 7: - In case the SCell has more than one actually transmitted SSB, the report should be based on beam level measurement result, i.e. MeasIdleConfig > MeasIdleCarrierNR > BeamMeasConfigIdle-NR > includeBeamMeasurements = true. - Proposal 8: It is feasible to introduce a new dedicated eEMR based fast SCell activation indication - The corresponding MAC-CE command could be provided together with MAC-CE for SCell activation - [Rel-18 EMR based SCell fast activation command] is received [after L3-RSRP reporting via Rel-18 EMR] and no later than the time when UE receives MAC-CE command for TCI activation

For the remaining part of Issue 1-2-1a, it is important to define the requirement when measIdleValidityDuration or measReselectionValidityDuration is configured for both FR1 and FR2.

Proposal 4: The requirements for fast SCell activation delay with R18 EMR include the following conditions:

  - For FR1:

    - use existing SSB based known SCell activation delay requirement with [2] SSB samples when measIdleValidityDuration or measReselectionValidityDuration is configured.

  - For FR2:

    - Use SCell activation delay with [2] SSB samples * beam sweeping +1 SSB samples when measIdleValidityDuration or measReselectionValidityDuration is configured.

# Summary

In this contribution, the following proposals are concluded:

Proposal 1: RAN4 to discuss the conditions required to ensure the applicability of using R18 EMR reporting to achieve fast SCell activation.

Proposal 2: RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured.

Proposal 3: The Y value for other SCell activation can reuse the corresponding FR1 and FR2 values defined in clause 8.3.2.

Proposal 4: The requirements for fast SCell activation delay with R18 EMR include the following conditions:

  - For FR1:

    - use existing SSB based known SCell activation delay requirement with [2] SSB samples when measIdleValidityDuration or measReselectionValidityDuration is configured.

  - For FR2:

    - Use SCell activation delay with [2] SSB samples * beam sweeping +1 SSB samples when measIdleValidityDuration or measReselectionValidityDuration is configured.

# References

- R4-2416854, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT.