3GPP TSG-RAN WG4 Meeting # 113	R4-2419745

Orlando, US, Nov 18 – Nov 22, 2024

Agenda Item: 7.19.3.1

Source: MediaTek Inc.

Title: Discussion on R19 MIMO for UE-initiated/event-driven beam management

Document for: Discussion

# Introduction

According to WF [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WID [1] is divided into two items to be discussed: (1) UE-initiated/event-driven beam management. (2) Other RRM requirements.

The discussion in this paper focus on the “UE-initiated/event-driven beam management”.

# Discussion

In the following sections, we provided our view and discussed sequentially.

- RRM requirement on UE-initiated/event-driven beam management

- Capability on UE-initiated/event-driven beam management

## 2.1 RRM requirement on UE-initiated/event-driven beam management

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay? <Way forward > - Option 1: - If multiple beams are measured with different measurement period, RAN4 to discuss how to consider the total measurement time. - Option 2: - It is proposed to take measurement period defend for Rel-18 LTM as baseline - Option 3: - RAN4 should investigate whether those measurement periods can be reused for defining measurement reporting delay requirements for UE-initiated/event-driven beam management - Option 4: - Use legacy measurement period for TL1-RSRP_Measurement_Period_SSB for SSB based intra-cell , inter-cell, TL1-RSRP_Measurement_Period_CSI-RS for CSI RS based. - Option 4a: - Remove Treport or set to 0 in legacy requirements. - Option 4b: - Remove considerations for HST enhancements/ scenarios - From legacy measurement period, for new beam report further discuss if M is needed or can be removed/assumed 1. - From legacy measurement period, for new beam report further discuss if P can be the same or modified for simplification. - Option 4c: - For FR2, the T_L1_measurement_period is scaled by N. Other options are not precluded. Issue 1-8: CSI-RS based L1-RSRP for inter-cell BM? <Way forward > - Option 1: - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case - Option 2: - For CSI-RS based inter-cell L1-RSRP measurement, since no specific discussion before, RAN4 needs to identify whether additional clarifications on the following points are necessary: - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC; - The prerequisite on L3 measurement on NSC. - Option 3: - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario in FR2. - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. - Other options are not precluded. Issue 1-4: How to consider the time window of triggering event determination in Event triggered measurement reporting? <Way forward > - Option 1: - RAN4 needs to consider the impact of triggering window defined in RAN1. - Option 2: - It is proposed to define requirements for both the case without filtering and the case with filtering (i.e. timer and counter M). - It is proposed that same reporting requirements are applied to the case without filtering and the case with filtering (i.e. timer and counter M) - Option 3: - No additional time-window-based filtering or other filtering should be applied for the L1-RSRP measurements - Option 4: - Discuss the necessity of filtering based on the two trigger-event detection solutions approved in RAN1. - For the basic solution, one-shot measurement is defined by RAN1. No room to consider filtering. - For the optional solution, multi-shot measurement with at least M instance is defined by RAN1. Filtering is feasible. - Option 5: - RAN4 to discuss how to define one Event-2 instance for following Event-2 case: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam - Option 6: - Wait further RAN1 progress. - Other options are not precluded.

Typically, UE perform periodic L1-RSRP measurement for event triggered measurement reporting. When UE met the event condition configured by NW, it will start to send notification to NW about beam reporting is ready to transmit on the later UL resource by either dynamic resource or pre-configured resource. The measured period could be considered as the time duration before UE initiate event beam reporting. Therefore, the legacy measurement period could be reused for SSB and CSI-RS based L1-RSRP measurement since no new behavior on L1-RSRP measurement itself. In addition, there’s no requirement for CSI-RS based L1-RSRP on inter-cell in legacy. To save time, we could postpone the discussion until the baseline requirements are finished.

Proposal 1: Reuse legacy measurement period for event triggered beam reporting.

- SSB based L1-RSRP measurement for both intra-cell and inter-cell.

- CSI-RS based L1-RSRP measurement for intra-cell.

  - Descope CSI-RS based L1-RSRP measurement for Inter-cell case.

The time window of triggering event is based on configuration from NW. E.g., the timer of time window and the event instance counter (M). If event is met for at least M times during time window, UE start to initial event beam reporting to NW. But it doesn’t mandate UE apply filtering on measured samples even if M configured as number larger than 1. As RAN4 aim to define the minimum requirement, the number of samples could be assumed to 1 as baseline.

Proposal 2: RAN4 to consider the number of samples is equal to one in L1-RSRP measurement for event triggered beam reporting.

Issue 1-5: How to consider UL resource in Event triggered measurement reporting? <Way forward > - Option 1: - RAN4 needs to consider the impact of UL resource waiting time for L1 event triggered reporting. - Option 2: - The beam reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. - No need to differentiate mode A and mode B - Option 3: - Report delay is defined as Tfirst-PUCCH or TPUCCH + TDCI + T1. Where TDCI = 0 for Mode B, Time to receive DCI with UL grant in Mode A; T1 = K2 for Mode A; Tfirst-PUSCH or TPUSCH for Mode-B - Option 4: - To determine how to count for the uncertainty in acquiring UL resource for UEIBR, RAN4 waits further conclusion from RAN1 on whether the first PUCCH is SR or not (for Mode-A). - Other options are not precluded. Issue 1-6: Whether to define requirements if end of measurement period is before the UL resource? <Way forward > - Option 1: - RAN4 to discuss whether and how to define requirement when measurement period is shorter than the UL resource waiting time - Other options are not precluded.

From RAN1 agreement, the UL resource for UE beam reporting is different between mode A and mode B. (Mode A: pre-configured UL resource, Mode B: dynamic UL resource). In general, there’s uncertainty time to UL resource when UE is ready to send notification or beam reporting to NW. The uncertainty time for UE to send notification on 1st PUCCH is the same between mode A and mode B since both UL resource are pre-configured on 1st PUCCH. The notification on 1st PUCCH is the earliest time for NW to know UE is ready to send event beam reporting. At least, RAN4 to consider the uncertainty time for event triggered beam reporting on 1st PUCCH. In addition, we don’t need to consider the end of measurement period is before or after UL resource since both cases are regarded as uncertainty time to UL resources.

Proposal 3: RAN4 at least to consider the uncertainty time to UL resource on 1st PUCCH since this is the earliest time for NW to know UE is ready to send event triggered beam reporting.

Issue 1-11: Whether to enhance beam sweeping in event triggered L1 beam reporting? <Way forward > - Option 1: - Discussions on beam sweeping enhancements should be postponed till the baseline requirement for delay and measurement period have taken place - Option 2: - Reuse the current assumption of beam sweeping for requirements for UE/ event triggered beam reporting. No enhancement. - Option 3: - Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting. - Other options are not precluded. Issue 1-12: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? <Way forward > - Option 1: - The existing accuracy requirements for L1 measurements can be applied. No new accuracy requirements. - Option 2: - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline. - For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements. - Option 3: - Discuss the accuracy for the two trigger-event solutions respectively. - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement; - For the optional trigger-event detection, reusethe existing L1-RSRP accuracy requirement. - Other options are not precluded. TS 38.133 #### 9.5.3.3 Aperiodic Reporting Reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively. The UE shall only send aperiodic L1-RSRP measurement reports, if a DCI trigger has been received. After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

The beam sweeping enhancement on multi-RX in Rel-18 is under multi-TRP scenario. In Rel-19, UE-initial event beam reporting is targeting at FR2 and single TRP. Therefore, beam sweeping enhancement is kind of not in the scope in our understanding.

Proposal 4: RAN4 do not consider beam sweeping enhancement on UE event triggered reporting since it is out of scope in this WI.

For the accuracy requirement, RAN4 to reuse legacy requirement since there’s no new measurement metrics except for L1-RSRP for event triggered reporting.

Proposal 5: RAN4 to reuse the legacy L1-RSRP accuracy requirement for UE-initiated/event-driven beam management.

## 2.2 Capability on UE-initiated/event-driven beam management

Issue 1-13: Capabilities for Support of Event Triggering and Reporting Criteria? <Way forward > - Option 1: - Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency. - Other options are not precluded. Issue 1-14: Compatibility of capabilities with LTM <Way forward > - Option 1: - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold. - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency. - Other options are not precluded.

There’re similar issues discussed about the capabilities for event triggered L1 measurement reporting in R19 LTM. In legacy, capabilities for support of event triggered L3 measurement reporting had been specified in section 9.1.4. For event-based reporting, each instance of event, with the same or different event identities, is counted as separate reporting criterion in Table 9.1.4.2-1.

In our understanding, we could follow the similar principle to define the number of events supported for UE-initiated beam reporting in MIMO. Since we’re targeting FR2 and sTRP with intra- and inter-cell beam management, the number of events for UE-initiated beam reporting should be specified for intra-frequency.

Proposal 6: Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency.

Refer to the discussion on R19 LTM, the evaluation method of events configured in mobility and MIMO may be different, so the events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell.

Proposal 7: Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

We prefer to specify a unified capability that indicates the total number of L1 events supported for mobility and MIMO. This allows UE with more flexibility to utilize the whole capability of L1 events compared to share the total number of L1 events to separate UE capability.

Proposal 8: Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

R19 LTM discussion on the capabilities for support of event triggered L1 measurement reporting, the number of events supported shall be defined. Please refer to LTM discussion for more details. 38.133 Table 9.1.4.2-1: Requirements for reporting criteria per measurement category Measurement category; Ecat; Note. Intra-frequency Note 1,2,3,4,5; 9; Events for any one or a combination of intra-frequency SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells. Inter-frequency Note 2,3,4,5; 10; Events for any one or a combination of inter-frequency SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells. Inter-RAT (E-UTRA FDD, E-UTRA TDD) Note 2,4,5; 10; Only applicable for UE with this (inter-RAT) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell.. Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSTD Note 2,4,5; 1; Inter-RAT RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 inter-RAT cell measurements. Only applicable for UE with this (inter-RAT RSTD via LPP [22]) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell.. Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSRP and RSRQ measurements for E-CID Note 2,4,5; 1; Inter-RAT RSRP and RSRQ measurements for E-CID reported to E-SMLC via LPP [22]. One report capable of at least in total 10 inter-RAT RSRP and RSRQ measurements. Applicable to UE capable of reporting inter-RAT RSRP and RSRQ to E-SMLC via LPP. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell.. Intra-frequency RSSI and channel occupancy measurements with CCA Note 1,2,3; 1; One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA.. Inter-frequency RSSI and channel occupancy measurements with CCA Note 2,3; 1; One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA.. Intra-frequency SSB-based measurements for NR E-CID Note 1,2,3,4,5; 1; Intra-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 9 intra-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP.. Intra-frequency CSI-RS based measurements for NR E-CID Note 1,2,3,4,5; 1; Intra-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 9 intra-frequency CSI-RSRP and/or CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16.. Inter-frequency SSB-based measurements for NR E-CID Note 2,3,4,5; 1; Inter-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 10 inter-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP.. Inter-frequency CSI-RS based measurements for NR E-CID Note 2,3,4,5; 1; Inter-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 10 inter-frequency CSI-RSRP and CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16.. DL RSTD Note 2,4,5; 1; DL RSTD measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-TDOA-MeasCapability, indicated via LPP [34]) DL RSTD measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for DL-TDOA. Only applicable for UE capable of reporting measurements for DL-TDOA to LMF via LPP [34].. UE Rx-Tx Note 2,4,5; 1; UE Rx-Tx measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for multi-RTT) UE Rx-Tx measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for multi-RTT. Only applicable for UE capable of reporting measurements for multi-RTT to LMF via LPP [34].. DL PRS-RSRP Note 2,4,5; 1; DL PRS-RSRP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34].. DL PRS-RSRPP Note 2,4,5; 1; DL PRS-RSRPP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRPP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34].. SRS-RSRP Note 2,3,4,5; 1; SRS-RSRP measurement reporting for CLI; 1 report capable of up to 32 SRS resources measurements. Only applicable for UE supporting cli-SRS-RSRP-Meas-r16.. CLI-RSSI Note 2,3,4,5; 1; CLI-RSSI measurement reporting for CLI; 1 report capable of up to 64 CLi-RSSI resources measurements. Only applicable for UE supporting cli-RSSI-Meas-r16.. NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode..

# Summary

In this paper, the discussion of R19 MIMO is provided. We have the following proposals:

Proposal 1: Reuse legacy measurement period for event triggered beam reporting.

- SSB based L1-RSRP measurement for both intra-cell and inter-cell.

- CSI-RS based L1-RSRP measurement for intra-cell only.

  - Descope CSI-RS based L1-RSRP measurement for Inter-cell case.

Proposal 2: RAN4 to consider the number of samples is equal to one in L1-RSRP measurement for event triggered beam reporting.

Proposal 3: RAN4 at least to consider the uncertainty time to UL resource on 1st PUCCH since this is the earliest time for NW to know UE is ready to send event triggered beam reporting.

Proposal 4: RAN4 do not consider beam sweeping enhancement on UE event triggered reporting since it is out of scope in this WI.

Proposal 5: RAN4 to reuse the legacy L1-RSRP accuracy requirement for UE-initiated/event-driven beam management.

Proposal 6: Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency.

Proposal 7: Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

Proposal 8: Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

# Reference

[1] RP-233090 New WID - Rel-19 MIMO

[2], WF on RRM requirements for NR_MIMO_Ph5_Part1 (Samsung)