3GPP TSG-RAN WG4 Meeting #113		R4-2419748

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	6.15.2.2

Source: 		Qualcomm Incorporated

Title: 			Enhancement on FR2 CSSF

Document for:	Discussion

# 1.	Introduction

In this paper, we present our view on the following aspects:

- CSSF for outside MG in SA mode

# 2.  Past Agreements

Following agreements happened during the last two meetings [1] [2].

### Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF Agreement in online session: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption” ### Issue 2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF Agreement: - RAN4 to consider following CA/DC mode for L3 measurement delay reduction by optimizing CSSFoutside_gap,i - Final applicability of the solutions can be FFS - EN-DC - EN-DC with FR2 only intra band CA - EN-DC with FR2 only inter band CA - EN-DC with FR1 +FR2 CA (FR1 PSCell) - EN-DC with FR1 +FR2 CA (FR2 PSCell) - NE-DC - NE-DC with FR2 only intra band CA - NE-DC with FR2 only inter band CA - NE-DC with FR1 +FR2 CA (FR1 PCell) - SA - FR1+FR2 CA (FR1 PCell) - FR1+FR2 CA (FR2 PCell) - FR2 only intra-band CA - FR2 only inter-band CA - NR-DC - FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF Agreement in online session: On top of the previous agreement on the baseline assumption of 2 searchers, for 3 searchers: - Discuss the applicable scenario(s) and/or solution(s) based on 3 searchers. Decide whether or not to define the requirement later. Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC Candidate options for further discussion: - Option 1: UE can measure one serving CC per band if multiple serving CCs are in the same band. For the CCs to be measurement, further discuss option 1A and option 1B - Option 1A: Reuse the legacy principle for neighboring cell measurement. - Option 1B: Based on network indication - The other CCs (not measured based on network indication) need not to be measured and reported. Network assume the same measurement results on the measured CCs and unmeasured CCs in the same band. - Option 2: UE can change the searcher occupancy ratio of measurement among the measured CCs - Option 2A: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions For 3 searchers: In addition to option 1 and 2 - Option 3: consider to use 3 searchers for CSSF reduction. - Depending on issue 2-4.

# 3. 	Discussion

## Impact of Three Searchers

RAN4 agreed to discuss the solutions regarding three searchers. The use of three searchers can reduce L3 measurement delay significantly.  

Table1 shows the CSSF in FR1 + FR2 CA (FR2 PCell) scenario of SA mode assuming two searchers. Table2 shows the CSSF in same FR1 + FR2 CA scenarios of SA mode assuming three searchers. Both these tables assume the absence of CSI-RS based L3 configuration.

Table 1: CSSF in FR1 + FR2 CA scenarios of SA mode (two searchers)

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 +FR2 CA (FR2 PCell)Note 1 | N/A | NSCC_SSB + Y | 1 | N/A | NSCC_SSB + Y | NSCC_SSB + Y |
| NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. Note: CSI-RS based L3 measurement is not configured in PCC and SCC. |  |  |  |  |  |  |

Table 2: CSSF in FR1 + FR2 CA (FR2 PCell) of SA mode (with one FR1 + two FR2 searchers)

| Scenario |  | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 +FR2 CA (FR2 PCell)Note 1 | N/A |  | NSCC_SSB_FR1 + Y_FR1 | 1 | N/A | NSCC_SSB_FR2 + Y_FR2-1 | NSCC_SSB_FR2 + Y_FR2-1 |
| - NSCC_SSB_FR1 = Number of configured SCell(s) in FR1 with only SSB based L3 measurement configured, which is measured without MG. - NSCC_SSB_FR2 = Number of configured SCell(s) in FR2 with only SSB based L3 measurement configured, which is measured without MG. - Y_FR1 is the number of configured inter-frequency FR1 MOs without MG that are being measured outside of MG; otherwise, it is 0. - Y_FR2 is the number of configured inter-frequency FR2 MOs without MG that are being measured outside of MG; otherwise, it is 0. Note: CSI-RS based L3 measurement is not configured in PCC and SCC. |  |  |  |  |  |  |  |

Let’s assume, NSCC_SSB_FR1 = NSCC_SSB_FR2 = 0.5 * NSCC_SSB and Y_FR1 = Y_FR2 = 0.5 * Y.

In that case, the CSSF for FR1 SCC, FR2 SCC and FR2 inter-freq MOs without measurement gap in one FR1 + two FR2 searcher scenario will be roughly half of that of existing legacy scenario.

That means, the assumption of one FR1 + two FR2 searchers will reduce the L3 delay of measuring SCCs and inter-freq MOs without measurement gap by approximately a factor of 2. Similarly, two FR1 + one FR2 and three FR2 searcher assumptions will significantly reduce the L3 delay of inter-freq measurement without measurement gap.

Observation 1: The assumption of three searchers significantly reduces the L3 delay of measuring FR2 SCCs and inter-freq  MOs without measurement gap.

- For example, assuming the number of SCCs and inter-freq MOs to be equally distributed in an FR1 + FR2 CA (FR2 Pcell) scenario, the assumption of one FR1 + two FR2 searchers will reduce the L3 delay of measuring SCCs and inter-freq MOs without measurement gap by approximately a factor of 2.

- The CSSF table in FR1 + FR2 CA (FR2 PCell) takes the form of Table 3 with and without three searcher assumption.

Table 3: CSSF in FR1 + FR2 CA (FR2 PCell) Scenario

| # of searchers |  | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Two searchers across FR1 & FR2 | N/A |  | NSCC_SSB + Y | 1 | N/A | NSCC_SSB + Y | NSCC_SSB + Y |
| One FR1 + Two FR2 searchers | N/A |  | Ceil(0.5 * (NSCC_SSB + Y)) | 1 | N/A | Ceil(0.5 * (NSCC_SSB + Y)) -1 | Ceil(0.5 * (NSCC_SSB + Y)) -1 |
| NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NSCC_SSB is equally distributed between FR1 and FR2. Y is equally distributed between FR1 and FR2, too. Note: CSI-RS based L3 measurement is not configured in PCC and SCC. |  |  |  |  |  |  |  |

As [3] shows, the processing capability of UE already matches that of three searchers in certain legacy scenarios. For example, when one UE measures within gap in one frequency range and another UE measures without gap in another frequency range, legacy rules require UE to apply three searchers.

Observation 2: When one UE measures within gap in one frequency range and another UE measures without gap in another frequency range, legacy rules require UE to apply three searchers.

The complexity associated with supporting three searchers in FR2 is greater than that of two FR2 + one FR1 searchers. Some UE might be able to support two FR2 + one FR1 searchers but not three FR2 searchers. Hence, it makes sense to allow to indicate whether it supports three FR2 searchers or two FR2 plus one FR1 searchers.

Observation 3:  Some UE might be able to support two FR2 plus one FR1 searchers but not three FR2 searchers.

Proposal 1: RAN4 introduces a new optional capability to allow UE to signal support of three searchers for CSSF enhancement.

- Note: UE can indicate the support of different levels of three searchers in the new capability:

  - Indication 00: three searchers are not supported.

  - Indication 01: three searchers are supported, in total. However, at most two searchers are supported in FR2, i.e., UE supports combination of {one FR1 + two FR2 searchers, two FR1 + one FR2 searchers, three FR1 searchers}

  - Indication 10: UE can support three searchers in total and in FR2, too. That means, UE supports the combination of {three FR2 searchers, one FR1 + two FR2 searchers, two FR1 + one FR2 searchers, three FR1 searchers}

# 4.	Conclusion

Observation 1: The assumption of three searchers significantly reduces the L3 delay of measuring FR2 SCCs and inter-freq  MOs without measurement gap.

- For example, assuming the number of SCCs and inter-freq MOs to be equally distributed in an FR1 + FR2 CA (FR2 Pcell) scenario, the assumption of one FR1 + two FR2 searchers will reduce the L3 delay of measuring SCCs and inter-freq MOs without measurement gap by approximately a factor of 2.

- The CSSF table in FR1 + FR2 CA (FR2 PCell) takes the form of Table 3 with and without three searcher assumption.

Table 3: CSSF in FR1 + FR2 CA (FR2 PCell) Scenario

| # of searchers |  | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Two searchers across FR1 & FR2 | N/A |  | NSCC_SSB + Y | 1 | N/A | NSCC_SSB + Y | NSCC_SSB + Y |
| One FR1 + Two FR2 searchers | N/A |  | Ceil(0.5 * (NSCC_SSB + Y)) | 1 | N/A | Ceil(0.5 * (NSCC_SSB + Y)) -1 | Ceil(0.5 * (NSCC_SSB + Y)) -1 |
| NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NSCC_SSB is equally distributed between FR1 and FR2. Y is equally distributed between FR1 and FR2, too. Note: CSI-RS based L3 measurement is not configured in PCC and SCC. |  |  |  |  |  |  |  |

Observation 2: When one UE measures within gap in one frequency range and another UE measures without gap in another frequency range, legacy rules require UE to apply three searchers.

Observation 3:  Some UE might be able to support two FR2 plus one FR1 searchers but not three FR2 searchers.

Proposal 1: RAN4 introduces a new optional capability to allow UE to signal support of three searchers for CSSF enhancement.

- Note: UE can indicate the support of different levels of three searchers in the new capability:

  - Indication 00: three searchers are not supported.

  - Indication 01: three searchers are supported, in total. However, at most two searchers are supported in FR2, i.e., UE supports combination of {one FR1 + two FR2 searchers, two FR1 + one FR2 searchers, three FR1 searchers}

  - Indication 10: UE can support three searchers in total and in FR2, too. That means, UE supports the combination of {three FR2 searchers, one FR1 + two FR2 searchers, two FR1 + one FR2 searchers, three FR1 searchers}

# 5.  Reference

[1] R4-2414026, Way Forward for NR_RRM_Ph5, Apple, RAN4 #112

[2] R4-2416853, WF on RRM requirements for NR_RRM_Ph5_Part1, RAN4 #112bis