3GPP TSG-RAN WG4 Meeting #113	R4-2419765

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.25.2

Source: 	Qualcomm Incorporated

Title: 		RRM core requirements for XR enhancements

Document for:	Discussion

# Introduction

For release 19, RAN4 started working on the new work item on XR enhancements (Phase 3) for NR where the following WID [1] objective is expected to have impact on RRM requirements:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

RAN1 has been discussing mechanisms to enable transmission/reception in gaps/restrictions that are caused by RRM measurements and have reached the following working assumption as conveyed in the LS [2]:

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

In this contribution paper, we discuss the impact of the working assumption made by RAN1, on RRM requirements and propose solutions to address them.

# Impact on RRM requirements due to MG deactivation

Firstly, although RAN1 has agreed upon the dynamic indication to enable Tx/Rx in particular gaps, RAN2 may further consider semi-static indication schemes as well, e.g., RRC based MG deactivation pattern. We understand that due to unpredictable nature of the XR data, dynamic indication is better suited from signaling perspective but at the same time, from UE implementation perspective, it may be quite challenging to deactivate a single MG occasion as this needs extensive processing on the modem SW and RF procedures. RRC/MAC-CE based semi-static indications enables the UE to handle MG deactivations in an efficient manner.

Proposal 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC or MAC-CE based signaling as well. Send LS to RAN2 and RAN1.

Coming back to the DCI based dynamic indication of MG deactivation, few key things to consider in RAN4 for RRM requirement specification are:

- Minimum time offset(s) X between indication to skip and skipped measurement occasion

- Measurement occasions that can be deactivated

- Impact on measurement delay/accuracy requirements

For the first issue, i.e., the minimum offset X between the indication to skip and the measurement occasion to be skipped, we think we can use DCI based pre-MG activation/deactivation delay requirements, i.e., X = 5 ms. Since it’s a similar procedure and this timeline was actively discussed during pre-MG, we don’t see a need to repeat the discussion and can directly use the delay values.

Proposal 2: Following DCI-based pre-configured MG activation/deactivation delay, specify the minimum offset X between the indication to skip and the measurement occasion to be skipped as 5 ms.

Furthermore, it may so happen that the network sends a DCI bit ‘1’ indicating the cancellation of a particular MG occasion and then later, but before the concerned MG occasion, sends another DCI bit ‘0’ negating the cancellation of the same MG occasion. This may result in unexpected UE behaviour. So to avoid this, UE simply ignores any DCI received after the first DCI indicating ‘1’ for a particular MG occasion.

Proposal 3: UE ignores any further DCI indication received after the first DCI bit ‘1’ indicating cancellation of a particular MG occasion.

RAN4 already started an initial discussion on the second issue, i.e., the measurement occasions that can be deactivated. There are three main types of measurements considered here

- L3 measurements with gaps

- L3 measurements outside gaps

- L1 measurements

Among these, L3 measurements with gaps are already agreed to be considered for the DCI based deactivation since MGs cause long interruption and may significantly delay the XR traffic. L3 measurements outside gaps and L1 measurements cause scheduling restrictions and it is being discussed whether such measurements can be cancelled by the DCI.

Most of the L1 measurements cause scheduling restrictions on a symbol level (typically SSB symbols and additional one or two symbols). Since these restrictions are very small, deactivation of such measurements may not bring much gains but will complicate the specification work considering various requirements and scenarios. Furthermore, the L1 measurements are performed with the periodicity of the RS. So receiving a separate DCI for each of these occasions cause a significant overhead on the UE processing bringing minimal benefits. With limited TU budget, RAN4 may not have sufficient time to finish all the work. So, considering the marginal gains that comes with L1 measurement deactivation and limited TUs, we think deactivation of L1 measurements can be deprioritized.

On the similar lines, measurement outside gaps are performed within SMTCs and the following scheduling restrictions are defined

- On SSB symbols to be measured and 1 data symbol before and after SSB​

  - FR1 with support of simultaneousRxDataSSB-DiffNumerology ​

  - FR1 without support of simultaneousRxDataSSB-DiffNumerology with deriveSSB_IndexFromCell enabled​

  - FR2 with deriveSSB_IndexFromCell enabled​

- On all symbols within the SMTC occasion​

  - FR1 without support of simultaneousRxDataSSB-DiffNumerology with deriveSSB_IndexFromCell disabled​

  - FR2 with deriveSSB_IndexFromCell disabled​

Similar to the scheduling restrictions for L1 measurements, measurement outside gaps also specify scheduling restrictions on a symbol level for most of the cases and the only case where we have scheduling restrictions on the entire SMTC is when the UE doesn’t support simultaneousRxDataSSB-DiffNumerology and/or deriveSSB_IndexFromCell is disabled​. Again, very limited use-case and almost no benefit in cancelling such measurements.

Proposal 4: Dynamic signaling to enable Tx/Rx doesn’t apply to scheduling restrictions caused by L1 measurements.

Proposal 5: Dynamic signaling to enable Tx/Rx doesn’t apply to scheduling restrictions caused by L3 measurements without gaps.

Moving on to the L3 measurements, most of the RRM measurement requirements specified in TS 38.133, such as PSS/SSS detection and SSB index identification period and measurement period, are specified as multiple of measurement gap repetition period (MGRP). Deactivation of any MG during these periods would result in UE not having sufficient measurement samples to meet the performance requirements related to delay and accuracy. One simple way to handle this would be to extend the delay requirements based on the number of deactivated MGs. A similar methodology was also followed during NR-U, where the delays were extended based on the number of LBT failures. In our opinion, from RRM requirements perspective, MG deactivation for XR can be handled in a similar manner.

Proposal 6: RAN4 to consider extending the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network.

- For DCI based dynamic indication, follow the NR-U approach to extend the L3 delays by the number of MGs deactivated during the period.

The next thing to consider is the type of MGs. RAN4 considers multiple measurement gaps such as per UE or per FR MGs, where one or more gaps can be configured as pre-configured MG or NCSG or MUSIM gaps. In some cases, multiple MGs can be activated concurrently where some gap occasions may collide in time. RAN4 already specifies mechanisms to handle such collisions in TS 38.133. We propose to apply the dynamic indication to enable Tx/Rx in a particular gap/restriction that are caused by RRM measurements after the overlapping is resolved. This introduces less specification impacts and UE implementation difficulties than jointly applying the overlapping resolution and indication to enable Tx/Rx in a particular gap/restriction that are caused by RRM measurements.

Proposal 7: When overlapping between gaps/restrictions that are caused by RRM measurements occurs, the dynamic indication to enable Tx/Rx in a particular gap/restriction that are caused by RRM measurements is applied after the overlapping is resolved.

# UAI aspects

The next issue to discuss is related to the UE assistance information (UAI) for measurement gap deactivation.

During RAN1#117, RAN1 discussed whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. Multiple UAI related to measurements occasions and related to channel conditions, traffic, UE mobility etc were discussed and reached a conclusion that RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. However, there was no consensus in RAN1 to support UE assistance information related to measurements occasions. Finally, RAN1 sent an LS [3], asking RAN4 to decide whether or not to introduce any UE assistance information related to measurement occasions.

Although the motivation behind deactivating MGs is quite clear, but the importance of MGs also needs to be considered. In some cases, the network may deactivate excessive number of measurement gaps leading to very long cell detection/identification and measurement periods. Since, it’s up-to UE implementation which MG occasion it uses for each frequency layer/cell to be measured, RAN4 requirements will have to extended for all frequency layers to be measured. Some UEs may not want to track every cell for an extended period of time as it may lead to use of stale measurement samples which may degrade performance accuracy as well as higher memory requirements to store samples for a longer period. Consequentially, extremely long measurement periods will have a negative impact on UE mobility performance. So, we think it’d be beneficial for the UE to transmit some UAI to assist network in configuring reasonable number of MG deactivations to ensure minimum impact on RRM performance. UAI such as number of consecutive MGs that can be skipped, maximum number of MGs that can be skipped etc are a good starting point for the discussion. In addition to that, UE may also indicate what type of MG (per UE/per FR/pre-MG/MUSIM gaps etc.) can be deactivated by the network.

Proposal 8: RAN4 to consider introducing UAI to indicate one or more of the following aspects for MG deactivation

- Maximum number of consecutive MGs that can be skipped

- Maximum number of MGs that can be skipped within a duration

- MG type that can be skipped

# Conclusion

Proposal 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1.

Proposal 2: Following DCI-based pre-configured MG activation/deactivation delay, specify the minimum offset X between the indication to skip and the measurement occasion to be skipped as 5 ms.

Proposal 3: UE ignores any further DCI indication received after the first DCI bit ‘1’ indicating cancellation of a particular MG occasion.

Proposal 4: Dynamic signaling to enable Tx/Rx doesn’t apply to scheduling restrictions caused by L1 measurements.

Proposal 5: Dynamic signaling to enable Tx/Rx doesn’t apply to scheduling restrictions caused by L3 measurements without gaps.

Proposal 6: RAN4 to consider extending the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network.

- For DCI based dynamic indication, follow the NR-U approach to extend the L3 delays by the number of MGs deactivated during the period.

Proposal 7: When overlapping between gaps/restrictions that are caused by RRM measurements occurs, the dynamic indication to enable Tx/Rx in a particular gap/restriction that are caused by RRM measurements is applied after the overlapping is resolved.

Proposal 8: RAN4 to consider introducing UAI to indicate one or more of the following aspects for MG deactivation

- Maximum number of consecutive MGs that can be skipped

- Maximum number of MGs that can be skipped within a duration

- MG type that can be skipped

# References

[1] 	RP-240791, “Revised WID on XR (eXtended Reality) for NR Phase 3”, Nokia, RAN#103

[2] R4-2407561, “LS on gaps/restrictions that are caused by RRM measurements”, Nokia, RAN1#118

[3] R4-2411007, “LS on UE assistance information”, Nokia, RAN4#112