3GPP TSG-RAN WG4 Meeting #112bis	R4-2419766

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.20.3

Source: 	Qualcomm Incorporated

Title: 		RRM core requirements for SBFD

Document for:	Discussion

# Introduction

For release 19 work item on Evolution of NR duplex operation: Sub-band full duplex (SBFD), the main objective is to specify the sub-band full duplex operation. In addition to that, enhancements to CLI handling scheme is another WID [1] objective that is expected to have impact on RRM requirements. RAN4 has already started discussions on the RRM requirements and reached a few high-level agreements [2,3] in the last meetings. In this paper, we further discuss various issues related to SBFD operation in general as well L1-CLI requirements.

# L1 based UE-to-UE CLI measurement requirements

As we know, RAN1 has agreed to define L1-SRS-RSRP and L1-CLI-RSSI as two new report quantities and is currently discussing the following methods to specify measurement resources:

RAN1 has agreed to support Method#1 and Method#2 for L1-CLI-RSSI and L1-SRS-RSRP respectively, while Method#3 is still under discussion. So, RAN4 can start the initial discussion on RRM requirements specification based on Method#1 and Method#2.

RAN4 can consider specifying one or more of the following requirements for new report quantities:

- Measurement capability

- Measurement side conditions

- Measurement period

- Measurement accuracies

- Measurement restrictions

- Scheduling restrictions

For measurement capability, similar to R16 CLI requirements, the L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE, so that the UE is not required to perform any RF retuning for such measurements.

Proposal 1: The L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE.

Scheduling restrictions requirements need to be defined but they depend on the timing offset between the DL timing and SRS arrival timing. In the last meeting, RAN4 agreed to reuse the R16 requirements on timing offset between the DL timing and the SRS arrival timing. With that, it’s straightforward to reuse the scheduling restriction requirements from Rel 16.

Proposal 2: Scheduling restriction requirements defined for R16 CLI also apply to R19 L1 CLI measurements

Some companies also brought up whether we need to define measurement restrictions for L1 CLI measurements. In our understanding, Rel-16 CLI measurements were considered as best effort measurements and UE mobility should not be affected with such measurements. With that assumption, RAN4 didn’t define any measurement restrictions or rules to handle the scenario when the CLI measurement resources partially or fully overlap with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. We think similar principles can be followed for L1 CLI measurements as well. Furthermore, there would be a huge specification impact of considering all collision scenarios and it’ll be hard to complete it with the limited WI TUs allocated during this release.

Proposal 3: If the L1-SRS-RSRP and/or L1-CLI-RSSI resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, corresponding measurement delay requirements are not specified.

For QCL assumptions, Rx beam assumptions and the number of L1 CLI resources to be monitored, it’s best to wait for more agreements from RAN1.

Moving on to the side conditions and corresponding measurement delays and accuracy requirements, we think the R16 SRS configurations can be assumed for SBFD as well, but the side conditions on SRS Es/Iot need to be re-evaluated. RAN4 has already sent an LS to RAN1 asking whether the CLI measurements are applicable only to intra-cell scenario or inter-cell scenario as well. In our opinion, if these measurements are intra-cell only, then RAN4 should consider higher values for the SRS Es/Iot values as compared to 1db value specified for R16 SRS-RSRP measurements. The reason is that low SNR of the SRS will cause very little leakage to the DL sub-bands and hence may not cause any interference to the DL.

Proposal 4: Reuse R16 assumptions on SRS configuration for L1 CLI measurements.

Proposal 5: RAN4 to consider higher side conditions on SRS Es/Iot for L1-SRS-RSRP measurements as compared to R16 L3 SRS-RSRP measurements (>1db). Exact value FFS.

For measurement delays, R16 CLI RSSI measurements are reported as single RSSI sample for each measurement resource configured for CLI-RSSI measurement within the configured measurement periodicity. We think this could directly apply to L1-CLI-RSSI measurements as well and no need to consider multiple samples for L1 RSSI measurements.

Proposal 6: Reuse the R16 CLI-RSSI measurement delay requirements for L1-CLI-RSSI.

# SBFD operation aspects

RAN4 has already agreed that there’s no impact to SSB based serving cell measurements since the SSB transmission is prioritized over UL. However, during the last meeting, RAN4 discussed whether the neighbour cell measurements are also prioritized and identified the following two options:

  - Down-select one of the two options:

    - Option 1:

      - As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission.

      - This can be revisited if it proves to be conflicting with further RAN1 agreements.

    - Option 2:

      - For inter-frequency measurement without gap, UL transmission is prioritized.

      - Follow option 1 in other cases.

There are two main aspects to consider here

- Whether the neighbor cell SSB is being transmitted or not

- Whether the neighbor cell measurements are prioritized over UL transmission

From SSB transmission perspective, the same assumption on SSB transmission should apply to all SBFD cells, i.e., the SSB transmission is prioritized. So, there is no question of whether the neighbor cell is transmitting the SSBs or not. From the measurement perspective, we think neighbor cell measurements are very critical to mobility performance and should not be compromised. So, similar to serving cell measurements, neighbor cell measurements should also have higher priority over UL transmission in SBFD slots/symbols irrespective of whether the measurements are done within gaps or not. So, we support option 1.

Proposal 7: As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission

# Conclusion

Proposal 1: The L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE.

Proposal 2: Scheduling restriction requirements defined for R16 CLI also apply to R19 L1 CLI measurements

Proposal 3: If the L1-SRS-RSRP and/or L1-CLI-RSSI resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, corresponding measurement delay requirements are not specified.

Proposal 4: Reuse R16 assumptions on SRS configuration for L1 CLI measurements.

Proposal 5: RAN4 to consider higher side conditions on SRS Es/Iot for L1-SRS-RSRP measurements as compared to R16 L3 SRS-RSRP measurements (>1db). Exact value FFS.

Proposal 6: Reuse the R16 CLI-RSSI measurement delay requirements for L1-CLI-RSSI.

Proposal 7: As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission

# References

[1] 	RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#104

[2] R4-2413900, “WF on SBFD RRM”, Huawei, RAN4#112

[3] R4-2416858, “WF on SBFD RRM”, Huawei, RAN4#112bis