3GPP TSG RAN WG4 Meeting #113		R4-2419769

Orlando, Florida, USA, November 18th – 22nd, 2024

Agenda item:	10

Source:	T-Mobile USA

Title:	Motivation for an Irregular Channel Bandwidth Work Item in Rel-19

WI/SI:

Release:	Rel-19

Document for:	Information

# 1	Introduction

In September of 2019, Sprint and other operators proposed a new Work Item for the creation of NR band n26 which included a 7 MHz channel BW [1]. During the discussion of the new WID, concern was raised that if one new channel bandwidth was added, there would be requests from operators for every channel bandwidth from 1 MHz to 100 MHz, which would greatly increase the design, testing and certification time and resources. Due to concerns about adding a new channel bandwidth, the new WID was approved without the 7 MHz channel bandwidth [2].

To try to find a solution for 7 MHz and other irregular channel bandwidths a new Study Item was approved to study “Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths.” [3] The Study Item began in Rel-17 and was eventually completed in Rel-18 and the resulting Technical Report is TR 38.844 [4].

Since that time, 6, 7 and 8 MHz channel bandwidths have been added for NR broadcasters to use. Also, 3 MHz channel BWs have been added, primarily at the request of railroad operators. Yet, we sill don’t have an standardized NR mechanism for efficient use of licensed mobile spectrum.

This document discusses the motivation for a new Work Item for Irregular Channel Bandwidths.

# 2	Discussion

## 2.1	Outcome of the irregular channel bandwidth study item

During the Study Item, the following channel bandwidths were requested and documented in TR 38.844:

Table 4-1: Summary of operators’ input for irregular channel bandwidth

| Band (s) | Channel Bandwidth(s) |
| --- | --- |
| n5 | 7, 11 MHz |
| n12, n85 | 6, 12 MHz |
| n26 | 7 MHz |
| n28 | 13 MHz |
| n29 | 6, 11 MHz |

In addition, 3 MHz was requested for n26 (and has subsequently been standardized) and 13 and 33 MHz were requested for n28

The following solutions were proposed in the Study Item:

- Next wider channel bandwidth with PRB blanking

- Overlapping CBWs from the network perspective

- Combined UE CBW

- Overlapping CA

Each of these have certain pros and cons, summarized in Table 7.2.1 from 38.844 [4].

Table 7.2.1. Comparison of different schemes

| Comparison Criteria | Overlapping CA (two cells) – described in Section 6.2.3 | Combined UE CBW (One cell) – described in Section 6.2.2 | Overlapping UE CBW – described in Section 6.2.1 | Larger CBW (one cell) – described in Section 6.1 |
| --- | --- | --- | --- | --- |
| Regulatory requirement | No issue | No issue | No issue | Potential issue on the BS side, gNB filters will be needed depending on BS implementation. |
| UE performance degradation relative to minimum requirements | Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. | Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. | No issue | UE ACS and in-band blocking degradation. Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. |
| gNB complexity | gNB has to support CA and schedule the data without collision in the two CC’s overlapping PRBs | gNB has to support splitting the signal into 2 RF carriers with a predefined phase relationship | gNB has to support the irregular channel BW (can also be implemented through RF combining of 2 channels) | No changes needed if the BS can meet regulatory requirements with the RF front end of the wider channel BW. Otherwise, gNB has to support the irregular channel BW |
| UE complexity | UE has to support intra-band NC CA. 1 less CC can be supported when irregular BW is used in combination from other bands, or more total CCs needed. | UE has to support RF architecture as in intra-band NC CA. Needs new capability to aggregate 2 RF channels in baseband. Complexity higher than CA because the baseband will need a new "combiner" module. 1 less CC can be supported when irregular BW is used in combination from other bands, or more total CCs needed. | No changes needed, supported by legacy UEs | No changes needed, supported by legacy UE |
| UE throughput | UEs supporting the feature can use the entire spectrum allocation, legacy UEs can use an already supported channel BW | UEs supporting the feature can use the entire spectrum allocation, legacy UEs can use an already supported channel BW | UE throughput based on existing channel BWs (5MHz for holdings <10MHz, 10MHz for holdings <15MHz, etc) | UE throughput based on how many RBs can be used |
| Spectral utilization | Channel edge guardband based on the aggregated channel BW (5MHz for <10MHz, 10MHz for <15MHz, etc), 2 SSBs are needed | Channel edge guardband based on the aggregated RF carrier BW (5MHz for <10MHz, 10MHz for <15MHz, etc), single SSB needed | Channel edge guardband based on the actual holding (can be same as Overlapping CA), 2 SSBs are needed | Depends on the usable number of RBs, single SSB needed |
| Cell Spectral utilization | Entire spectrum holding can be used even only with legacy UEs | Entire spectrum holding can be used even only with legacy UEs for some scenarios depending on whether a single SSB can be used to configure legacy channels at both edges of the spectrum holding. Otherwise, entire spectrum can be used only by new UEs, all legacy UEs have to use the same regular channel BW part of the spectrum holding. | Entire spectrum holding can be used even only with legacy UEs | Entire spectrum holding, but with wider guard bands than in the other methods, can be used even only with legacy UEs |
| Network capacity | Entire spectrum can be used by multiplexing different UEs(even legacy UEs) | Entire spectrum can only be used for new UEs, whether legacy UEs can be multiplexed to cover entire channel depends on the configuration and bandwidth | Entire spectrum can be used by multiplexing different UEs in the frequency domain | Entire spectrum can be used by any UE |
| Legacy UE support | Legacy UEs supported, can use one of the aggregated CCs | Legacy UEs can use part of the spectrum that contains the SSB | Legacy UEs supported | Legacy UEs supported |
| RAN1/2/4 Specification impact | RAN1/2 – new UE capabilities needed, RAN4 – new band combinations, changes to channel spacing definition, Overlapping CA reqs applicability, new demod requirements for UEs | RAN2 – impact on new capability RAN4 –core requirements equivalent to new channel BW for BS, new demod requirements for UEs | RAN4 – BS requirements for new channel BW | RAN4 – BS requirements for new channel BW, possibly restrictions of the suitable scenarios. New asymmetric bandwidth combinations for UE are needed. However these combinations would be “regular” BW combinations, so existing process. |

Probably because none of these solutions are ideal, there has not been a follow-on Work Item to make any of these solutions available to operators, other than the 10 kHz raster work that helped with the even/odd PRB issue that was discovered during the Study Item. The next wider channel bandwidth with PRB blanking might be possible today for allocations not near the edge of the band, but the potential near-far issue is problematic, especially for coverage bands.

## 2.2	CA with small channel bandwidths

To move toward a solution for the operators, we would like to propose the creation of a few small channel bandwidths that could be combined with other existing channel bandwidths to make any 1 MHz channel bandwidth available to operators. For example, we already now have 3 MHz channel bandwidths defined, which could be combined via CA with other channel bandwidths to cover 8, 13, 18 and 23 MHz, for example.

### 2.2.1	Option 1: Add 1, 2 and 4 MHz

3GPP could potentially add 1, 2 and 4 MHz CHBWs in order to cover any CHBW in 1 MHz increments with intra-band contiguous CA. 4 MHz could use the punctured SSB that is used for 3 MHz CHBW. While 1 and 2 MHz channels are not wide enough to handle even the punctured SSB, so they would need CA without an SSB. This seems like it should be possible, but the approach would need consideration for any potential issues.

Figure 1 shows how Option 1 channel BWs could be combined to meet potential channel bandwidths. Yellow indicates channel bandwidths that were requested by operators during the study item.

Figure 1Example Option 1 CA bandwidth combinations

### 2.2.2	Option 2: Add 4, 6 and 7 MHz

A second alternative would be for 3GPP to add 4, 6 and 7 MHz CHBWs in order to cover any CHBW in 1 MHz increments with intra-band contiguous CA. 4 MHz could use the punctured SSB that is used for 3 MHz CHBW, while 6 and 7 MHz could use the regular SSB.

Figure 2 shows how Option 2 channel BWs could be combined to meet potential channel bandwidths. Yellow indicates channel bandwidths that were requested by operators during the study item.

Figure 2 Option 2 CA bandwidth combinations

### 2.2.3	Option comparison

To avoid potential issues with SSB-less 1 and 2 MHz carriers, we propose going with 4, 6 and 7 MHz channel bandwidths, rather than 1, 2 and 4 MHz. However, 4 MHz is not needed for any of the channel bandwidths that were requeste4d and documented during the Study Item, so it may not be needed.

## 2.3	Exemplary objectives for WID

Potential core objectives

- Determine if operators have a need for and channel bandwidths which would require 4 MHz channel bandwidth

- Specify necessary RAN4 requirements to support deploying NR channel bandwidths of 6 and 7 MHz, and potentially 4 MHz depending on operator requests [RAN4], including n85/n12 for example bands for 6 MHz and n5/26 as example bands for 7 MHz:

  - Specify RF requirements:

    - Specify the required RF requirements for optional 6 and 7 MHz, and potentially 4 MHz channel bandwidths, including PRB numbers and minimum guardband

    - Limit to 15 kHz for FDD bands only to minimize RAN4 workload

Potential Performance objectives:

Specify necessary UE/BS performance requirements for NR operation in dedicated FDD FR1 spectrum allocations from approximately 3MHz up to below 5MHz, corresponding to the core requirements:

- Specify necessary UE demodulation performance and CSI reporting requirements (RAN4)

- Specify necessary BS demodulation performance requirements (RAN4)

- Specify necessary BS conformance tests (RAN4)

# 3	Conclusions

In this paper we have presented our views on how irregular channel bandwidths can finally be supported for operators with licensed channel bandwidths that are not multiples of 5 MHz. These channel bandwidths could be used for both terrestrial and non-terrestrial networks.

Proposal:	Consider a new Rel-19 WI to enable support of 6 and 7 and potentially 4 MHz channel bandwidths.

# 4	References

RP-191873, “New WID: Introduction of NR Band n26,” Sprint, AT&T, Dish Network

RP-192273, “New WID: Introduction of NR Band n26,” Sprint, AT&T, Dish Network

RP-202103, "New SID: Study on Efficient utilization of licensed 	spectrum that is not aligned with existing NR channel bandwidths", T-Mobile USA, Ericsson

TR 38.844, “Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidth”