3GPP TSG-RAN WG4 Meeting #113                                                               R4-2419790

# Orlando, US, November 18th – 22th, 2024

Agenda item:			7.9.6

Source:	Moderator (CHTTL)

Title:	Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF

Document for:	approval

Backgrounds

The SAN and UE RF requirement of the Rel-19 NR NTN Ku band work item had been discussed in RAN4#112bis. This WF captures the result of the discussion in the meeting.

WF for NTN Ku band UE RF

Sub-topic 1-1: General issues for Ku band UE RF requirements

- Issue 1-1-2: Antenna Models to be considered

- The 20cm x 20cm, 40cm x 40cm and 60cm x 60cm size of the VSAT antenna models can be considered for the VSAT types with electronic steering antenna in the Ku band normative work

  - Detail parameters can be further checked and FFS on mapping to the 38.101-5 VSAT types and conducting the coexistence study

- Issue 1-1-3: Common antenna type vs. separate antenna type

- Proposals:

  - Proposal 1 (Huawei): For full duplex FDD mode under the band plan #1 (DL: 10.7~12.75GHz, UL: 14~14.5GHz), the common Tx/Rx antenna with duplexer can be assumed for mechanical steered antenna type (Parabolic antenna) and the separate Tx/Rx antenna will be chosen for phased array antenna type.

- Result and WF based on the feedback:

  - For full duplex FDD mode under the band plan #1 (DL: 10.7~12.75GHz, UL: 14~14.5GHz), the following can be considered

    - The common Tx/Rx antenna with duplexer can be assumed for mechanical steered antenna type (Parabolic antenna)

    - Usually separate Tx/Rx antenna are used for the phased array antenna type, however there is also new types with common Tx/Rx antenna with duplexer

      - Companies are encouraged to provide the details of the latter new types.

- Issue 1-1-4: Regulations to be considered

- Aligned with the result in thread [313], RAN4 will work on the following band plans to cover all regions with the concerned regulation at least

  - Downlink 10.70 – 12.75 GHz (Space-to-Earth) and Uplink 14 – 14.5 GHz (Earth-to-Space) concerning requirements from ECC regulation and ETSI standards

  - Downlink 10.70 – 12.7 GHz (Space-to-Earth) and Uplink 14 – 14.5 GHz (Earth-to-Space) concerning FCC requirements.

  - FFS for other agreed band plans

Sub-topic 1-2: General issues for Ku band UE RF requirements

- Issue 1-2-2: Maximum EIRP and TRP requirement

- The following aspects will be consider if setting the maximum EIRP & TRP of the NTN Ku band in 38.101-5

  - An absolute maximum upper limit on 88.2 dBm EIRP fulfilling GSO scenarios, while allowing separating consideration for non-GSO scenarios

  - Upper EIRP limits in the regulation

  - Enable flexibility for different application and scenarios

- Issue 1-2-3: Maximum EIRP requirements and the minimum antenna size for the CEPT Ku band

- For the CEPT Ku band, the following requirements from ECC regulation will be considered, FFS on how to reflect to the specifications and the VSAT types.

  - NTN VSAT operating with GSO	14.0 – 14.25 GHz	Max EIRP: 60 dBW (from ECC Decision(17)04)

  - NTN VSAT operating with GSO	14.25 – 14.5 GHz	Max EIRP: 50 dBW (from ECC Decision(03)04)

  - NTN VSAT operating with GSO	14.0 – 14.25 GHz	Max EIRP: 60 dBW (from ECC Decision(06)03)

  - Mobile VSAT / Land ESIM		      14.0 – 14.5 GHz		Max EIRP: 54.5 dBW (from ECC Decision(18)04)

  - Mobile VSAT / Maritime ESIM	      14.0 – 14.5 GHz	      minimum antenna size: 0.6 m (from ECC Decision (05)10)

- Issue 1-2-4: Off-axis EIRP requirements for the CEPT Ku band

- For the UL band 14.0 - 14.5 GHz (E-to-S) concerning requirements from ECC regulation and ETSI standards:

  - Specify the off-axis EIRP density limit based on EN 301 428, EN 302 340 and EN 302 977 as a starting point, the ECC decision can be further checked whether additional updated is needed.

- Issue 1-2-5: On-axis spurious and off-axis spurious requirement for the CEPT Ku band

- For the UL band 14.0 - 14.5 GHz (E-to-S) concerning requirements from ECC regulation and ETSI standards:

  - Specify the same on-axis spurious limits in “Carrier-on” state for both outside and inside as specified in EN 303 980, EN 303 981 as the starting point

  - Specify the same requirement as Ka band for the general and off-axis spurious limits in “Emissions disabled” state as starting point.

  - FFS on off-axis spurious limits in “Carrier-on” and “Carrier-off” state

- Issue 1-2-7: Requirements for the FCC Ku band

- For the Ku band with uplink 14 – 14.5 GHz (Earth-to-Space) concerning FCC requirements, consider the following as the starting point:

  - The current additional spectrum emission mask requirement for n511, n510 can be applied to the Ku band.

  - Consider the co-polarization & off-axis cross-polarization gain limit in 38.101-5 based on the Table 1-2-7.1~1-2-7.4.

Table 1-2-7.1: Co-polarization gain limit in the plane tangent to the GSO arc for the conventional Ku band

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 1.5° ≤ θ ≤ 7° | 29 – 25log10(θ) |
| 7° ≤ θ ≤ 9.2° | 8 |
| 9.2° ≤ θ ≤ 19.1° | 32 – 25log10(θ) |
| 19.1° < θ ≤ 180° | 0 |

Table 1-2-7.2: Co-polarization gain limit in the plane perpendicular to the GSO arc for the conventional Ku band

| θ value | Co-polarization gain (dBi) |
| --- | --- |
| 3° ≤ θ ≤ 19.1° | 32 – 25log10(θ) |
| 19.1° < θ ≤ 180° | 0 |

Table 1-2-7.3: Cross-polarization gain limit in the plane tangent to the GSO arc for the Ku band

| θ value | Cross-polarization gain (dBi) |
| --- | --- |
| 1.8° ≤ θ ≤ 7° | 19 – 25log10(θ) |

Table 1-2-7.4: Cross-polarization gain limit in the plane perpendicular to the GSO arc for the Ku band

| θ value | Cross-polarization gain (dBi) |
| --- | --- |
| 3° ≤ θ ≤ 7° | 19 – 25log10(θ) |

  - Consider the off-axis EIRP density limits requirement in 38.101-5 based on the Table 1-2-7.5~1-2-7.7.

Table 1-2-7.5: Off-axis EIRP density limits for co-polarized transmissions in the plane tangent to the GSO arc for the conventional Ku-band

| θ value | Maximum Off-axis EIRP (dBW) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 1.5° ≤ θ ≤ 7° | 15 – 25log10(θ) | 4 |
| 7° ≤ θ ≤ 9.2° | -6 | 4 |
| 9.2° ≤ θ ≤ 19.1° | 18 – 25log10(θ) | 4 |
| 19.1° < θ ≤ 180° | -14 | 4 |

Table 1-2-7.6: Off-axis EIRP density limits for co-polarized transmissions in the plane perpendicular to the GSO arc for the conventional Ku-band

| θ value | Maximum Off-axis EIRP (dBW) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 3° ≤ θ ≤ 19.1° | 18 – 25log10(θ) | 4 |
| 19.1° < θ ≤ 180° | -14 | 4 |

Table 1-2-7.7: Off-axis EIRP density limits for cross-polarized transmissions in the plane tangent to the GSO arc and in the plane perpendicular to the GSO arc for the conventional Ku-band

| θ value | Maximum Off-axis EIRP (dBW) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 1.5° ≤ θ ≤ 7° | 5 – 25log10(θ) | 4 |